3GPP TSG-SA WG2 Meeting #145E 	S2-2104384r5

Elbonia, 17 - 28 May, 2021

Source:	Huawei, HiSilicon

Title:	Clarification on EAS discovery via EASDF

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC /Rel-17

Abstract of the contribution: This contribution clarify several issue related to EAS discovery via EASDF.

- Discussion

Signalling optimization

In S2#144E meeting, it has been discussed how to optimize the signalling between SMF and EASDF, especially for the case when the DN deployment has been changed, e.g. the Local DNS Server address has been changed. According to current procedure, if the DN deployment (e.g. EAS deployment information) has been changed, the SMF need to update the DNS message handling rule for all the UEs that may access the local DN. This causes a lot of signalling.

This technical issue is solved by different paper.

ECS option

The ECS option per DNAI can be pre-configured in the SMF. It is proposed the SMF gets the ECS option per DNAI based on configuration. The Editor's note about how the SMF gets the ECS option can be deleted.

Remove step 10

One EN on whether ULCL/BP should be inserted when the UE reports EAS FQDN in DNS query to SMF.

It is suggested that the ULCL/BP is inserted only based on the DNS response. Before the DNS response, the EAS is not selected yet. Although the SMF has selected DNAI for the DNS query, the SMF cannot guarantee that the returned EAS is located in the local DN selected by the SMF.

Regarding the signalling caused by extra reporting of DNS response, it will only add one extra reporting per PDU Session, because after the ULCL/BP has been inserted, the reporting of DNS response per the local DN can be cancelled by updating the DNS message handling rule. Furthermore, indeed the DNS query report causes a lot of signalling. It is better that the DNS message handling rule for handling DNS query message is provisioned to the EASDF at PDU session establishment and after UE mobility as early as possible, i.e. before any DNS query. So that the per DNS query retrieval of the DNS message handling rule (e.g. which option to be taken) can be eliminated if possible.

Alternatively, the EASDF can report the DNS message once if the reporting condition is met.

This will really reduce the signalling. But we are open to still keep the option of retrieving DNS message handling rule based on FQDN of DNS query.

The procedure agreed in clause 6.2.3.2.2. of TS 23.548 is used for improving the server discovery for edge computing to resolve the FQDN into the optimal EAS with improved low latency. According to the procedure cited below, if the ULCL/BP insertion is triggered by the EAS information received in the DNS response message, the DNS response is sent to the UE based on the related rule updated after the ULCL/BP insertion, before sending the DNS response to the UE the EASDF needs to buffer and wait until extra 5 steps including step 15, 16, 17, 18 and 19 are executed. However, if the traffic routing information is available, the ULCL/BP insertion can be triggered by the related condition e.g. UE mobility into the related DNAI. So the ULCL/BP insertion can be triggered before receiving the DNS response. Therefore, without triggering ULCL/BP insertion by the EAS information received in the DNS response message, the DNS response can be sent to the UE once it is received by the EASDF with lower latency without waiting for the execution of the DNS response buffering, the ULCL/BP insertion determination and the rule update for DNS response forwarding, furthermore, the interaction frequency between SMF and EASDF can be reduced too.

Hence, it is suggested that the ULCL/BP is inserted based on DNS response, and also taking the ULCL/BP insertion performed based on the UE location and traffic routing information into account while setting the rules to handle DNS message(s). The traffic routing information can be triggered to be retrieved by the DNS query message for the requested application if it is not available yet.

Multiple local DN available

One FFS is added on whether multiple ECS options are provided to EASDF. It assumes that multiple local DN may be available to the UE, and the same EAS may have deployment in these multiple local DN. This is a valid scenario.

There are 2 options to solve the local DN selection:

- Option 1: provide info of all the available local DN to the EASDF. The EASDF tries to discover an EAS from all the available local DN.

- Option 2: The SMF selects a local DN and provides only the selected local DN to the EASDF. The EASDF discovers the EAS from the selected local DN.

In option 1, it is not very clear how the EASDF would select the local DN. Currently, the DNS query message can only carry one ECS option. Is a precedence needed for the EASDF to try to discovery EAS from the local DN based on the priority? So does it means that the EASDF do the DNS query per each ECS option? This adds complexity and not sure how the EASDF select one.

The EASDF is simpler in Option 2. It has a risk that the EASDF cannot discover EAS from the local DN selected by the SMF, for example, due to failure or overload. However normally more than one EAS address is returned from the DNS server. So it should be no problem.

So from our view one ECS options is the enough. However from last meeting discussion it seems some more discussion is needed. Hence we do not directly remove that EN for the time being.

Multiple EAS address in the DNS response

It is possible that DNS server return multiple possible EAS(es) in the DNS response. For UL-CL/BP insertion, the SMF can only use the EAS IP address associated with one DNAI. Thus it is suggested that the EAS IP address returned to the UE needed be managed as that order by the SMF.

IP address range(s) per DNAI for traffic routing (UL-CL/BP setting)

The IP address range(s) per DNAI provided by AF can be applied for traffic routing at UL CL to forward EAS traffic to local PSA. The PCF sends the IP address range(s) per DNAI to the SMF as part of the PCC rules, then the SMF determines the traffic detection rules and traffic routing rules applied to UL CL. Or the SMF can determine the traffic detection rules and traffic routing rules based on IP address range(s) per DNAI from the preconfigured EAS deployment information.

DNS message type in the DNS message reporting

According to the definition of DNS message handling rule, the EASDF reports DNS message content (e.g., FQDN from DNS Query or DNS Response) to the SMF when the DNS message matches a DNS message handling rule for reporting.

However, if only FQDN is included in the DNS reporting, the SMF cannot differentiate the corresponding DNS message type. Then the next action of SMF, e.g., providing information to build the ECS option information or performing the UL CL/BP and Local PSA selection, cannot be decided. Therefore the DNS message type should be included in the DNS message reporting, in order to assist SMF to decide the proper action.

- Proposal

It is proposed to do the change as suggested above principle.

**************************** 1st Change ***************************

6.2.3.2.2	EAS Discovery Procedure with EASDF

For the case that the UE DNS Query is to be handled by EASDF, the following applies.

-	The AF may provide EAS deployment information to UDR, including the list of FQDNs supported by applications for each DNAI, the IP address range(s) corresponding to each DNAI, and the DNS server identifier (consisting of IP address and port) for each DNAI, as defined in clause 5.6.7 of TS 23.501[2]. The AF may update the information as described in clause 4.3.6.2 of TS 23.502[3].

-	During the PDU Session establishment procedure, the SMF gets the EAS deployment information via the PDU Session related policy information from PCF or the SMF is preconfigure with the EAS deployment information the, and the SMF selects an EASDF and provides its address to the UE as the DNS Server to be used for the PDU Session. The UE sends DNS Query to the EASDF. The SMF may configure the EASDF with DNS message handling rules to forward DNS messages of the UE to a relevant DNS server and/or report when detecting DNS messages.  The DNS message handling rule includes information used for DNS message detection and associated action(s). It is defined as following:

-	Precedence of the DNS message handling rule

-	DNS message detection template (includes at least one of the following):

-	DNS message type = DNS Query or DNS Response

-	If DNS message type = DNS Query

-	Source IP address (i.e. UE IP address)

-	Array of (FQDN ranges)

-	If DNS message type = DNS Response

-	Array of FQDN ranges and/or array of EAS IP address ranges

NOTE 1:	For DNS message type = Query, the UE IP address provided at DNS context creation is considered if not provided explicitly as part of the template.

-	Action(s) (includes at least one action) the possible actions include:

-	Report DNS message content (e.g., FQDN or EAS IP address and corresponding DNS message type) to SMF. The report can be one-time report or every-time report when the reporting condition is met.

-	Send the DNS message to a preconfigured DNS server/resolver or a local DNS server associated with the indicated DNAI as following (The DNAI is included in the DNS handling rule):

-	Build an optional ECS option in the DNS message based on the EAS deployment information associated with the DNAI

Editor’s Notes:	It is FFS whether the information for the EASDF to build the ECS option may contain more than one IP address to deal with following cases: 1) some EAS may be deployed on the part of the DN with public IP addressing space while other EAS may be deployed on the part of the DN with private IP addressing space, 2) there may be multiple candidate L-PSA (+ULCL) UPF: for example some expensive L-PSA at the DU site related with fairly specific applications that are demanding in terms of delays and some more vanilla L-PSA + ULCL UPF that can provide traffic offload but that are more central and cheaper due to a better pooling effect in a more central location, 3) there may be multiple L-PSA UPFs: for example some applications are supposed to be accessed via one DNAI with one L-PSA UPF and the other applications are supposed to be accessed via another DNAI with another L-PSA UPF.

-	Replacement of the DNS message target address with the indicated DNS Server Address; if no DNS Server Address is provided  by the SMF, then the EASDF is to forward the DNS message to a locally preconfigured DNS server/resolver.

-	Buffer the DNS message and report DNS message content to the SMF

-	Send the buffered DNS response message to UE.

When the EASDF forwards a DNS request, it shall always ensure it receives the DNS answer (putting its own address as the source of the request)

The SMF may use following information to create DNS message handling rules associated with a PDU session:

-	Local configuration associated with the (DNN, S-NSSAI) of the PDU Session, and/or

-	EAS deployment information provided by the AF or preconfigured in the SMF, and/or

-	Information derived from the UE location such as candidate L-PSA (s).

-	PDU Session information, like PDU Session L-PSA(s) and ULCL/BP

NOTE 2:	For example, the SMF can derive the IP address for ECS based on the N6 IP address(es) associated with serving L-PSA(s) locally configured or in the NRF.

NOTE 3:	Providing in DNS ECS option an IP address associated with the L-PSA UPF protects the privacy of the (IP address of the) UE.

-	If the FQDN in a DNS Query matches the FQDN(s) provided by the SMF, based on instructions by SMF, one of the following options is executed by the EASDF:

-	Option A: The EASDF includes the EDNS Client Subnet (ECS) option into the DNS Query message as defined in RFC 7871[6], and sends the DNS Query message to the DNS server for resolving the FQDN. The DNS server may resolve the EAS IP address considering the ECS option, and sends the DNS Response to the EASDF.

-	Option B: The EASDF sends the DNS Query message to a Local DNS server which is responsible for resolving the FQDN within the corresponding Local DN. The EASDF receives the DNS Response message from the Local DNS server.

NOTE 4:	Option B does not support the scenario where the EASDF has no direct connectivity with the local DNS servers.

The SMF instructions for a matching FQDN may as well indicate EASDF to contact SMF. SMF then provides the EASDF with a DNS message handling rule.

-	If the DNS Query from the UE does not match a DNS message handling rules set by the SMF, then the EASDF may simply forward the DNS Query towards a preconfigured DNS server/resolver for DNS resolution.

-	When the EASDF receives a DNS Response message, the EASDF may notify the EAS information (i.e. EAS IP address(es), optionally the EAS FQDN and optionally the corresponding IP address within the ECS DNS option) to the SMF if the DNS message reporting condition (i.e. the EAS IP address or FQDN is within the IP/FQDN range) provided by the SMF is met. The SMF may then select the target DNAI based on the EAS information and trigger UL CL/BP and L-PSA insertion as specified in clause 6.3.3 in TS 23.501 [2] based on the Notification.

The information to build the ECS option or the Local DNS server address provided by the SMF to the EASDF are part of the DNS message handling rules to handle DNS queries from the UE. This information is related to DNAI(s) for that FQDN(s) for the UE location. The SMF may provide DNS message handling rules to handle DNS queries from the UE to the EASDF when the SMF establishes the association with the EASDF for the UE, and may update the rules at any time when the association exists. For the selection of the candidate DNAI for an FQDN for the UE, the SMF may consider the UE location, network topology and information of EAS deployment information received as part of PDU Session related policy information for the PDU Session while it is provided/modified/deleted as defined in TS 23.503 [4] clause 6.4 or be preconfigured into the SMF. The EAS deployment information is provisioned by the AF via the procedure of AF influence on traffic routing as defined in in clause 5.6.7.1 of TS 23.501 [2] and in clause 4.3.6.2 of TS 23.502 [3].. After the UE mobility, if the provided Information for ECS option or the Local DNS server address needs be updated, the SMF may send an update to DNS message forwarding rules to the EASDF.

Once the UL CL/BP and L-PSA have been inserted, the SMF may decide that the DNS messages for the FQDN are to be handled by local DNS resolver/server from now on. This option is further described in clause 6.2.3.2.3.

the SMF may instruct the EASDF not to report DNS messages to SMF corresponding to some FQDN ranges and/or EAS IP address ranges e.g. once the UL CL/BP and L-PSA have been inserted,. After the removal of the L-PSA corresponding to the DNAI, the SMF may instruct the EASDF to restart the repors oft the DNS messages.

.

Figure 6.2.3.2.2-1: EAS discovery procedure with EASDF

1.	UE sends PDU Session Establishment Request to the SMF as shown in step 1 of clause 4.3.2.2.1 of TS 23.502 [3].

2.	During the PDU Session Establishment procedure, the SMF selects EASDF as described clause 6.3 of TS 23.501 [2]. The SMF includes the IP address of the EASDF as DNS server in PDU Session Establishment Accept message as in step 11 of clause 4.3.2.2.1 of TS 23.502 [3]. The UE configures the EASDF as DNS server for that PDU Session.

3.	The SMF invokes Neasdf_DNSContext_Create Request (UE IP address, DNN, callback URI, DNS message handling rules) to the selected EASDF.

This step is performed before step 11 of PDU Session Establishment procedure in clause 4.3.2.2.1 of TS 23.502 [3].

The EASDF creates a DNS context for the PDU Session, and stores the UE IP address, the callback URI and DNS message handling rule(s) into the context.

The EASDF is provisioned with the DNS message handling rule(s), before the DNS Query message is received at the EASDF or as a consequence of the DNS Query reporting.

4.	The EASDF invokes the service operation Neasdf_DNSContext_Create Response.

Editor's note:	How to guarantee that the UE uses the EASDF's IP address for the subsequent DSN Query in step 8 is FFS.

5.	The SMF may invoke Neasdf_DNSContext_Update Request (EASDF Context ID, DNS message handling rules) to EASDF. The update may be triggered by UE mobility, e.g. when UE moves to a new location, or by a reporting by EASDF of a DNS Query with certain FQDN, or, the update may be triggered by insertion/removal of Local PSA, e.g. to update rules to handle DNS messages from the UE or by new PCC rule information.

6.	The EASDF responds with Neasdf_DNSContext_Update Response.

7.	The UE sends DNS Query message to the EASDF.

8.	If the DNS Query message matches a DNS message handling rule for reporting, the EASDF sends the DNS message report to SMF by invoking Neasdf_DNSContext_Notify Request.

9.	The SMF responds with Neasdf_DNSContext_Notify Response.

10. [Optional] The SMF may perform UL CL/BP and Local PSA selection and insert UL CL/BP and Local PSA.

Based on received FQDN information received from the EASDF and other UPF selection criteria, as specified in clause 6.3.3 in TS 23.501 [2], the SMF may perform UL CL/BP and Local PSA selection and insertion as described in TS 23.502 [3]. If the PCC rule for the application associated with the received FQDN is not available, the SMF can retrieve the PCC rule for the application associated with the received FQDN and the traffic routing information may be included.

Editor's note:	It is FFS whether UL CL / BP insertion should be mentioned at this step.

11.	If DNS message handling rule for the FQDN received in the report need to be updated, e.g. provide updates to information to build the ECS option information, the SMF invokes Neasdf_DNSContext_Update Request (DNS message handling rules) to EASDF.

For Option A, the DNS handling rule includes corresponding IP address to be used to build the ECS option. For Option B, the DNS handling rule includes corresponding local DNS Server IP address. The EASDF may as well be instructed by the DNS handling rule to simply forward the DNS Query to a pre-configured DNS server/resolver.

12.	The EASDF responds with Neasdf_DNSContext_Update Response.

13.	The EASDF handles the DNS Query message received from the UE as the following:

-	For Option A, the EASDF adds the ECS option into the DNS Query message as specified in RFC 7871[6] and sends it to C-DNS server;

-	For Option B, the EASDF sends the DNS Query message to the Local DNS server.

If no DNS message detection template within the DNS message handling rule provided by the SMF matches the requested FQDN in the DNS Query, the EASDF may simply send a DNS Query to a pre-configured DNS server/resolver.

14.	EASDF receives DNS Responses from the DNS system and determines that a DNS Response can be sent to the UE.

15.	The EASDF may send an DNS message reporting to the SMF by invoking Neasdf_DNSContext_Notify request including EAS information if the EAS IP address or the FQDN in the DNS Response message matches the reporting condition provided by the SMF. The DNS message reporting may contain multiple EAS IP address if the EASDF has received multiple EAS IP address(es) from the DNS servers it has contacted. The DNS message reporting may contain the FQDN and the ECS option received in the DSN Response message.

Per the received DNS message handling rule, the EASDF does not send the DNS Response message to the UE but waits for SMF instructions (in step 18) , i.e. buffering the DNS Response message.

16.	The SMF invokes Neasdf_DNSContext_Notify Response service operation.

17.	The SMF may perform UL CL/BP and Local PSA selection and insert UL CL/BP and Local PSA.

Based on received EAS information received from the EASDF and other UPF selection criteria, as specified in clause 6.3.3 in TS 23.501 [2], the SMF may determine the DNAI and determine the associated N6 traffic routing information for the DNAI. The SMF may perform UL CL/BP and Local PSA selection and insertion as described in TS 23.502 [3]. In case of UL CL, the traffic detection rules and traffic routing rules are determined by the SMF based on IP address range(s) per DNAI included the PCC rules as defined in clause 5.6.7 in TS 23.501 [2]. Or the SMF determines the traffic detection rules and traffic routing rules based on the IP address range(s) per DNAI included in the preconfigured EAS deployment information..

18.	The SMF invokes Neasdf_DNSContext_Update Request (DNS message handling rules).

The DNS message handling rule indicates the EASDF to send a DNS Response buffered in Step 15 to UE. The DNS message handling rule may indicate the EASDF not to send further DNS Response message corresponding to FQDN ranges and/or EAS IP address ranges for the selected DNAI.

19.	The EASDF responds with Neasdf_DNSContext_Update Response.

20.	The EASDF sends the DNS Response to UE.

**************************** Second Change ***************************

7.1.2.5	Neasdf_DNSContext_Notify service operation

Service operation name: Neasdf_DNSContext_Notify.

Description: EASDF reports DNS signalling related information to the consumer when receiving DNS Query or DNS Response.

Input, Required: DNS message reporting information (EAS information specified in clause 6.2.3.2.2 and corresponding DNS message type).

Input, Optional: None.

Output, Required: Success or Failure.

Output, Optional: None.

********************************* End Change ****************************