3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104386

Elbonia, May 17– 28, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon,Tecent

Title:	Upates to EAS Re-discovery Procedure at Edge Relocation

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract: Updates to EAS Re-discovery Procedure at Edge Relocation over Distributed Anchor Connectivity Model on multi home case.

1. Introduction/Discussion

There is an EN in clause 6.2.2.3 TS23.548 as follows:

Editor' note:	The scenario of Change of SSC mode 3 PDU Session Anchor with IPv6 Multi-homed PDU Session as in clause 4.3.5.3 is FFS.

For the EAS re-discovery for SSC mode 3 PDU Session with IPv6 Multi-homed PDU Sessions, this indeed is the session breakout connectivity. In this case if the EAS is to be reallocated and rediscovery need be triggered, the related solution has been described in clause 6.2.3.3. So it is clarified how to handle that in that case.

2. Text Proposal

It is proposed to capture the following changes vs. TS 23.548.

# * * * * First change * * * *

6.2.2.3	EAS Re-discovery Procedure at Edge Relocation

6.2.2.3	EAS Re-discovery Procedure at Edge Relocation

Editor's note:	This clause also describes rediscovery (UE based) and aspects and assumptions based on applicable clause 9.2.2 in the TR.

In order to change the PDU Session Anchor serving a PDU Session of SSC mode 2/3 for a UE, SMF triggers session continuity, service continuity and UP path management procedures as indicated in clause 4.3.5.1 and 4.3.5.2 of TS 23.502 [3]. During this procedure, for SSC mode 2/3, it is recommended that the UE applies the following behaviour:

The UE DNS cache should be bound to the IP connection. When the UE detects the PDU Session release or IP address changes, the UE removes the old DNS cache related to removed IP address, for example, the old Edge Application Server address information.

NOTE:	UE DNS cache refers to cache at any level (OS and Application). Whether the DNS cache of App is included or influenced depends on application's behaviour and UE implementation.

With this behaviour, when the establishment of a new PDU Session triggers EAS rediscovery for an application, UE can reselect a new EAS for that application.

For SSC mode2, the procedure in clause 4.3.5.1 applies with following differences:

-	In Step 3, when the new PDU Session has been established, UE can reselect a new EAS for the application with an EAS Discovery procedure if the recommended UE behaviour has been followed.

For SSC mode3 with multiple PDU sessions, the procedure in clause 4.3.5.2 applies with following difference:

-	In step 5, the UE can reselect a new EAS for the application with an EAS Discovery procedure if the recommended UE behaviour has been followed.

For SSC mode#3 with IPv6 Multi-homed PDU Session that all new traffic going via new IPv6 prefix, the procedure in clause 4.3.5.3 applies with following difference:

-	After step 10-11 where SMF notifies the UE of the availability of the new IP prefix, the UE starts using it for all new traffic, including DNS Queries. The UE can reselect a new EAS for the application with an EAS Discovery procedure if the recommended UE behaviour has been followed.

Then UE can reselect a new EAS for the application with an EAS discovery procedure as defined in clause 6.2.2.2.

NOTE: 	For SSC# 3 with multi-homed PDU sessions, an EAS re-discovery indication may as well be sent as described in clause 6.2.3.3.

# * * * * End of changes * * * *