3GPP TSG-SA WG2 Meeting #145E 	S2-2104397r03

Elbonia, 17 - 28 May, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Support of Local MBS

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document proposes to update clause 7.2.4 of TS 23.247, i.e., Support of Local multicast service.

# Introduction

Clause 7.2.4 of TS 23.247 is for local MBS. This document adds some missed part in this clause. The main updates includes missed handover procedure, and some small corrections.

# Discussion

Hanover Procedure

Currently, the local MBS related mobility procedure in 7.2.4.2.3 focus on handover procedure for location-dependent content MBS session, i.e., whether the target NG-RAN should accept the handover and establish the forwarding resources based on the area session ID and area information. However, clause 7.2.4 does not mention how to release the local MBS related transmission resource if the UE moves out the MB service area i.e., handover procedure for limited area MBS session.

To achieve this, the SMF need first to acquire the UE location information in target NG-RAN and then determine how to do. SMF can use the existing method to acquire the UE location information in target NG-RAN:

- For Xn handover, the UE location information in the target NG-RAN is always included in the N2 PATCH SWITCH REQUEST sent from target NG-RAN to AMF and AMF sends the UE location information to SMF via Nsmf_PDUSession_UpdateSMContext Request.

- For N2 handover, the UE location information in the target NG-RAN is always included in the HANDOVER REQUIRED sent from source NG-RAN to AMF and AMF sends the UE location information to SMF via Nsmf_PDUSession_UpdateSMContext Request.

Based on the UE location information in target NG-RAN:

- If the UE moves out the MB service area, SMF release the impacted MBS Session context, e.g., release the mapped unicast resource in PSA.

- If the UE still in the MB service area, the SMF handle it as usual.

Proposal 1: It is proposed to add a new sub-clause regarding handover procedure for limited area MBS session.

# Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change* * * *

##### 7.2.4.3.3	Handover procedure with limited area MBS session

The Handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

- Before the Handover, The UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID.

- For Xn based handover in clause 7.2.3.2, Source RAN includes MBS Session ID and MBS area session ID to the Target RAN during Handover preparation phase. For N2 based handover in clause 7.2.3.3, this step corresponds to Handover Request and Handover Required message, respectively.

- For Xn based handover in clause 7.2.3.2, target RAN responses to Source RAN, with the accepted MBS Session ID. When Target RAN supports 5MBS but the UE is no longer in the MBS service area, Target RAN  does not allocate RAN resources for the MBS Session to the UE. For N2 based handover in clause 7.2.3.3, this step corresponds to Handover Request acknowledge message.

- For Xn handover, if the UE is handed over to a target cell outside the MBS service area, the SMF does not provide the MBS session related information in N2 SM Info to the target RAN.

- For N2 handover, if the UE is handed over to a target cell outside to the MBS service area, the SMF does not provide the MBS session related information in N2 SM Info to the target RAN.

Editor’s note: Align with handover procedure is needed.

Editor’s note: Handover to the target that does not support 5MBS is FFS.

# * * * * End of changes * * * *