3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104398

Elbonia, May 17– 28, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Xn/N2 based handover from non-MBS supporting NG-RAN

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document adds the mobility procedure related to handover from non-MBS supporting source NG-RAN.

- Introduction

In SA2 #144E meeting, the basic call flow of mobility procedure for MBS was agreed. In TS 23.747, clause 7.2.3.2 and clause 7.2.3.3 depict the handover procedures from MBS supporting source NG-RAN. For the handover procedure from non-MBS supporting source NG-RAN, it is still missed and need to be filled up.

This document adds the related mobility procedure to support handover from non-MBS supporting source NG-RAN.

- Discussion

The inter-NG-RAN handover from non-MBS supporting source NG-RAN includes the following scenarios:

- Scenario 1: Both the source NG-RAN and the target NG-RAN does not support 5MBS.

- Scenario 2: The source NG-RAN does not support 5MBS and the target NG-RAN supports MBS.

For the first scenario, since the 5GC Individual MBS traffic delivery method is used at the source NG-RAN node, the existing Xn/N2 based handover procedure defined in TS23.502 [6] can be utilized.

For the second scenario, the main issue is how SMF is aware the target NG-RAN MBS capability and do the delivery method switching from Individual delivery to shared delivery. For the issue on how SMF is aware the target NG-RAN MBS capability, it can be referred from S2-2104394.

- Proposal

It is proposed to capture the following changes vs. TS23.247.

# * * * * First change* * * *

#### 7.2.3.4 Xn/N2 based handover from non-MBS supporting NG-RAN node

When the UE has joined the MBS session and the source NG-RAN node does not support 5MBS, the 5GC Individual MBS traffic delivery method is used for the MBS session data delivery. When the Xn/N2 based handover procedure is triggered, the UE is handed over to the target NG-RAN node per existing Xn /N2 based handover procedure defined in TS23.502 [6].

The following applies for an Xn handover from an NG-RAN node not supporting MBS to an NG-RAN node supporting MBS:

- The source NG-RAN node requests the mapped QoS Flow(s) in the associated unicast PDU session to be handed over to the target NG-RAN node. The target NG-RAN indicates its support of MBS to SMF.

Editor’s Note: It is FFS that based on the network configuration if the target NG-RAN is able to deduce the MBS Session mapped to the QoS Flow(s) in the associated unicast PDU session, it applies 5GC share MBS traffic delivery for the UE.

- After successful handover the SMF triggers modification of the PDU Session resources at NG-RAN including transfer of the MBS related information in N2 SM Info, i.e. the SMF changes the MBS session data delivery method from 5GC Individual MBS traffic delivery method to 5GC shared MBS traffic delivery method, and stops the multicast traffic forwarding at the UPF.

The following applies for an N2 handover from an NG-RAN node not supporting MBS to an NG-RAN node supporting MBS:

- The target NG-RAN node indicates its support of MBS, while it prepares to command the UE receiving MBS traffic via shared MBS Session Resources based on MBS related information in N2 SM Info. The NG-RAN node adds the UE into the MBS session context and responds to the SMF via AMF.

- After successful handover, the SMF stops the multicast traffic forwarding at the UPF i.e. individual delivery.

# * * * * End of changes * * * *