3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104404r02

May 17 – 28, 2021	revision of S2-2102659

Source:	China Mobile

Title:	Network exposure through SBA interface between UPF and local NEF

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC/Rel-17

Abstract of the contribution: This contribution proposes how UPF reports information to Local NEF.

# Discussion

In this paper, we propose to introduce UPF event exposure service to support the local network information exposure to local NEF. The SBI interface between local PSA UPF and local NEF can support the QoS monitoring information exposure, which can make the association between local UPF and local NEF more flexibly.

# Proposal

It is proposed to add the following change to TS 23.548.

************************ Start of Change ************************

### 6.4.2	Network Exposure to Edge Application Server via Local NEF

Local NEF deployed at the edge may be used to support network exposure timely to local AF. The local NEF may support one or more of the functionalities described in TS 23.501 [2] clause 6.2.5.0. and may support a subset of the APIs specified for capability exposure based on local policy. In order to support the network exposure locally, the local NEF shall support event exposure service operation for the local AF. The local NEF selection by AF is described in TS 23.501 [2] clause 6.2.5.0 and clause 6.3.14.

The local AF subscribes to low latency exposure of QoS Monitoring results from the PCF via a local NEF or NEF. If the NEF detects that it is not the most suitable NEF instance to serve the local AF request, it may redirect the AF to alocal NEF.

NOTE:	If the notifications need to go via the local NEF, then the local NEF needs to be involved to be able to map these notifications to the URI where the AF expects to receive them.

The local AF may also subscribe to event exposure service via PCF directly. In this case, reporting is done directly from the UPF to the local AF.

Based on the indication of local event notification and operator's policy, the PCF may include an indication of local event notification (including target local NEF or target local AF address) within the PCC rule that it provides to the SMF.

The SMF sends the QoS monitoring request to the RAN and N4 rules to the L-PSA UPF. N4 rules may indicate that a service data flow needs local notification of QoS Monitoring. When QoS monitoring of GTP-U Path(s) is used, itis also activated if needed. This is as defined in TS 23.501 [2] clause 5.33.3. The L-PSA UPF may be enhanced to support event exposure service, as defined in TS 23.502 [3] clause 5.2.x.When N4 rules indicate that a service data flow needs local notification of QoS Monitoring, upon the detection of the QoS monitoring event (as defined in 23.501 clause 5.33.3), the L-PSA UPF sends the Nupf_EventExposure_Notify to the Notification Target Address via received from the SMF in the Session Reporting Rule.

The Notification Target Address may correspond to the AF or to a local NEF. When the Notification Target Address corresponds to a Local NEF, the local NEF reports the QoS Monitoring information to the AF.

During UE mobility, the SMF may trigger the L-PSA UPF relocation/reselection and then send the N4 rules to the new L-PSA UPF to indicate the service data flow needs local notification of QoS Monitoring. The UE mobility may also trigger AF relocation or local NEF reselection, then the local AF should update the subscription for local exposure with QoS monitoring results via local NEF, towards the PCF. This updated /new subscription is then propagated via SMF (via PCC rule updates) and then to the L-PSA UPF via N4 rules.

Figure 6.46.2-1: The association establishment between local UPF and local NEF

0.	The UE establishes a PDU Session as defined in clause 4.3.2.2.1 of TS 23.502 [3] A Local PSA is used by this PDU Session.

1.	The AF initiates an AF session with required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3].

In the request, the AF may subscribe local notification of QoS monitoring for the service data flow to PCF via Local NEF or NEF. For the QoS monitoring, the AF shall include the corresponding QoS monitoring parameters as defined in clause 5.33.3 of TS 23.501 [2].

The AF may also first initiate an AF Session with PCF and later subscribe to local notification of QoS monitoring to PCF by invoking Npcf_Authorization_Subscribe service operation.

The local AF/ NEF may discover a local NEF based on existing method as specified in TS 23.501 [2] clause 6.2.5.0 and using parameters as those specified in clause 6.3.14. Alternatively, if the NEF detects that it is not the most suitable NEF instance to serve the local AF request, the NEF may redirect the AF to a (more) local NEF. The NEF may use information on the PDU Session Anchor of the PDU Session for this determination.

The indication for AF request network real-time information is also provided. Then the Local NEF may subscribe the local notification of QoS monitoring to PCF.

2.	The PCF makes the policy decision and initiates the PDU Session modification procedure as defined in clause 4.3.3.2 of TS 23.502 [3], steps 1b, 3b, 4-8b.

If the local notification of QoS monitoring is subscribed, the PCF includes the indication of local event notification (including target local NEF address) for the service data flow within the PCC rule.

If the SMF receives the indication of local event notification form the PCF and the SMF determines that the L-PSA UPF supports such reporting, the SMF sends QoS monitoring parameters and associate them with the target local NEF address to the L-PSA UPF via N4 rules. Otherwise the SMF activates N4 reporting. The PCF may determine that the duplicated notification is required, i.e. both local notification to the AF (i.e. sent from UPF) and central notification (i.e. sent from the PCF/SMF) is required and indicate it to the SMF with the PCC information. In this case, the SMF may activate the N4 reporting together with the direct reporting to the local NEF.

Editor's note:	The text above needs to be clarified: does it correspond to 2 AF sending QoS monitoring for the same SDF? Is it a realistic use case?

3.	The L-PSA UPF obtains QoS monitoring information as defined in TS 23.501 [2] clause 5.33.3.

4.	The L-UPF sends the notification related with QoS monitoring information over Nupf_EventExposure_Notify service operation.

5.	Local NEF reports the real-time network information to local AF by invoking Nnef_EventExposure_Notify service operation.

6.	Due to e.g. UE mobility, the PSA relocation and/or EAS relocation may happen as described in clause 6.3. During the PSA and/or EAS relocation, the AF or the NEF may trigger a new L-NEF discovery as in step 1.

7.	The new AF may initiate a new AF session to the new Local NEF or NEF/PCF to (re-)subscribe the local notification of QoS monitoring as described in steps 2-4.

8.	The old AF revokes the AF session with the old local NEF or NEF/PCF.

********************************* End Change ****************************