SA WG2 Meeting #145E	S2-2104421r01

17 – 28 May, 2021, Electronic meeting

Source:	Samsung, Ericsson

Title:	Update on MBS Session update for Broadcast

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document updates the MBS Session update for Broadcast procedure to use common procedure.

# Introduction

This document modifies the MBS Session update for Broadcast procedure to use common procedure.

# Proposal

It is proposed to capture the following changes in TS23.247.

* * * * Start of 1st Change * * * *

### 7.3.3	MBS Session Update for Broadcast

The MBS Session Update for broadcast is used by the AF to update the broadcast area or service requirements of the MBS Session.

Editor´s note:	MBSTF could optionally be in the user plane and the call flow should be updated to also cover that possibility.

Editor's note:	It is FFS whether there is any other parameters of service requirements need to be updated except for ARP.

Editor's note:	The services and messages used in this procedure are FFS.

Figure 7.3.3-1: MBS Session Update for Broadcast

1.	The AF starts MBS session update procedure by sending Modify MBS Session Request to the NEF/MBSF with TMGI(step 1~7 in the figure 7.1.1-3). The AF may adjust service requirement and/or broadcast area.

Editor´s note:	There are dependence on S2-2104190/S2-2104309 which introduce the MBS session update procedure in clause 7.1.1

2.	The MB-SMF sends MBS Session Resource Update to the AMFs with TMGI, the updated 5G QoS Profile and the updated MBS service area.

Depending on the change of the MBS service area, the MB-SMF may send MBS Session Start to some AMFs in the new MBS service area and MBS Session Stop to some other AMFs in the old MBS service area.

Editor´s note:	It is FFS if the MB-SMF needs stores the AMFs or can select AMFs based on service area and forwards the service area to AMFs to avoid that AMF needs to maintain information about broadcast session.

3.	The AMF sends MBS Session Resource Update to NG-RANs with TMGI, the updated 5G QoS Profile and the updated MBS service area.

Depending on the change of the MBS service area, the AMF may send MBS Session Resource Setup to some NG-RANs in new MBS service area and MBS Session Resource Release to some other NG-RANs in old MBS service area.

Editor´s note:	It is FFS if the RAN nodes are stored in the AMFs or selected based on service area.

4.	The NG-RAN updates MBS Session Context.

5.	The NG-RAN sends MBS Session Resource Update Response to the AMF.

6.	The AMF sends MBS Session Resource Update Response to the MB-SMF.

7.	The NG-RAN updates the MBS Session. It takes place in parallel with step 5 to step 6.

* * * * End of Changes * * * *