3GPP TSG-SA WG2 Meeting #145E	S2-2104438r01

Electronic Meeting, May 17 – 28, 2021

Title:	[Draft] Response LS on Scheduling Location in Advance to reduce Latency

Response to:	S2-2103735 (R2-2104420) Response LS on Scheduling Location in Advance to reduce Latency

Release:	Rel-17

Work Item:	5G_eLCS_Ph2

Source:	SA2

To:	RAN2

Cc:	RAN1, RAN3

Contact Person:

Name:                   Ming Ai

E-mail Address:   aiming (at) catt (dot) cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

SA2 thanks RAN2 for their response LS on Scheduling Location in Advance to reduce Latency. SA2 discussed the questions and agreed the following answers.

1.	Definition of "scheduled location time":

The definition of scheduled location time is not fully clear to RAN2 since the SA2 endorsed CR describes the scheduled location time differently in different parts of the CR and there is a different description of scheduled location time in the SA2 LS itself. RAN2 would like to have a clear definition of scheduled location time first as this may impact what is signalled to UE and/or NG-RAN. Please see below for detailed comments:

In section 4.1c it says "The request includes the scheduled location time T" which is described as the start of location preparation phase. So, time T is the scheduled location time.

Section 4.1c also says "the scheduled location time allows an external LCS Client, AF or the UE to specify a time in the future at which a current location of the UE is to be obtained". However, in Figure 4.1c-1, the time when the LCS client, AF or UE obtains the location is T+t2. The time T in the figure is shown as the time at which the UE or NG-RAN obtains the location measurements.

The SA2 LS description also states "…support scheduling of location of a target UE in advance using a scheduled location time at which location measurements for the target UE would be obtained by the UE (in the case of DL measurements) and/or NG-RAN (in the case of UL measurements)". This seem to align with the time T in Figure 4.1c-1 which show it as a scheduled measurement time.

In section 6.3.1, step 25 describes the scheduled location time as the time at which the LMF must obtain the UE location, not the time at which the LCS client receives the location or the time at which the LMF schedules the measurement.

Answer 1: SA2 agrees that the definition of Scheduled Location Time is a future global time (e.g. UTC) at which a UE is to be located. The location of the UE at the Scheduled Location Time needs to be obtaind. It is used by LMF to determine the time to trigger positioning procedures.

2.  	In section 6.1.2, there is the following editor’s note:

"Editor's note: Feedback from RAN is needed to verify whether location measurements can be scheduled 		to occur at a UE or NG-RAN at a specific scheduled location time."

Please clarify the requirement whether measurements in UE/NG-RAN need to be scheduled at the scheduled location time received from 5GC or at a time before the scheduled location time received from 5GC.

Answer 2: The location measurements should be scheduled by LMF based on the Scheduled Location Time, e.g. the LMF triggers the measurements happen near the scheduled location time T. SA2 confirms that the Scheduled Location Time is not sent to RAN and UE, but from system design perspective, SA2 would like to ask RAN WGs to investigate whether such time provide by CN to RAN and/or UE could help the reduction of the LCS latency.

3.	Can SA2 clarify the time format used for the scheduled location time T which is provided to an LMF (e.g., UTC, etc.)?

Answer 3: It is SA2’s understanding that the scheduled location time T is a UTC time. However, the actual format of parameter T is under Stage-3’s remit.

4.	For a Deferred 5GC-MT-LR for periodic location events, RAN2's understanding is that a scheduled location time may apply for the first periodic location report only. For each succeeding periodic location report, the

"scheduled location time" is equivalent to the periodic reporting interval. Please confirm whether RAN2's understanding is correct, or whether different location times T can apply for succeeding periodic location reports.

Answer 4: The “scheduled location time” is applied to the first periodic location report only. The periodic reporting interval is controlled by the existing parameter of “the time interval between successive location reports”.

5.	Clause 4.1c says "A scheduled location time can be used with a 5GC-MT-LR, 5GC-MO-LR or deferred 5GC-MT-LR for periodic or triggered location events."

RAN2 would like to understand how a scheduled location time can be applicable to triggered location events given its sporadic nature.

Answer 5: The scheduled location time applies to periodic location event report. Whether it applies to triggered event type still under discussion in SA2.

2. Actions:

To SA2 group.

ACTION: 	SA2 kindly asks RAN2 to take the above information into account.

3. Date of Next TSG-SA2 Meetings:

TSG-SA2 Meeting #146-e		16 - 27 August 2021			Electronic Meeting

TSG-SA2 Meeting #147-e		18 - 22 October 2021			Electronic Meeting