SA WG2 Meeting #145e	S2-2104482r04

# May 17 – 28th, 2021; Elbonia               		(revision of S2-2102162r03)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	network action related with the UE not using the EASDF's IP address for its DNS Query

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: Issue that we cannot guarantee that the UE uses the EASDF's IP address for its subsequent DNS Query

# 1 Discussion

# 2 Proposal

It is proposed to update TS 23.548 as follows

FIRST CHANGE only the part of the clause 6.2.3.2.2 that is modified by the PCR is shown, the rest of the clause is unchanged by the PCR

2.	During the PDU Session Establishment procedure, the SMF selects EASDF as described clause 6.3 of TS 23.501 [2].

If a UE uses for its DNS queries an IP address different from the EASDF's IP address provided by the SMF, the SMF may, based on local policies, configure the UPF to detect DNS traffic sent by the UE to a different DNS server than the EASDF; this corresponds to a PDR referring to the ports used by DNS protocols (53, 853) and to the EASDF's IP address and possibly targeting FQDN(s) of interest for Edge Computing and with a corresponding FAR and/or URR allowing to carry out the actions described in step 7.

NOTE: 	This mechanism is applicable only to DNS requests sent by the UE over well known ports;

The SMF includes the IP address of the EASDF as DNS server in PDU Session Establishment Accept message as in step 11 of clause 4.3.2.2.1 of TS 23.502 [3]. The UE configures the EASDF as DNS server for that PDU Session.

3	The SMF invokes Neasdf_DNSContext_Create Request (UE IP address, DNN, callback URI, DNS message handling rules) to the selected EASDF.

This step is performed before step 11 of PDU Session Establishment procedure in clause 4.3.2.2.1 of TS 23.502 [3].

The EASDF creates a DNS context for the PDU Session, and stores the UE IP address, the callback URI and DNS message handling rule(s) into the context.

The EASDF is provisioned with the DNS message handling rule(s), before the DNS Query message is received at the EASDF or as a consequence of the DNS Query reporting.

4.	The EASDF invokes the service operation Neasdf_DNSContext_Create Response.

65.	The SMF may invoke Neasdf_DNSContext_Update Request (EASDF Context ID, DNS message handling rules) to EASDF. The update may be triggered by UE mobility, e.g. when UE moves to a new location, or by a reporting by EASDF of a DNS Query with certain FQDN, or, the update may be triggered by insertion/removal of Local PSA, e.g. to update rules to handle DNS messages from the UE or by new PCC rule information.

6.	The EASDF responds with Neasdf_DNSContext_Update Response.

7.	The UE sends DNS Query message.

To handle the case where the UE does not send the DNS Query message to the EASDF, in addition to the actions of step 2, the SMF may, based on local policies,

-	further request (via URR) the UPF to notify the SMF when such traffic has been detected. The corresponding information may be then provided by the SMF to NF(s) that have subscribed about receiving it

NOTE 1:	This option may be used e.g. for logging or trouble shooting purpose.

-	further request the UPF to forward non-ciphered (i.e. on port 53) DNS traffic from the UE that is not targeting the EASDF's IP address on a tunnel to the EASDF where the source address of the tunnel is that of the UE and the destination address that of the EASDF; The EASDF may be instructed via DNS handling rules to forward the DNS requests received in such tunnel without changing the target DNS server address  but after having put its own IP address as source of the DNS request and possibly added an EDNS ECS option.

NOTE 2: 	This option only applies when the DNS traffic is not ciphered and allows the EASDF to see the DNS response and to notify the SMF per DNS handling rules targeting DNS responses; based on such notifications the SMF can take actions such as to trigger SSC mode 2 /mode 3 or to insert an UL CL or a BP in the data path of the PDU Session.

The local policies mentioned above may be based on the (DNN, S-NSSAI).

The rest of the procedure assumes the UE has indeed used the EASDF as its network DNS resolver.

NEXT CHANGE (2)

End of changes