3GPP TSG-WG SA2 Meeting #145E e-meeting 	S2-2104507r02

Elbonia, May 17– 28, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon, Samsung

Title:	Clarification on UE indicating support for refreshing cached EAS information

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract: This contribution introduces that UE indicates its support for refreshing cached EAS information during PDU Session Modification procedure.

# 1. Introduction/Discussion

In last SA2#144E meeting, it was approved that UE indicates its support for refreshing cached EAS information during PDU Session Establishment procedure as described in TS 23.548. This proposal proposes that UE may also indicate its support for refreshing cached EAS information during PDU Session Modification procedure when the UE moves from EPS to 5GS with N26.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.548.

# * * * * First change * * * *

#### 6.2.3.3	EAS Re-discovery Procedure at Edge Relocation

The support for EAS rediscovery indication procedure enables the UE to refresh stale EAS information stored locally so that the UE can trigger EAS discovery procedure to discover new EAS information.

For PDU Session with Session Breakout connectivity, if the UE indicates its support for refreshing stale EAS information to the SMF during the PDU Session Establishment procedure or when the UE moves from EPS to 5GS using PDU Session Modification procedure for EPC to 5GC handover, the SMF may send to the UE, the impact field with the EAS rediscovery indication (see the step 2 of Figure 6.2.3.3-1) after the insertion/change/removal of an L-PSA, based on AF influence or its local configuration, using the PDU Session Modification procedure.

This procedure is used by the SMF to trigger the EAS rediscovery procedure when a new connection to EAS need to be established. It applies to both Session Breakout using ULCL and Session Breakout using BP.

Figure 6.2.3.3-1: EAS re-discovery procedure at Edge relocation

During a previous EAS Discovery procedure on this PDU Session the UE may have EAS information (i.e. EAS IP address corresponding to an EAS FQDN locally stored, e.g. acquired during the previous connection with the EAS (for more information see Annex C UE considerations for EAS (re)discovery).

1a.	Due to the UE mobility the SMF triggers L-PSA insertion, change or removal for the PDU Session. The insertion, change or removal of L-PSA triggers EAS rediscovery.

1b. The AF triggers EAS relocation e.g. due to EAS load balance or maintenance, etc. and informs the SMF the related information as described in clause 4.3.6 AF influence on traffic routing procedure in TS 23.502 [3].

2.	This step may be performed as part of step 1a/1b. The SMF performs the network requested PDU Session Modification procedure from the step 3b-11b as defined in clause 4.3.3.2 TS 23.502 [3].

If the UE has indicated that it supports to refresh  EAS information stored locally corresponding to the impact field per the EAS rediscovery indication from network, the SMF may send the impact field with the EAS rediscovery indication. SMF determines the impacted EAS(s) which need be rediscovered as the following:

-	If an L-PSA is inserted/relocated/removed, the SMF determines the impact field, which is associated with the local DN to be inserted, relocated or removed and identified by FQDN(s) or IP address range(s) of the old EAS, based on the association between FQDN(s)/IP address range(s) and DNAI received from AF via AF influenced traffic steering enforcement control information in the PCC rules or SMF local configuration on the local DN.

-	For AF triggered EAS rediscovery, the AF may indicate the EAS rediscovery for the impacted applications, which are identified by FQDN(s), to the SMF via the AF influence on traffic routing procedure.

The SMF sends PDU Session Modification Command (EAS rediscovery indication, [impact field]) to UE. The EAS rediscovery indication indicates to refresh the cached EAS information. The impact field is used to identify which EAS(s) information need to be refreshed. The impact field includes the local DN information corresponding to the impacted EAS(s), which are identified by FQDN(s) or IP address range(s) of the old EAS(s). If the impact field is not included, it means all EAS(s) information associated with this PDU Session need to be refreshed.

The SMF may choose new DNS settings for the PDU Session and if so, it provides them to the UE as new DNS server (see Option C in clause 6.2.3.2.3). Otherwise the UE uses the existing DNS server for EAS rediscovery.

For the following connection with the EAS(s) for which the EAS rediscovery needs to be executed per the received EAS rediscovery indication and impact field, the UE has been instructed not to use the old EAS information stored locally. Instead it should trigger EAS discovery procedure to get new EAS information as defined in clause 6.2.3.2.

NOTE 1:	It is conditioned to the UE implementation that the indication and impact field trigger an EAS Rediscovery procedure for the application. If the EAS rediscovery indication is not sent to the UE Application Layer, then DNS query to discover a new EAS is triggered only when then Application Layer DNS cache expires. For more information see Annex C.

NOTE 2:	The active connection(s) between the UE and the EAS(s) are not impacted.

# * * * * End of changes * * * *