3GPP TSG-SA WG2 Meeting #145-E	S2-2104523

# Elbonia, 17 – 28 May 2021

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2937 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NSAC with consideration of access type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The UE can be registered for an S-NSSAI via 3GPP access and/or non-3GPP access. It is proposed that the AMF provides access type to the NSACF, and the NSACF takes access type into account based on the operator policy configured at the NSACF. For example, - if the counting methodology is on a terminal count basis, the number of UEs registering for the S-NSSAI can be counted as one. - if the counting methodology is on a registration count basis, the number of UEs registering for the S-NSSAI can be counted as two. However, what counting methodology is used by the NSACF can be invisiable to the AMF. In SA2#141e, SA2 received an LS from CT4 in S2-2006808. The below question was asked by CT4: Question3: If the UE newly registers via 3GPP while it is already registered via non-3GPP access (or vice versa) shall the UE be counted once or twice? SA2 sent an LS response to CT4 in S2-2008238. However, the answer to the question 3 was not provided at that time. Therefore, it is proposed to discuss if a further response to CT4 is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | AMF provides access type to the NSACF, and the NSACF takes access type into account based on the operator policy configured at the NSACF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clear how to count the UE registration via 3GPP acess and/or non-3GPP access. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st change ***

5.15.11	Network Slice Admission Control Function

The Network Slice Admission Control Function (NSACF) monitors and controls the number of registered UEs per network slice for the network slices that are subject to Network Slice Admission Control (NSAC).

The NSACF is configured with the maximum number of UEs per network slice which are allowed to be served by each network slice that is subject to NSAC. The NSACF is also configured with the information indicating which access type is specified for the S-NSSAI subject to NSAC (i.e. 3GPP Access Type, Non-3GPP Access Type, or both).

5.15.11.1	Maximum number of UEs per network slice admission control

The NSACF controls (i.e. increase or decrease) the current number of UEs registered for a network slice so that it does not exceed the maximum number of UEs allowed to register with that network slice. The NSACF also maintains a list of UE IDs registered with a network slice that is subject to NSAC. When the current number of UEs registered with a network slice is to be increased, the NSACF first checks whether the UE Identity is already in the list of UEs registered with that network slice and if not, it checks whether the maximum number of UEs per network slice for that network slice has already been reached. The NSACF takes access type into account for increasing and decreasing the number of UEs per network slice.

The AMF triggers a request to NSACF for maximum number of UEs per network slice admission control when the UE's registration status for a network slice subject to NSAC may change, i.e. during the UE Registration procedure in clause 4.2.2.2.2 in TS 23.502 [3], UE Deregistration procedure in clause 4.2.2.3 in TS 23.502 [3], Network Slice-Specific Authentication and Authorisation procedure in clause 4.2.9.2 in TS 23.502 [3], AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure in clause 4.2.9.3 in TS 23.502 [3], and AAA Server triggered Slice-Specific Authorization Revocation in clause 4.2.9.4 in TS 23.502 [3].

The UE can register for an S-NSSAI via 3GPP access and/or non-3GPP access as described in clause 5.15.5.2.1. The AMF triggers a request to NSACF for maximum number of UEs per network slice admission control for each registration over an Access Type. The AMF provides the current access type to the NSACF when triggering a request to NSACF. The NSACF takes access type into account for increasing and decreasing the number of UEs per network slice. For example, if the information configured in the NSACF includes 3GPP Access Type only, the NSACF counts registration via 3GPP Access Type only. If the information configured in the NSACF includes both Access Types, and the UE newly registers via 3GPP access while the UE is already registered via non-3GPP access (or vice versa), the NSACF, based on the operator policy, determines whether the UE is counted once or twice.

*** End of the changes ***