3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-210453901

# Elbonia, May 17 – 28, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2940 | rev | 01 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on NID |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In case the SNPN provide connectivity for UEs with credentials owned by Credentials Holder, mobility between SNPN and SNPN/PLMN should be supported. Additionally, Service Discovery between SNPN and Credentials Holder should be supported. These procedures require the SNPN should be unique identified. As a result, the NID shall be assigned through Coordinated assignment model instead of Self-assignment mode, so that the SNPN can be uniquely identified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add a NOTE to state that SNPNs that support UE access using credentials owned by the Credentials Holder are assumed to use NIDs that are unique amongst the involved SNPNs and Credential Holders |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The SNPN cannot be unique identified in case the SNPN provide connectivity for UEs with credentials owned by Credentials Holder. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 5.30.2.9.1	General

SNPNs may support UE access using credentials owned by a Credentials Holder separate from the SNPN. In this case the Session Management procedures (i.e. PDU Sessions) terminate in an SMF in the SNPN.

NOTE:	SNPNs that support UE access using credentials owned by the Credentials Holder are assumed to use NIDs that are unique amongst the involved SNPNs and Credential Holders.

# * * * * End of changes * * * *