# 3GPP TSG-SA WG2 Meeting #145E	S2-2104622r06

# 17th May – 28th May 2021, Electronic												(revision of S2-210XXX)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2849 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Registration procedure for UE onboarding SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, Nokia, Nokia Shanghai Bell, Ericsson, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| ` |  |  |  |  |  |  |  |  |  |  |
| Reason for change: |  | Implement the conclusion from clause 8.4.1 in TR 23.700-07 to support a registration procedure that enables support for UE onboarding using Default UE credentials with an ON-SNPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new clause is introduced to describe the modifications to the General Registration procedure described in clause 4.2.2.2.2 to support UE registration with Onboarding SNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Registration procedure for UE onboarding is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.2.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR... CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*************** Start of changes ***************

4.2.2.2.X	Registration with Onboarding SNPN

This clause specifies how a UE can register to ON-SNPN for provisioning the UE with SO-SNPN credentials and other information to enable SO-SNPN access as defined in TS 23.501 [2] clause 5.30.2.X.

The Registration procedure for onboarding SNPN shall be supported as specified in clause 4.2.2.2.2 with the following changes to the steps in the call flow represented in Figure 4.2.2.2.2-1:

Figure 4.2.2.2.X-1: Onboarding Registration Procedure

- UE to (R)AN:  AN parameters shall include Onboarding indication if the UE is accessing 5GS for Onboarding.  The registration type "SNPN Onboarding" indicates that the UE wants to perform SNPN Onboarding Registration (i.e., allows the UE to access an ON-SNPN for the purpose of provisioning the UE with SO-SNPN credentials). For SNPN Onboarding Registration, an Onboarding SUCI shall be included derived from the Onboarding SUPI, as described in TS 23.501 [2] clause 5.30.2.x.2.6..

NOTE:	 When the UE is performing SNPN Onboarding Registration, the UE does not include a Requested NSSAI as the UE is not pre-configured with a S-NSSAI for the purpose of UE onboarding in the ON-SNPN.

2.	Based on the onboarding indication in step 1, the RAN selects an AMF as described in TS 23.501 [2] clause 6.3.5.

3.	(R)AN to AMF: The N2 message contains the Registration Request as described in Step 1.

4-7.	 Skipped.

8.	 When the AMF receives a NAS Registration Request with the 5GS Registration Type set to "SNPN Onboarding", the AMF knows that the UE is registered for onboarding, so the AMFstarts functionality to restrict the network usage to only onboarding for this registered UE, the AMF selects an AUSF as described in TS 23.501 [2] clause 6.3.4. Based on ON-SNPN policies, the AMF may start an implementation specific timer configured for UE Onboarding.

9.	 The authentication is performed as described in TS 33.501 [15]. For Onboarding registration, the AUSF in ON-SNPN does not select a UDM but instead it selects a DCS for authentication of the UE. Once the authentication is completed, the AUSF provides relevant security related information to the AMF.

10-20.	 Skipped.

21.	AMF to UE: The AMF sends a Registration Accept message to the UE indicating that the Registration Request for Onboarding SNPN has been accepted. The AMF does not include the allowed NSSAI.

21b.	Skipped.

22.	UE to AMF: The UE sends a Registration Complete message to the AMF.

23-25.	Skipped.

*************** End of changes ***************