3GPP TSG-WG SA2 Meeting #145E e-meeting  	S2-2104628r4

# Elbonia, May 17 – 28, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2957 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding protocol stacks for trusted WLAN access for N5CW device |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5WWC |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current specification, there are neither control plane nor user plane protocol stacks for trusted WLAN access for N5CW device. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the control and user plane protocol stacks for trusted WLAN access for N5CW device. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Both control and user plane protocol stacks for trusted WLAN access for N5CW device are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.x (new), 8.3.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 8.2.x	Control Plane for Trusted WLAN Access for N5CW Device

Figure 8.2.x-1: Control Plane for trusted WLAN access for N5CW device

The EAP protocol applies only for performing EAP-based access authentication procedure to connect to a trusted WLAN access network.

# * * * * Second change * * * *

### 8.2.y	 User Plane for Trusted WLAN Access for N5CW Device

Transport

Non-3GPP (WLAN)

Non-3GPP (WLAN)

Lower layers

Lower layers

N3 Stack

N3 Stack

N5CW Device

TWAP

TWIF

UPF

Yt'

Yw

N3

N9 Stack

Relay

N9 Stack

N9

UPF (PDU Session Anchor)

PDU Layer

PDU Layer

Transport

Transport

Relay

Application

N6

Transport

Non-3GPP (WLAN)

Non-3GPP (WLAN)

Lower layers

Lower layers

N3 Stack

N3 Stack

N5CW Device

TWAP

TWIF

UPF

Yt'

Yw

N3

N9 Stack

Relay

N9 Stack

N9

UPF (PDU Session Anchor)

PDU Layer

PDU Layer

Transport

Transport

Relay

Application

N6

Legend:

-	Transport: this layer refers to the transport of PDUs between the N5CW device and TWIF (see clause 4.2.8.5.4).

In this release of the specification, Trusted WLAN Access for N5CW Device only supports IP PDU Session type.

Figure 8.2.y-1: User Plane for trusted WLAN access for N5CW device

# * * * * End of changes * * * *