3GPP TSG-SA2 Meeting #145E	S2-2104635

# May 17th - May 28th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2960 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Time Synchronization service exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-05-17 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The time synchronization service exposure in clause 5.20 conflicts what has been agreed in clause 5.27.1.8 in CR2791. The former assumes that the Nnef_ParameterProvision service is used to store the time synchronization parameters to the UDM, whereas the latter describes that the NEF stores the Notification Target Address in the UDR. It is proposed to follow the principles in CR2791. See S2-2104637 for a complementing CR to TS 23.502. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the time synchronization service in clause 5.20 that uses the NEF parameter provisioning service. Add a new event for "UE availability and capabilities for time synchronization service" under the Monitoring Events exposure in clause 5.20. Add a reference to TS 23.502 for the service parameters to control the (g)PTP time synchronization service and remove the service parameters from 5.27.1.8 to avoid duplication. Separate the (g)PTP time synchronization and 5G reference time distribution into different services. Correct the procedure in 5.27.1.8 for setting up the QoS flows for (g)PTP traffic. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.20, 5.27.1.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** BEGIN 1st CHANGE *****

5.20	External Exposure of Network Capability

The Network Exposure Function (NEF) supports external exposure of capabilities of network functions. External exposure can be categorized as Monitoring capability, Provisioning capability, Policy/Charging capability and Analytics reporting capability. The Monitoring capability is for monitoring of specific event for UE in 5G System and making such monitoring events information available for external exposure via the NEF. The Provisioning capability is for allowing external party to provision of information which can be used for the UE in 5G System. The Policy/Charging capability is for handling access and mobility management, QoS and charging policies for the UE based on the request from external party. The Analytics reporting capability is for allowing an external party to fetch or subscribe/unsubscribe to analytics information generated by 5G System (this is further defined in TS 23.288 [86]).

Monitoring capability is comprised of means that allow the identification of the 5G network function suitable for configuring the specific monitoring events, detect the monitoring event, and report the monitoring event to the authorised external party. Monitoring capability can be used for exposing UE's mobility management context such as UE location, reachability, roaming status, and loss of connectivity. AMF stores URRP-AMF information in the MM context to determine the NFs that are authorised to receive direct notifications from the AMF. UDM stores URRP-AMF information locally to determine authorised monitoring requests when forwarding indirect notifications. 

Monitoring capability can be used to expose UE's availability for time synchronization service. In this case the NEF may monitor events at the DS-TT(s) and NW-TT regarding IEEE Std 1588 [126] or IEEE Std 802.1AS [104] configuration operation as specified in clause 5.27.1.4 and upon detecting an event from DS-TT or NW-TT, the NEF may report the UE availability for time synchronization event to the authorised external party.

Provisioning capability allows an external party to provision the Expected UE Behaviour or the 5G-VN group information or service specific information to 5G NF via the NEF. The provisioning comprises of the authorisation of the provisioning external third party, receiving the provisioned external information via the NEF, storing the information, and distributing that information among those NFs that use it. The externally provisioned data can be consumed by different NFs, depending on the data. In the case of provisioning the Expected UE Behaviour, the externally provisioned information which is defined as the Expected UE Behaviour parameters in TS 23.502 [3] clause 4.15.6.3 or Network Control parameter TS 23.502 [3] clause 4.15.6.3a consists of information on expected UE movement, Expected UE Behaviour parameters or expected Network Configuration parameter. The provisioned Expected UE Behaviour parameters may be used for the setting of mobility management or session management parameters of the UE. In the case of provisioning the 5G-VN group information the externally provisioned information is defined as the 5G-VN group parameters in TS 23.502 [3] clause 4.15.6.7, and it consists of some information on the 5G-VN group. The affected NFs are informed via the subscriber data update as specified in TS 23.502 [3] clause 4.15.6.2. The externally provisioned information which is defined as the Service Parameters in clause 4.15.6.7 of TS 23.502 [3] consists of service specific information used for supporting the specific service in 5G system. The provisioned Service Parameters may be delivered to the UEs. The affected NFs are informed of the data update.

Policy/Charging capability is comprised of means that allow the request for session and charging policy, enforce QoS policy, apply accounting functionality and requests to influence access and mobility management policies. It can be used for specific QoS/priority handling for the session of the UE, and for setting applicable charging party or charging rate.

Analytics reporting capability is comprised of means that allow discovery of type of analytics that can be consumed by external party, the request for consumption of analytics information generated by NWDAF.

An NEF may support CAPIF functions for external exposure as specified in clause 6.2.5.1.

An NEF may support exposure of NWDAF analytics as specified in TS 23.288 [86].

***** BEGIN 2nd CHANGE *****

5.27.1.8	Exposure of Time Synchronization

5G System supports time synchronization service that can be activated and deactivated by AF. Exposure of time synchronization comprises the following capabilities:

-	The AF may learn 5GS and/or UE availability and capabilities for time synchronization service.

-	The AF may request to control the (g)PTP time synchronization service with specific service parameters for targeted UE(s). The AF controls activation and deactivation of the time synchronization service for the target UE(s).

-	The AF may request to control the 5G reference time discribution for targeted UE(s).

The NEF exposes the 5GS and/or UE availability and capabilities for synchronization service to the AF. The exposed information includes the list of UE identities, and may include the supported time synchronization distribution methods (i.e. supported IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation modes or 5G clock sync). The NEF exposure of UE availability and capabilities for (g)PTP time synchronization service is described in clause 4.15.3.2.x in TS 23.502 [3].

The AF request to control the (g)PTP time synchronization service is sent to the NEF and may target a UE or multiple UEs. The request may be targeted to:

-	Individual UEs identified using GPSI, Group of UEs identified using External Group Identity, or an IP address/Prefix or a MAC address, or any UE accessing the combination of DNN, S-NSSAI.

The request to control the (g)PTP time synchronization service may contain the service parameters as described in clause 4.15.x.2 in TS 23.502 [3].

The request to control the 5G reference time distribution is described in clause 4.15.x.3 in TS 23.502 [3].

Editor's note:	It is assumed that the NG-RAN already provides 5G timing reference time over Uu towards the UE. Additional impact to activate this "Access Stratum-based 5G clock sync" when it is requested by the AF is FFS.

-

Editor's note:	The clock accuracy attribute for time synchronization configuration, NEF exposure of 5G time domain accuracy, AF input regarding requested accuracy and use of time sync accuracy within 5G System is FFS.

The NEF uses the Time Synchronization parameters received from AF to control the time synchronization service. When IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation have been selected, the NEF determines the necessary (g)PTP parameters to activate and control the service in DS-TT(s) and NW-TTs. For this purpose, the NEF uses the PMIC or BMIC to manage the IEEE Std 1588 [126] or IEEE Std 802.1AS [104] operation in the DS-TT or NW-TT, respectively (see clause 5.27.1.4).

Editor's note:	FFS the time synchronization information provided by the AF that may be stored in the UDR and how the NEF may update or revoke information at the UDR for existing or future PDU Sessions.

For handling (g)PTP traffic, the PCF, according to PCC rule authorization, chooses a 5QI and dynamically set the PDB and/or MDBV according to requirements for (g)PTP protocol. The PCF provides the SMF with a PCC rule generated based on the AF request to control the (g)PTP time synchronization service to the NEF. The SMF may take the information in the PCC rule to modify a PDU Session to create or modify or release a QoS flow for transmitting the (g)PTP messages. The PCF acknowledges the policy request to the NEF. The NEF may report the result of the time synchronization request to the AF.

***** END OF CHANGES *****