SA WG2 Meeting SA2#145-e	S2-2105140

17 - 28 May, 2021, Electronic meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.203 | CR | 1135 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Additional authorization functionality in support of MPS for Data Transport Service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Perspecta Labs, CISA ECD, T-Mobile USA INC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MPS2 |  |  |  |  |  | Date: |  |  | May 10, 2021 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | MPS for Data Transport Service allows service users to use a UE with an MPS subscription and a UE without an MPS subscription. Currently, the AF performs all authorization for MPS for Data Transport Service. We add that authorization of subscribed UEs could be done by the PCF using existing procedures. The AF will still need to perform authorization of service users without an MPS subscription via methods that are outside the scope of 3GPP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added the above to clause 6.1.11.5. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not possible to authorize a Service User with an MPS subscription for MPS for DTS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.11.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.401 CR 3642 |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | CR 2971 for TS 23.501 and CR 0602 for TS 23.503 implement similar changes for 5GS |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST AND ONLY CHANGE

#### 6.1.11.5	Bearer priority for MPS for Data Transport Service

MPS for Data Transport Service enables the prioritization of all traffic on the default bearer and other bearers upon AF request. The QoS modification to the default bearer and other bearers is done based on operator policy and regulatory rules by means of local PCRF configuration.

NOTE 1:	If no configuration is provided, MPS for Data Transport Service applies only to the default bearer.

Upon receipt of an MPS for Data Transport Service invocation/revocation request from the UE, the AF or the PCRF authorizes the request. If the UE has an MPS subscription, MPS for Data Transport Service is authorized by the AF or the PCRF, based on AF decision. If the Service User is using a UE that does not have an MPS subscription, the AF authorizes MPS for Data Transport Service.

-	In the case that the AF authorizes the MPS for Data Transport Service request, after successful authorization, the AF sends the MPS for Data Transport Service request to the PCRF over Rx for QoS modifications, including an indication that PCRF authorization is not needed. In this case, the PCRF shall not perform a subscription check for MPS for Data Transport Service requests. The AF also indicates to the PCRF whether the request is for invoking or revoking MPS for Data Transport Service.

-	In the case that the AF does not authorize the MPS for Data Transport Service request, the AF sends the request over Rx to the PCRF for authorization and QoS modifications, including an indication that PCRF authorization is needed. In this case, the PCRF shall perform an MPS subscription check for the MPS for Data Transport Service request. The AF also indicates whether the request is for invoking or revoking MPS for Data Transport Service. The PCRF will inform the AF when the UE does not have an MPS subscription associated with the request.

After successful authorization by either AF or PCRF as described above, the PCRF shall, at the activation of MPS for Data Transport Service over Rx, perform the same steps for QoS modifications as described in clause 6.1.11.3 for the activation of the Priority EPS Bearer Service.

NOTE 2:	To keep the PCC rules bound to the default bearer, the PCRF can either modify the ARP/QCI of these PCC rules accordingly or set the Bind to Default Bearer PCC rule attribute.

The PCRF shall inform the AF that it successfully acted upon the MPS for Data Transport Service invocation/revocation request.

The PCRF shall at the deactivation of MPS for Data Transport Service over Rx perform the same steps described in clause 6.1.11.3 for the deactivation of the Priority EPS Bearer Service.

If the bearers are deactivated for other reasons than an AF request, the PCRF shall notify the AF by terminating the Rx session.

The AF may also request an SDF for priority signalling between the UE and the AF, where the AF includes the Priority indicator over Rx, in order to enable the PCRF to set appropriate QoS values for the signalling bearer.

END OF CHANGES