a3GPP TSG-WG SA2 Meeting #146E e-meeting  	S2-2105696r06

Elbonia, August 16 – 27, 2021

Title:	[DRAFT] LS Reply on Support of Asynchronous Type Communication in N1N2MessageTransfer

Response to:	LS (S2-2105239/C4-212401) on Support of Asynchronous Type Communication in N1N2MessageTransfer from CT4

Release:	Release 17

Work Item:	SBIProtoc17, TEI17

Source:	[Huawei to be] SA2

To:	CT4, CT1

Cc:

Contact Person:

Name:	Haiyang, Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	 TS 23.501 CR 3062

1. Overall Description:

SA2 thanks the LS on support of ATC from CT4. SA2 would like to answer the questions as following:

Q1: What are the use cases targeted by Asynchronous Type Communication? Do they require support of the aforementioned procedures? Does ATC apply for all UEs when activated in AMF?

SA2 Answer 1: The Asynchronous Type Communication could be applied for those procedures/messages where no immediate response from UE is expected and the intended functionality shall not be impacted by the delay of the message delivery. For example, the SMF does not need to get an immediate response during PDU Session Modification for QoS Rule deletion. So, ATC could be applied for Network triggered Service Request and PDU Session modification.  For the procedures/messages requiring immediate response, (for example the positioning related procedure, it is assumed real-time reporting is needed), ATC is not applied. SA2 has agreed the attached CR to clarify this.

Q2: Should ATC be applicable for N1N2MessageTransfer in the aforementioned procedures for all UEs/PDU sessions served by a given AMF or should the AMF configured with ATC be permitted to page UEs for scenarios where the NF service consumer expects a timely response from the UE?

SA2 Answer 2: As mentioned in Answer 1, ATC could be applicable for those procedures when no immediate response from UE is expected and no intended functionality is impacted. Some company mentions there are NAS Timer(s) for several NAS messages but there is no agreement in SA2 to broaden the ATC use case to procedures where the NAS timers would need to be adjusted as the consequence of using ATC.

Q3: If so, how does the AMF know whether ATC can be used for a given current N1N2MessageTransfer request (i.e. whether the NF service consumer expects a timely response from the UE or not)?

SA2 Answer 3: SA2 specifies it in the attached CR that the ATC can apply only when the requesting NF informs the AMF that the AMF is allowed to use ATC.

Q4: Can the AMF use ATC or not for delivering mtData?

SA2 Answer 4: Please also see Answer 1. In principle, ATC could be allowed also on MT Data assuming delivering mtData does not require immediate response from UE.

Q5: Should CT4 consider a solution that enables the NF service consumer of N1N2Message Transfer to indicate if ATC may be applied for the sending of the N1 message, so that the AMF configured with ATC may still page the UE in CM-IDLE state if the sender of the message (e.g. SMF) expects a timely response for the procedure to succeed.

SA2 Answer 5: Yes, CT4 could consider such solution. SA2 has agreed the attached CR to support such solution, where the requesting NF indicates that ATC is allowed and the AMF determines locally whether to apply ATC or not. ATC is allowed only if there isn't any strict response requirements. Besides  the Rel-16 AMF may apply ATC based on its implementation and Rel-17 AMF may implement it based on the indication from the requesting NF, thus there would be no compatibility issue.

2. Actions:

To CT4 and CT1 group.

ACTION: 	SA2 asks CT4 and CT1 group to

Kindly take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #147E		October 18 – 22, 2021			Elbonia

TSG-SA2 Meeting #148E		November 15 – 19, 2021			Elbonia