3GPP TSG-SA WG2 Meeting #146E e-meeting 	S2-2106122r03

Elbonia, August 16 – 27, 2021	(merged with S2-2105892)

Source:	Huawei, HiSilicon, Samsung

Title:	Support of multicast service available within a limited area

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This document do further clarification for supporting of multicast service available within a limited area.

# Discussion

There’re multiple ways to provide the service area to UE: 1) pre-configuration, e.g., in the APP install package; 2) via Service Announcement; 3) via Application-level interaction;

There’re multiple ways to provide the service area to 5GC: 1) pre-configuration, e.g., based on SLA; 2) AF provides during configuration procedure.

There would be no MBS Service with limited service area that is not local MBS Service. So, it is unclear why a local MBS service with limited area needs additional indication addition to the service area information.

Clause 7.2.4.3 specifies the procedures related to limited area MBS session, however, some descriptions in sub-clause 7.2.4.3.2 (i.e., the MBS join and Session establishment procedure) and sub-clause 7.2.4.3.3 (i.e., Handover procedure) are not clear and need to be further clarified especially related to the 5GC individual delivery.

- For 7.2.4.3.2, i.e., the MBS join and Session establishment procedure for multicast service available within a limited area

There could be a case the limited area MBS session cover some cells in non-MBS supporting NG-RAN nodes, while the non-MBS supporting NG-RAN node cannot perceive the MBS service area. In this case, if the SMF subscribe to AMF for the UE location information, it may control the service area more precisely. Then, for the UE involved in 5GC individual MBS traffic delivery, based on the location information provided by NG-RAN node, if the UE camping cell is out of MBS service area, the SMF trigger to delete the associated QoS flow from the associated unicast PDU session. Later, if the UE moves back to a cell belongs to the MBS service area, the SMF trigger to add the associated QoS flow back to the associated unicast PDU session.

- For 7.2.4.3.3, i.e., the handover procedure for multicast service available within a limited area

The following clarifications as per the principles in clause 7.2.3.

Let see the following scenarios, Fig.1-Fig.4 demonstrate the UE moving trajectories start from camping in a source cell belong to the MBS service area. Fig.1 depicts the UE move to a target cell does not belong to the MBS service area. Then, Fig.2 shows the UE still in the multicast group and further mover to a target cell also does not belong to the MBS service area. As UE moving, Fig.3 illustrate this group member UE move to a target cell belong to the MBS service area. Based on the moving trajectory in Fig.3, Fig.4 shows the UE further move to a target cell still in the MBS service area. The source cell and target cell are respectively refers to the camping cells in the source NG-RAN and in the target NG-RAN.

Fig.1. From MBS service area to out MBS service area

- Fig.1 demonstrates the group member UE handover from a source cell in MBS service area to a target cell out of the MBS service area. Base on Fig.1, the following discussions are given:

- For the UE handover from a MBS supporting NG-RAN node.

The handover procedure is performed as defined in clause 7.2.3.2 and 7.2.3.3 with the SMF need detect whether the target cell is out of MBS service area. If the target cell is out of the MBS service area and the source cell in the MBS service area:

- For the Xn/N2 based handover, Source RAN does not includes the MBS Session information, the associated QoS flow information, and the MBS service area to the Target RAN during Handover preparation phase. The SMF does not provides the MBS session information, the associated QoS flow information, and the MBS service area in N2 SM Info to the target NG-RAN.

- For the UE handover from a non-MBS supporting NG-RAN node.

The handover procedure is performed as defined in clause 7.2.3.4 with the SMF need detect whether the target cell is out of MBS service area. If the target cell is out of the MBS service area and the source cell in the MBS service area:

- For the Xn based handover, after handover procedure, the SMF trigger to delete the associated QoS flow in NG-RAN and PSA.

- For N2 based handover, in Nsmf_PDUSession_UpdateSMContext response (N2 SM information) sent from SMF to target NG-RAN. The SMF does not provides the MBS session information, the associated QoS flow information, and the MBS service area in N2 SM Info to the target NG-RAN.

Fig.2. From out MBS service area to out MBS service area

- Fig.2 demonstrates the group member UE handover from a source cell not in MBS service area to a target cell also not in the MBS service area. Base on Fig.2, the following discussions are given:

- For the UE handover from a MBS supporting NG-RAN node.

The handover procedure is performed as defined in clause 7.2.3.2 and 7.2.3.3 with the SMF need detect whether the target cell is out of MBS service area. If the target cell is out of the MBS service area and the source cell also not belong to the MBS service area:

For the Xn/N2 based handover, since no MBS session information in source NG-RAN, no operation for source NG-RAN needed. The SMF does not provides the MBS session information, the associated QoS flow information, and the MBS service area in N2 SM Info to the target NG-RAN.

- For the UE handover from a non-MBS supporting NG-RAN node.

The handover procedure is performed as defined in clause 7.2.3.4 with the SMF need detect whether the target cell is out of MBS service area. If the target cell is out of the MBS service area and the source cell also not belong to the MBS service area:

For the Xn based handover, after handover procedure, same operation for SMF as defined in 7.2.3.4. The SMF does not provides the MBS session information, the associated QoS flow information, and the MBS service area in N2 SM Info to the target NG-RAN.

- Fig.3 demonstrates the group member UE handover from a source cell not in MBS service area to a target cell in the MBS service area. Base on Fig.3, the following discussions are given:

Fig.3. From out MBS service area to MBS service area

- For the UE handover from a MBS supporting NG-RAN node.

The handover procedure is performed as defined in clause 7.2.3.2 and 7.2.3.3 with the SMF need detect whether the target cell is out of MBS service area. If the target cell is in the MBS service area and the source cell is not in the MBS service area:

- For the Xn/N2 based handover in clause 7.2.3.2 and 7.2.3.3, since no MBS session information in source NG-RAN, no operation for source NG-RAN needed. The SMF provides the MBS session information, the associated QoS flow information, and the MBS service area in N2 SM Info to the target NG-RAN.

- For the UE handover from a non-MBS supporting NG-RAN node.

The handover procedure is performed as defined in clause 7.2.3.4 with the SMF need detect whether the target cell is out of MBS service area. If the target cell is in the MBS service area and the source cell is not in the MBS service area:

- For the Xn based handover, after handover procedure, the SMF trigger to setup the associated QoS flow in NG-RAN and PSA.

- For N2 based handover, in Nsmf_PDUSession_UpdateSMContext response (N2 SM information) sent from SMF to target NG-RAN. The SMF provides the MBS session information, the associated QoS flow information, and the MBS service area in N2 SM Info to the target NG-RAN.

- Fig.4 demonstrates the group member UE handover from a source cell belong to the MBS service area to a target cell also belong to the MBS service area. Base on Fig.4, the following discussions are given:

Fig.4. From MBS service area to MBS service area

- For the UE handover from a MBS supporting NG-RAN node.

The handover procedure is performed as defined in clause 7.2.3.2 and 7.2.3.3 with the SMF need detect whether the target cell is out of MBS service area. If the target cell is in the MBS service area and the source cell also in the MBS service area:

For the Xn/N2 based handover in clause 7.2.3.2 and 7.2.3.3. Source RAN includes the MBS Session information, the associated QoS flow information, and the MBS service area to the Target RAN during Handover preparation phase. The SMF provides the MBS session information, the associated QoS flow information, and the MBS service area in N2 SM Info to the target NG-RAN.

- For the UE handover from a non-MBS supporting NG-RAN node.

The handover procedure is performed as defined in clause 7.2.3.4 with the SMF need detect whether the target cell is out of MBS service area. If the target cell is in the MBS service area and the source cell also in the MBS service area:

- For the Xn based handover, after handover procedure, same operation for SMF as defined in 7.2.3.4.

- For N2 based handover, the SMF provides the MBS session information, the associated QoS flow information, and the MBS service area in N2 SM Info to the target NG-RAN.

# Proposal

It is proposed to capture the following changes vs. TS 23.247.

# * * * * First change* * * *

#### 7.2.4.3	Support of multicast service available within a limited area

##### 7.2.4.3.1	Local MBS service area information provided by AF

For local multicast services, the MBS service area information is provided to the UE and the 5GC as specified in clause 7.1.1.1 with the following differences and clarifications:

-	For the Service Announcement, MBS service area information may be included.

-	In step 7, the MBS service area information for a multicast session may be provided by the AF.

##### 7.2.4.3.2	MBS join and Session establishment procedure for multicast service available within a limited area

For the case that the multicast service is only available within a limited area, the UEs outside the MBS service area are not allowed to join the multicast service.

The MBS join and Session establishment for multicast service available within a limited area are performed as specified in clause 7.2.1.3 with the following differences and clarifications:

-	The local multicast session has been configured where the AF provided the MBS service area information as specified in clause 7.2.4.3.1.

-	The UE may have information about local multicast service including MBS service area and local MBS service indication via Service Announcement as specified in clause 7.2.4.3.1.

-	In step 1, if the UE determines that it is in the MBS service area based on the information about local multicast service e.g. obtained via Service Announcement, the UE sends the PDU Session Modification Request (MBS Session ID) as a Join Request to join the multicast group. If the UE determines that it is outside the MBS service area, the UE does not send the Join Request.

-	In step 4, the SMF obtains the MBS service area (i.e. Cell ID list or TAI list) of the indicated MBS session from the MB-SMF, and the SMF determines the indicated MBS session corresponds to local multicast service based on the MBS service area.

-	In step 5, the SMF checks whether the UE is inside or outside the MBS service area.

-	If the SMF detects that the UE is inside the MBS service area, the SMF continues the process of multicast session join as specified in clause 7.2.1.3 with the following additions:

-  The SMF sends the UE a PDU Session Modification Command indicating a Join Accept as a response to the Join Request. The Joint Accept includes the MBS service area (i.e. Cell ID list or TAI list).

The multicast session information sent by the SMF to the NG-RAN includes the MBS service area (i.e. Cell ID list or TAI list).

-	If the SMF detects that the UE is outside the MBS service area, the SMF rejects the multicast session join, so sends a Join Reject to the UE indicating that the MBS join is rejected. The Joint Reject includes the reject reason (outside of local service area) and the MBS service area (i.e. Cell ID list or TAI list).

In this case, the MBS Session establishment (i.e. resources establishment for MBS traffic delivery) for the UE is not performed.

NOTE 1:	There may be the case that the UE determines that it is inside the MBS service area based on the geographical area information or civic address information as MBS service area information provided by the AF, so sends a Join Request but the UE is outside the MBS service area.

NOTE 2:	Which SM NAS message is used to deliver the Join Reject (e.g. PDU Session Modification Reject) is defined in stage 3 specifications.

-	If the Join Request from the UE is accepted and 5GC individual MBS traffic delivery is used, the SMF subscribes to the UE mobility event notification from the AMF (e.g. UE moving into or out of Area Of Interest, which is set byMBS service area), by invoking Namf_EventExposure_Subscribe service operation as specified in clause 5.2.2.3.2 of TS 23.502 [6].

-

-	For the UE that received the Join Reject from the SMF, later, if the UE detects that it is inside the MBS service area based on the MBS service area provided in the Join Reject, the UE sends the PDU Session Modification Request (MBS Session ID) to join the multicast group.

# * * * * Second change* * * *

##### 7.2.4.3.3	Handover procedure with limited area MBS session

At handover, the request and allocation of resources by supporting source and supporting target NG-RAN node shall take the MBS service area information into account.

Based on the UE mobility event notification provided from AMF, if the SMF detects that the UE is outside the MBS service area, the SMF removes the associated QoS Flow(s) used for 5GC individual MBS traffic delivery not to forward the corresponding MBS traffic to the UE.

The Handover procedure for the UE is performed as defined in clause 7.2.3 with the following additions:

-	Before the Handover, the UE is camping at Source RAN and receiving multicast data corresponding to the MBS Session ID.

-

-

-

-

-	For Xn based handover in clause 7.2.3.4, if the target cell is out of MBS service area, after handover procedure the SMF does not trigger the modification of the PDU Session, and delete the associated QoS flow in target NG-RAN and UPF(PSA).

-	For N2 based handover in clause 7.2.3.4, if the target cell is out of MBS service area, the SMF delete the associated QoS flow in UPF(PSA).

-

# * * * * End of changes * * * *