SA WG2 Meeting #S2-146E	S2-2106284

Aug 16 – 27, 2021, Electronic meeting

# Source:	China Mobile, Huawei, Hisilicon, Xiaomi

# Title:	New SID: Study on architecture enhancement for tactile and media services

# Document for:	Approval

# Agenda Item:	9.1.3

3GPP™ Work Item Description

For guidance, see 3GPP Working Procedures, article 39; and 3GPP TR 21.900.
Comprehensive instructions can be found at http://www.3gpp.org/Work-Items

Title: 	Study on architecture enhancement for tactile and media services

Acronym:	FS_AETM

Unique identifier:

1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | X |  |
| No | X |  |  |  | X |
| Don’t know |  | X | X |  |  |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent and child Work Items

| Parent and child Work Items |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one but are not part of the hierarchical structure.}

| Other related Work Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 900027 | Study on supporting tactile and multi-modality communication services (Release 18) | Study of requirement about tactile and multi-modality communication services |
| 890009 | Operation Points for 8K VR 360 Video over 5G | SA4 study to specify operation points as well as new media decoding capabilities to enable support for up to 8K video. |
| 870013 | Feasibility Study on Typical Traffic Characteristics for XR Services and other Media | SA4 study to collect traffic, codecs and protocol characteristics to identify and document additional relevant XR and other media services and related requirements. |
| 800014 | Study on Audio-Visual Service Production (Release 17) | Multiple devices collecting data for the same task with strict KPI requirements. |

3	Justification

Tactile and multi-modality communication services enable multi-modality interactions, combining ultra-low latency with extremely high availability, reliability and security. It can be applied in multiple fields, including: Remote human interaction, tele-operation, social network of robots, industry and commercial IoT services, etc. Multi-modality services. An architecture study in R18 is required to support the scenarios and requirement identified by SA WG1 TACMM.

The typical tactile and multi-modality communication service may include following modalities which may affect the user experience:

-	Video; Audio;

-	Information perceived by sensors, e.g. detection about brightness, temperature, humidity of the environment; equipment working status report; locality or angle report.

-	Haptic data: can be feelings when touching a surface (e.g., pressure, texture, vibration, temperature), or kinesthetic senses (e.g. gravity, pull forces, sense of position awareness).

To support tactile and multi-modality communication services, the 5G system may need to address service requirement of different types of traffic steams with coordinated QoS selection and packet processing,  guaranteed latency and reliability, time synchronization of these parallel information, in order to ensure best service experience. Multiple modality service flows are required to be transmitted from one/multiple application servers to one or multiple UEs at almost the same time in order to enable the integrated processing. The latency for some tactile and multi-modality services are critical. And new QoS parameters need to be introduced. The KPI requirement can be updated based on SA1 progress.

One typical media of tactile and multi-modality is XR (e.g. cloud AR/VR), and in 5G era, mobile multimedia services will contribute more and more traffics to 5G network. All media traffics, in spite of which codecs were used, have some common characteristics. These information are very useful for better transmission control and efficiency. However, currently 5GS uses common QoS mechanisms to handle media services together with other data services without taking full advantage of these information.

-	Packets within a frame have dependency with each other since the application needs all of these packets for decoding the frame. Hence one packet loss will make other correlative packets useless even they are successfully transmitted. For example, XR applications impose requirements in terms of Application Data Units (ADUs), rather than in terms of single packets/PDUs.

-	Packets of same video stream but different frame types (I/P frame) or even different positions in the GoP (Group of Picture) are of different contributions to user experience, so a layered QoS handling within the video stream can potentially relax the requirement thus lead to higher efficiency.

The new QoS parameters or index may be needed for new media services e.g. XR.

In order to help application adapt to network status and provide better QoE, the network information exposure to applications is needed, especially for the media service which has large traffic burst.

Some cooperation with RAN WG may be needed to provide end-to-end solution.

4	Objective

The study item aims atIdentify the system architecture aspects related to support advanced media services, e.g., XR services, and tactile/multi-modality communication services.

Objective1: Enhancements for supporting multi-modality service:

-	Study how to enable presenting related tactile and multi-modal data (e.g., audio, video and haptic data related to a specific time) to the user at a similar time.

Objective 2: Enhancements of network exposure to support interaction between 5GS and application：

-	Support AF requirements for QoS policy control of grouping multi-modality traffic flows per UE or among multiple UEs,

-	Expose QoS information including, e.g., MDBV/burst rate, Burst timing offset, temporary degradation, to the Application to enable traffic adaptation and traffic synchronization.

Objective 3: Enhancement for media service transmission:

-	Identify PDUs belonging to the same media unit (e.g., video frame, Application Data Unit) and the size of media unit, etc.

-	Support media unit level integrated transmission, considering, e.g., packets dependency within a media stream, media unit error rate, media unit delay budget and media unit policy control.

-	Support media unit based QoS framework.

-	Support Layered QoS handling for different media units within a media stream considering importance of packet of media units.

-	Support uplink-downlink transmission coordination to meet RTT (Round-Trip Time) requirements.

Objective 4: Enhancements of Mobility management considering traffic pattern of media services:

-	Handover enhancement considering the traffic pattern of media services to minimizing service impacts during handover.

-	Power saving enhancement to support, e.g. support CDRX enhancement or XR traffic power saving.

Objective 5: Support of network slice for mobile media:

-	Potentially define a network slice type supporting mobile media transmission.

The time for this study item is about 16TUs.

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Proposed Spec no. or series | Type (see note 1) | Rapporteur(s) (see note 2) | For info at TSG# | For approval at TSG# | Remarks |
| New TR 23.7xy | Internal TR | Dan Wang, China Mobile, wangdanyjy@chinamobile.com Hui Ni, Huawei, hui.ni@huawei.com |  |  | The first Rapporteur is responsible for Objective 1&2 and TR editing. The second Rapporteur is responsible for Objective 3&4&5. |

Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs. 
“Internal TR” is intended for 3GPP internal use only whereas “External TR” may be transposed by Ops.

Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as “Remarks”.

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |
| --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# |

6	Work item Rapporteur(s)

Dan Wang, China Mobile, wangdanyjy@chinamobile.com

Hui Ni, Huawei, hui.ni@huawei.com

7	Work item leadership

SA2

8	Aspects that involve other WGs

SA3 for security aspects.

SA4 for media codec aspects.

SA5 for management and charging aspects.

RAN1/2/3 for RAN part enhancements.

9	Supporting Individual Members

Supporting IM name

China Mobile

CATT

China Telecom

China Unicom

CAICT

Huawei

HiSilicon

Spreadtrum Communications

vivo

ZTE

Qualcomm

Xiaomi

OPPO

Intel

Oracle

Broadcom

CBN

ABS

Toyota

Interdigital

Futurewei