SA WG2 Meeting #147E (e-meeting)	S2-2107094r07

Oct 18 – 22, 2021

Title:	[Draft]LS reply on limited service availability of an SNPN

Response to:	S2-2107044/R2-2109114

Release:	Release 17

Work Item:	eNPN, NG_RAN_PRN_enh-Core

Source:	SA2

To:	RAN2

Cc:	CT1

Contact Person:

#### Name:	Xiaowan Ke

#### E-mail Address:	xiaowan.ke@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1. Overall Description:

SA2 would like to thank RAN2 for the LS (S2-2107044/ R2-2109114). SA2 has agreed the following answer for the question raised in the LS (S2-2107044/ R2-2109114):

Question: Can CT1/SA2 to confirm that the R17 SNPN-capable UEs that are not in SNPN Access Mode and R17 Non-SNPN capable UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services via any SNPN.

Answer: SA2 confirms R17 Non-SNPN capable UEs cannot camp on an SNPN cell supporting emergency services to obtain emergency services via any SNPN.

On the other hand, in case the R17 SNPN-capable UE is in limited service state and is not registered in SNPN or PLMN there is no consensus in SA2 on how to interpret the following text that exists in TS 23.501:

TS 23.501 contains the following text in clause 5.30.2.4.1 that exists since rel.16:

“When the UE is set to operate in SNPN access mode the UE only selects and registers with SNPNs over Uu as described in clause 5.30.2.4.”

In clause 5.30.2.4.1

“If the UE is in limited service state, the UE shall attempt to camp on an acceptable cell of any available SNPN supporting emergency calls (irrespective of SNPN ID or GIN) or on any available PLMN supporting emergency calls (irrespective of PLMN ID).”

“If a UE is not set to operate in SNPN access mode, even if it is SNPN-enabled, the UE does not select   and register with SNPNs. A UE not set to operate in SNPN access mode performs PLMN selection procedures as defined in clause 4.4 of TS 23.122 [17]. For a UE capable of simultaneously connecting to an SNPN and a PLMN, the setting for operation in SNPN access mode is applied only to the Uu interface for connection to the SNPN. Clause D.4 provides more details.”

And clause 5.16.4.1:

“For Emergency Services over NR via SNPN, other than eCall over IMS the UEs in limited service state determine that the cell supports Emergency Services over NR from a broadcast indicator in AS and indication that the SNPN supports Emergency Services.”

Hence, SA2 has no consensus on whether R17 SNPN-capable UEs that are not in SNPN Access Mode can or cannot camp on an SNPN cell supporting emergency services to obtain emergency services via any SNPN.

2	Actions

To RAN2

SA2 respectfully asks RAN2 to take above answers into consideration.

3	Dates of next TSG SA WG2 meetings

3GPP SA2#148	from 2021-11-15	to 2021-11-19	Electronic Meeting

3GPP SA2#149	from 2022-02-14	to 2022-02-15	Electronic Meeting