SA WG2 Meeting #147E	S2-2107119

October 18 – 22, 2021, E-Meeting

# Title:	[DRAFT] Reply LS on rejected NSSAI for the maximum number of UE reached when TAIs belonging to different PLMNs

# Response to:	(S2-2107020/ C1-214895)

# Release:	Rel-17

# Work Item:	eNS_Ph2

Source:	[Ericsson, will be SA2]

To:	CT1

Cc:	-

Contact Person:

Name:	Peter Hedman

E-mail Address:	peter (dot) hedman (at) ericsson (dot) com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	-

1. Overall Description:

SA2 thanks CT1 for the LS on rejected NSSAI for the maximum number of UE reached when TAIs belonging to different PLMNs.

CT1 asked:

CT1 is discussing the UE handling on the received rejected NSSAI for the maximum number of UEs reached when the registration area contains TAIs belonging to different PLMNs. There are two options observed:

Option 1: the rejected NSSAI for the maximum number of UEs reached is applicable to all of these PLMNs in the registration area;

Option 2: the rejected NSSAI for the maximum number of UEs reached is applicable to the registered PLMN only.

SA2 answer:

The NSAC procedures are defined separately per S-NSSAI and therefore the NSAC procedure is performed on the registered PLMN and consequently option 2 applies.

2. Actions:

To CT1:

ACTION: 	SA2 asks CT1 to take the above information into account.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		November 15 – 19, 2021		Elbonia

TSG-SA2 Meeting #149E	February 14 – 25, 2022			Elbonia