3GPP TSG-SA2 Meeting # 147E (e-meeting)	S2-2107195r01

# Elbonia, October 18 - 22, 2021											 (revision of )

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0006 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Multicast MBS Session: resolving ENs and cleanup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | #1 Clause 7.2.7 includes the following EN: Editor´s note: Whether this procedure can be combined with clause 7.2.6 will be addressed in the next meeting It is proposed to resolve the EN by combining 7.2.7 to 7.2.6 because they are the same procedure. Separating them may be interpreated that update of QoS in 7.2.6 and update of MBS service area 7.2.7 are different procedures that cannot be executed together executed separately, which is incorrect. #2 For TMGI allocation and deallocation, the service operation names in MB-SMF are Nmbsmf_ TMGI_Allocate and Nmbsmf_ TMGI_Deallocate specified in clause 9.1.2, while the ones in NEF are Nnef_ MBSTMGI_Allocation and Nnef_ MBSTMGI_Deallocation specified in 9.4.2. It is proposed to use allocate/deallocate, which is more suitable in service operation naming. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Merge clause 7.2.7 into clause 7.2.6, and corrections are also made to improve consisteny In clause 9.4.2, update the service operation names to Nnef_MBSTMGI_Allocate and Nnef_MBSTMGI_Deallocate |  |  |  |  |  |  |  |  |
| - |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Risk of interpreting the update of QoS and update of MBS service area as separate procedure that are not executed together which is incorrect Inconsistent service operation names. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.6, 7.2.7, 9.4.2.2, 9.4.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

NG-RAN

AMF

MB-SMF

MB-UPF

1. Decide to establish shared delivery for a multicast session

2. N2 Message ()

3. Nmbsmf_MBSSession_ContextUpdate Request

4. N4mb Session Modification

6. Nmbsmf_MBSSession_ContextUpdate Response

7. N2 Message ()

5. Store AMF ID for the MBS session

### 7.2.6	Multicast session update procedure

Multicast session update procedure is invoked by the AF to update the service requirement and/or MBS Service Area for an ongoing multicast session. For the interaction between AF and MB-SMF, see clause 7.1.1.6 and 7.1.1.7.

Figure 7.2.6-1: Multicast MBS Session update procedure.

1.	This procedure is triggered by the MB-SMF receiving the updated service requirement and/or MBS service area for a multicast MBS Session, see clauses 7.1.1.6 and 7.1.1.7.

2.	The AF providing the updated service area may also inform UEs at application level about the new service area via a service announcement.

NOTE 1: If a UE is located in a radio cell which was previously outside the service area and is now inside the updated service area, the UE can join the multicast service as specified in clause 7.2.1.3.

3.	he MB-SMF update the updated QoS profile and/or the updated MBS Service Area for the multicast MBS session, and invokes Namf_MBSCommunication_N2MessageTransfer service operation (MBS Session ID, N2 SM information (TMGI, QoS profiles for multicast, MBS Service Area, Area Session Id)) to the AMF(s).

4.	The involved AMF sends N2 MBS session request (N2 SM information) to NG-RAN nodes based on the TMGI and RAN node IDs stored in the AMF for the MBS session.

5.	NG-RAN updates the MBS session context, and if only the ARP of QoS parameters is updated, NG-RAN node also updates the QoS parameters of the associating PDU Sessions.

6.	The NG-RAN(s) acknowledges N2 MBS session request by sending an N2 MBS session Response message to the AMF.

7.	The AMF uses the Nmbsmf_MBSSession_ContextUpdate to the MB-SMF.

8.	MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify request (TMGI, QoS profiles for multicast, MBS Service Area, [Area Session ID]) to the SMFs.

.	SMF determines the UE list regarding the TMGI included in the message.

9. [Optional] If 5GC individual MBS traffic delivery is involved, when the MBS Service Area is updated, for all the UEs that have joined the MBS Session, if the SMF does not have the latest UE location, the SMF queries AMF which then query the NG-RAN for the current location of the UE to determine whether the UE is within the updated MBS Service Area.

10. [Optional] if the 5GC individual MBS traffic delivery is involved for the UE, when the QoS is updated  SMF triggers PDU Session Modification procedure for each UE as defined in TS 23.502 [6]. For MBS Service Area update, for each UE that joined the multicast MBS Session, the SMF triggers PDU Session Modification procedure as defined in TS 23.502 [6] with the following enhancement:

-	Towards the UE, the SMF may provide the changed MBS service area in N1 container to the UE. For a UE previously inside the MBS service area but now outside the updated MBS service area of the multicast session, the SMF may also inform the UE in the N1 container that the UE has been removed from the multicast session, and in this case the SMF also requests the UE to remove the associated QoS Flow(s).

- 	Towards the NG-RAN, the SMF may need to update the associated PDU session.

****************** NEXT CHANGES ***************

### 7.2.7	Void

9. Determine affected UEs

****************** NEXT CHANGES ***************

#### 9.4.2.2	Nnef_MBSTMGI_Allocate service operation

Service operation name: Nnef_ MBSTMGI_Allocate

Description: This service is used by the NF Service Consumer to request allocation of TMGI(s), or to refresh the expiry time for already allocated TMGI(s).

Inputs, Required: Number of TMGIs (may be zero if only a refresh of expiry time is requested).

Inputs, Optional: TMGI(s) to be refreshed.

Outputs, Required: TMGIs, Expiry Time, Success or not.

Outputs, Optional: None.

****************** NEXT CHANGES ***************

#### 9.4.2.3	Nnef_MBSTMGI_Deallocate service operation

Service operation name: Nnef_MBSTMGI_Deallocate

Description: This service is used by the NF Service Consumer to request deallocation the TMGI(s).

Inputs, Required: TMGI(s).

Inputs, Optional: None.

Outputs, Required: Success or not.

Outputs, Optional: None.

****************** END CHANGES ***************