3GPP TSG-SA2 Meeting # 147-E	S2-2107223r05

# Elbonia, 18 October 2021 - 22 October 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3260 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Link delay measurement for the end-to-end Transparent Clock |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Tencent, Tencent Cloud |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIOT |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The functional description on how the end-to-end Transparent Clock modifies the correction field to support the path delay measurement is documented in the annex H.4 based on standard IEEE 1588-2019. However, how the end-to-end Transparent Clock supports the link delay measurement is not provided in the annex H.4. Based on the IEEE 1588-2019, if the one-step end-to-end Transparent Clock support the link delay measurement, the end-to-end Transparent Clock needs to separately update the correction field of the PTP Pdelay_Req and PTP Pdelay_Resp messages with each residence time in the 5GS. If the two-step end-to-end Transparent Clock supports the link delay measurement, the end-to-end Transparent Clock needs to separately calculate each 5GS residence time of the PTP Pdelay_Req and PTP Pdelay_Resp messages and add these two 5GS residence time to the correction field of the associated PTP Pdelay_Resp_Follow_UP message. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the annex H.4 to provide the functional description how the end-to-end Transparent Clock support the link delay measurement as : - If the one-step end-to-end Transparent Clock supports the link delay measurement, the end-to-end Transparent Clock separately updates the correction field of the PTP Pdelay_Req and PTP Pdelay_Resp message with each residence time in the 5GS. - If the two-step end-to-end Transparent Clock supports the link delay measurement, the end-to-end Transparent Clock separately calculates each 5GS residence time of the PTP Pdelay_Req and PTP Pdelay_Resp message and add these two residence time to the correction field of the associated PTP Pdelay_Resp_Follow_UP message. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The DS-TT and NW-TT support operating as an end-to-end Transparent Clock can not support link delay measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | H.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1ST CHANGE

# H.4	Path and Link delay measurements

The procedure described in this clause is applicable if DS-TT and NW-TT support operating as a boundary clock or as a time-aware system or as peer to peer Transparent Clock, and when the PTP instance in 5GS is configured to operate as a time-aware system or as a Boundary Clock or as peer to peer Transparent Clock. Whether DS-TT/NW-TT support operating as a boundary clock or as a time-aware system or as peer to peer Transparent Clock (support of the IEEE Std 802.1AS [104] PTP profile) may be determined as described in clause K.2.1.

PTP ports in DS-TT and NW-TT may support the following delay measurement mechanisms:

-	Delay request-response mechanism as described in clause 11.3 of IEEE Std 1588 [126];

-	Peer-to-peer delay mechanism as defined in clause 11.4 of IEEE Std 1588 [126];

-	Common Mean Link Delay Service.

Depending on the measurement mechanisms supported by DS-TT and NW-TT as well as the configured clock mode of 5GS, the PTP ports in DS-TT and NW-TT are configured as follows:

-	PTP ports configured to operate as a time-aware system according to IEEE Std 802.1AS [104] may be configured to use the peer-to-peer delay mechanism or Common Mean Link Delay Service;

-	PTP ports configured to operate as a Boundary Clock according to IEEE Std 1588 [126] may be configured to use the delay request-response mechanism, the peer-to-peer delay mechanism or Common Mean Link Delay Service.

PTP ports in 5GS configured to operate as a peer-to-peer Transparent Clock according to IEEE Std 1588 [126] shall use the peer-to-peer delay mechanism.

PTP ports in 5GS configured to operate as an end-to-end Transparent Clock according to IEEE Std 1588 [126] do not actively participate in path and link measurements mechanisms but shall calculate and add residence time and delay asymmetry information to PTP messages as defined in clause 10.2.2 of IEEE Std 1588 [126].

If DS-TT and NW-TT support operating as an end-to-end Transparent Clock, then the residence time for one-step operation as an end-to-end Transparent Clock for the path and link measurements is calculated as follows:

-	Upon reception of a PTP Delay_Req/Pdelay_Req/Pdely_Resp message from the upstream PTP instance, the ingress TT (i.e. NW-TT or DS-TT) makes an ingress timestamping (TSi) for the message.

-	The ingress timestamp is conveyed to the egress TT via the PDU Session as described in clause H.2.

-	The PTP port in the egress TT then creates egress timestamping (TSe) for the PTP message for external PTP network. The difference between TSi and TSe is considered as the calculated residence time spent within the 5G system for this PTP message expressed in 5GS time.

-	The PTP port in the egress TT modifies the payload of the PTP Delay_Req/Pdelay_Req/Pdelay_Resp message that it sends towards the downstream PTP instance as follows:

-	Adds the calculated residence time to the correction field.

-	Removes Suffix field that contains TSi.

If DS-TT and NW-TT support operating as an end-to-end Transparent Clock, then the residence time for two-step operation as an end-to-end Transparent Clock for the path and link measurements is calculated as follows:

-	Upon reception of a PTP Delay_Req/Pdelay_Req/Pdelay_Resp message from the upstream PTP instance, the ingress TT (i.e. NW-TT or DS-TT) makes an ingress timestamping (TSi) for the message.

-	If the ingress TT receives a Pdelay_Resp message with the twoStepFlag set to FALSE, then the ingress TT modifies the twoStepFlag to TRUE and creates a PTP Pdelay_Resp_Follow_UP message.

-	The ingress timestamp is conveyed to the egress TT via the PDU Session as described in clause H.2.

-	The PTP port in the egress TT then creates egress timestamping (TSe) for the PTP message for external PTP network. The difference between TSi and TSe is considered as the calculated residence time spent within the 5G system for this PTP message expressed in 5GS time. The egress TT then stores the calculated residence time and removes Suffix field that contains TSi before sending the PTP Delay_Req/Pdelay_Req/Pdelay_Resp message towards the downstream PTP instance.

-	Upon reception of the PTP Delay_Resp message associated with the PTP Delay_Req, the egress TT for the PTP Delay_Req message (i.e. the ingress TT for the PTP Delay_Resp message) modifies the payload of the PTP Delay_Resp message that it sends towards the ingress TT of the PTP Delay_Req message (i.e. egress TT for the PTP Delay_Resp message) as follows:

-	Adds the (previously stored) calculated residence time to the correction field.

-	Upon reception (or local creation) of the PTP Pdelay_Resp_Follow_UP message associated with the previously received PTP Pdelay_Resp message, the ingress TT for the PTP Pdelay_Resp_Follow_UP message modifies the payload of the PTP Pdelay_Resp_Follow_UP message that it sends towards the egress TT for the PTP Pdelay_Resp_Follow_UP message as follows:

-	Adds the (previously stored) calculated residence time of the associated PTP Pdelay_Req message to the correction field.

-	Upon reception of the PTP Pdelay_Resp_Follow_UP message associated with the PTP Pdelay_Resp, the egress TT for PTP Pdelay_Resp_Follow_UP message modifies the payload of the PTP Pdelay_Resp_Follow_UP message that it sends towards the downstream PTP instance as follows:

-	Adds the (previously stored) calculated residence time of the associated PTP Pdelay_Resp messages to the correction field.

END OF CHANGES