3GPP TSG-WG SA2 Meeting #147E e-meeting 	S2- 2107250r10

Elbonia, October 18 – 22, 2021	(revision of S2-210XXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0442 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to ML model provisioning and AnLF / MTLF split functionality |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson, ETRI, Lenovo, Motorola Mobility |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Terminology for NWDAF containing MTLF and NWDAF containg AnLF There are different terms used in TS 23.288 for describing an NWDAF containing AnLF: either "NWDAF (AnLF)" or "NWDAF containing AnLF". There are different terms used for an NWDAF containing MTLF. Either "NWDAF (MTLF)", or "NWDAF(MTLF)" or "NWDAF containing MTLF". Note 1 in clause 5.1 is misleading when combined with other specification text from TS 23.288. It reads: "NOTE 1: NWDAF can contain Model Training logical function, Analytics logical function, or both. An NWDAF contains both logical functions and is denoted as NWDAF, unless explicitly mentioned as NWDAF (AnLF) and NWDAF (MTLF)." If NWDAF (AnLF) means an NWDAF containing only AnLF as suggested by bold text above, then some specification text in TS 23.288 is incorect, e.g. in 5.2 where text related to discovery of NWDAF (AnLF) via NRF would be restricted to NWDAF (AnLF) while it obviously also applies to NWDAF containing both AnLF and MTLF. For the reasons above, proposal is to only use terminology "NWDAF containing AnLF" and "NWDAF containing MTLF". It also needs to be clarified that an "NWDAF containing AnLF" can also contain MTLF, and the other way round. Discovery for NWDAF containing MTLF Clause 5.1 indicates the following: NOTE 4: The NWDAF(MTLF) registers the ML model provisioning service when it has trained ML model(s) for the Analytics ID. And later, clause 5.2 specifies that NWDAF containing MTLF registers ML model filter information to NRF, and then NRF provides the profile of NWDAF containing MTLF to the consumer, such profile containing ML model filter information for the available models. Having NWDAF containing MTLF registering the ML model filter information will lead to huge growth of the NWDAF profile in NRF: indeed, whenever a new model is available with different ML model filter information, the related ML model filter information will be added to the NWDAF profile in NRF. It should also be noted that ML model filter information can contain many different parameters. While S-NSSAI and area of interest are the most obvious ones, other ML model filter information can be used. The ML model filter information parameters are the same as the Analytics filter information. Just looking at observed service experience analytics, filters can be application ID, S-NSSAI, NSI ID, area of interest, DNN, DNAI, RAT type, Frequency, application server address and UPF anchor ID. Any combination of such filters should also be taken into account. Which means that the NWDAF containing MTLF will probably registers a huge number of available ML models (via the corresponding ML model filter information),as many as there are different combinations of ML model filter information requested by ML model consumers. On the other hand, the NWDAF containing AnLF, in Rel-17, is locally configured with (a set of) IDs of NWDAFs containing MTLF and the Analytics ID(s) supported by each NWDAF containing MTLF to retrieve trained ML models. This makes discovery of NWDAF containing MTLF via NRF less relevant, and not compatible with configuration in the NWDAF containing AnLF. It is proposed to keep NRF registration aspects for NWDAF containing MTLF minimal in Rel-17, limited to the registration of the Nnwdaf_MLModelProvision service, as suggested by clause 5.1. And to revisit this issue in Rel-18, possibly in conjunction with investigating the storage of ML models which have been deferred from Rel-17. In clause 6.2A.2: The optional parameters "Subscription Correlation ID (for modification of an existing ML model subscription)" and "expiry time" are missing, while they are listed in clause 7.5.2. Alignment between the two clauses is needed. Others NOTE 3 in clause 5.1 should be expanded to clarify that "ML Model provisioning/sharing between multiple NWDAFs containing MTLF is not specified in this Release of the specification", in alignment with 6.2A.0. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Terminology "NWDAF containing AnLF" and "NWDAF containing MTLF" is introduced in a consistent way. Parameters for ML model provisioning service are modified. Discovery of MWDAF containing MTLF is simplified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent specification. Possible deployment limitations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 3.2,, 5.2, 6.2A.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [2] and TS 23.503 [4] apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

NEXT CHANGE

5.2	NWDAF Discovery and Selection

The NWDAF service consumer selects an NWDAF that supports requested analytics information and required analytics capabilities and/or requested ML model information by using the NWDAF discovery principles defined in clause 6.3.13 of TS 23.501 [2].

Different deployments may require different discovery and selection parameters. Different ways to perform discovery and selection mechanisms depend on different types of analytics/data (NF related analytics/data and UE related analytics/data). NF related refers to analytics/data that do not require a SUPI nor group of SUPIs (e.g. NF load analytics). UE related refers to analytics/data that requires SUPI or group of SUPIs (e.g. UE mobility analytics).

In order to discover an NWDAF (AnLF) using the NRF:

-	If the analytics is related to NF(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 1:	If the selected NWDAF cannot provide the requested data analytics, e.g. due to the NF(s) to be contacted being out of serving area of the NWDAF, the selected NWDAF might reject the analytics request/subscription or it might query the NRF with the service area of the NF to be contacted to determine another target NWDAF.

-	If the analytics is related to UE(s) and the NWDAF service consumer (other than an NWDAF) cannot provide an Area of Interest for the requested data analytics, the NWDAF service consumer may select an NWDAF with large serving area from the candidate NWDAFs from discovery response. Alternatively, in case the consumer receives NWDAF(s) with aggregation capability, the consumer preferably selects an NWDAF with aggregation capability with large serving area.

NOTE 2:	If a selected NWDAF cannot provide analytics for the requested UE(s) (e.g. the NWDAF serves a different serving area), the selected NWDAF might reject the analytics request/subscription or it might determine the AMF serving the UE as specified in clause 6.2.2.1, request UE location information from the AMF, and query the NRF with the tracking area where the UE is located to discover another target NWDAF serving the area where the UE(s) is located.

If the NWDAF service consumer needs to discover an NWDAF that is able to collect data from particular data sources identified by their NF Set IDs or NF types, the consumer may query NRF providing the NF Set IDs or NF types in the discovery request.

NOTE 3:	The NF Set ID or NF Type of a data source serving a particular UE, can be determined as indicated in Table 5A.2-1.

In order to discover an NWDAF that has registered in UDM for a given UE:

-	NWDAF service consumers or other NWDAFs interested in UE related data or Analytics, if supported, may make a query to UDM to discover an NWDAF instance that is already serving the given UE.

If an NWDAF service consumer needs to discover NWDAFs with data collection exposure capability, the NWDAF service consumer may discover via NRF the NWDAF(s) that provide the Nnwdaf_DataManagement service and their associated NF type of data sources or their associated NF Set ID of data sources as defined in clause 6.3.13 of TS 23.501 [2].

In order to discover an NWDAF(MTLF):

- The NWDAF containing MTLF shall include the ML model provisioning services (i.e., Nnwdaf_MLModelProvision, Nnwdaf_MLModelInfo) as one of the supported services during the registration in NRF when trained ML models are available for one or more Analytics ID(s) and may provide a (list of) Analytics ID(s), the ML model Filter for the trained ML model per Analytics ID, if available.

-	During the NWDAF(MTLF) discovery, the NRF may return one or more candidate NWDAF(MTLF) instance(s) to the NF consumer and each candidate NWDAF(MTLF) instance includes the Analytics ID(s), the ML model Filter information for the trained ML model per Analytics ID, if available.

A PCF may learn which NWDAFs being used by AMF, SMF and UPF for a specific UE, via signalling described in clause 4.16 of TS 23.502 [3]. This enables a PCF to select the same NWDAF instance that is already being used for a specific UE.

NEXT CHANGE

6.2A.2	Contents of ML Model Provisioning

The consumers of the ML model provisioning services (i.e. an AnLF of NWDAF) as described in clause 7.5 and clause 7.6 may provide the input parameters as listed below:

-	Information of the analytics for which the requested ML model is to be used, including:

-	A list of Analytics ID(s): identifies the analytics for which the ML model is used.

-	[OPTIONAL] ML model Filter Information: enables to select which ML model for the analytics is requested, e.g. S-NSSAI, Area of Interest. Parameter types in the ML Model filter information are the same as parameter types in the Analytics Filter Information which are defined in procedures.

-	[OPTIONAL] Target of Analytics Reporting: indicates the object(s) for which ML model for the analytics is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	[OPTIONAL] ML Model target period: indicates time interval [start, end] for which ML model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g. via UTC time).

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the ML model provider NWDAF with this subscription.

-	[OPTIONAL] Subscription Correlation ID (for modification of an existing ML model subscription).

-	[OPTIONAL] Expiry time.

The ML model provider NWDAF (i.e. an MTLF of NWDAF) provides to the consumer of the ML model provisioning service operations as described in clause 7.5 and 7.6, the output information as listed below:

-	(Only for Nnwdaf_MLModelProvision_Notify) The Notification Correlation Information.

-	ML model information, which includes the ML model file address (e.g. URL or FQDN) for the Analytics ID(s).

-	[OPTIONAL] Validity period: indicates time period when the provided ML model information applies.

-	[OPTIONAL] Spatial validity: indicates Area where the provided ML model information applies.

NOTE:	Spatial validity and Validity period are determined by MTLF internal logic and it is a subset of AoI if provided in ML model Filter information and of ML Model target period, respectively.

END OF CHANGES