3GPP TSG-WG SA2 Meeting #147E e-meeting	S2-2107272

e-Meeting, 18th-22nd October, 2021

# Title:	Reply LS on Slice list and priority information for cell reselection

# Response to:	LS S2-2107039/R2-2108928 on Slice list and priority information for cell reselection from RAN 2

# Release:	Rel-17

# Work Item:	eNS_Ph2, NR_Slice-Core

Source:	SA2

To:	RAN2

Cc:	RAN3 and CT1

Contact Person:

Name:	Dan, Wang

E-mail Address:	wangdanyjy at chinamobile dot com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	Rev of S2-2107556

1. Overall Description:

SA2 thanks RAN2 for their LS on Slice list and priority information for cell reselection.

The questions from RAN2 are as following:

1. Can UE NAS provide to UE AS a list of slices (or slice group(s)), each with its corresponding priority for cell reselection evaluation in RRC_IDLE/RRC_INACTIVE?

2. Can the concept of Slice group and its signalling (Slice Group and its identifier) be supported using NAS signalling?

SA2 has some discussion and the answers are below:

For Q1, Yes, this is possible to enable, but there were opionions expressed that the prioritization can be fully UE internal (i.e. implementation specific) and other opinions that there is a need to enable a network control with some deterministic options for such prioritization. The attached CR (as basis for further work) provides an option enabling such network control as well as an option for allowing UE implementation specific prioritization. SA2 would like to get feedback on this from other WGs.

For Q2, SA2 understands that the usage of such Slice Group is to avoid the broadcast of S-NSSAIs, and the attached CR (as basis for further work) provides the means to extend the NAS signalling (Registration procedure and UE Configuration Update procedure) to enable the UE to retrieve the Slice Group that the S-NSSAIs of the Configured NSSAI belongs to. The association of S-NSSAIs to Slice Group can be updated at any time.

The name of the Slice Group was discussed and it was discussed whether the Slice Group should be tied to cell reselection or whether the same Slice Groups also can be used by other AS functionality like the slice based RACH functionality, as to allow the usage to be more than cell reselection the agreed CR uses the name "Network Slice RRM Group" (NSRRMG). If another name is more suitable, SA2 will align the naming.

SA2 question to RAN2:

SA2 would like to understand how RAN2 came to the conclusion that "one slice belongs to one and only one slice group", and whether it is acceptable for RAN2 that it is possible that a network slice can be associated to none, one or more slice groups?

NOTE:	Since current 5G system do not support the above mechanism and there is no WID for such work, SA2 agreed to use a new TEI17 WID for this work.

2. Actions:

To: RAN2, RAN3 and CT1

ACTION:

SA2 asks kindly RAN2, RAN3 and CT1 to take the above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		November 15 – 19, 2021

TSG-SA2 Meeting #149E		February 14 – 25, 2022