SA WG2 Meeting #S2-147E	S2-2107281r03

18 - 22 October, 2021, Electronic meeting	(revision of S2-21yyxxxx)

# Source:	ZTE, LG Electronics, Samsung, Alibaba, Apple, AT&T, CATT, China Telecom, China Unicom, Convida Wireless LLC, Ericsson, Intel, InterDigital, KDDI, Lenovo, Matrixx, MITRE, Motorola Mobility, NEC , Nokia, Nokia Shanghai Bell , NTT Docomo, OPPO, Oracle, Orange, Qualcomm, Sanechips, Sharp, T-Mobile USA, Spreadtrum, Tencent, Verizon UK Ltd, Xiaomi

# Title:	Study on Enhancement of Network Slicing Phase 3

{"Revised" to be used only for WID already approved at plenary. For Revised WIDs, two versions have to be provided in a zip file: a clean one and one with revision marks showing the differences with the previously plenary-approved WID.}

# Document for:	Approval

# Agenda Item:	9.1.3

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title:	Study on Enhancement of Network Slicing Phase 3

{Free text. It has to be the same as in the "Title:" section above. Studies have to start by "Study on"}

Acronym:	FS_eNS_Ph3

{Propose an acronym. Final acronym to be confirmed at the plenary. The sign "-" is a level separator between (Feature)-(Building Block)-(Work Task). The sign "_" can be freely used. Studies have to start by "FS_". Each acronym level has to be simple and short, 7 characters max recommended}

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	{Rel-18}

{Note that this field above indicates the proposed Release at the time of submission of the WID to TSG approval. It can later be changed without a need to revise the WID. The updated target Release is indicated in the Work Plan}

1	Impacts

{For Normative work, identify the anticipated impacts. For a Study, identify the scope of the study}

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  | X | X | X |  |
| No |  |  |  |  |  |
| Don't know | X |  |  |  | X |

2	Classification of the Work Item and linked work items

2.1	Primary classification

This work item is a …

{Tick one box. "Feature / Building Block / Work Task" form a hierarchical structure. E.g. no Building Block can be proposed without a corresponding parent Feature. The full structure of all existing Work Items is shown in the 3GPP Work Plan in ftp://ftp.3gpp.org/Information/WORK_PLAN}

|  | Feature |
| --- | --- |
|  | Building Block |
|  | Work Task |
| X | Study Item |

2.2	Parent Work Item

{"Parent" Work Item refers to the related, earlier-Stage, Work Item, e.g. the related Stage 1 Work Item shall be indicated here when a Stage 2 normative Work Item or Study Item is presented. "Parent" Work Item can also refer to the related preceding Study Item e.g. the related Study Item and the earlier-stage Work Item shall be indicated here when a normative-work Work Items is started. List here all parent Work Items of which requirements are either fully or partially covered by the proposed Item. }

{This section is mandatory to be filled out by the rapporteur. This section is to be filled with care: it indicates to the companies monitoring the parent Work Item that it will be addressed in this study/work item.}

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| FS_EASNS | SA1 | 880035 | Study on Enhanced Access to and Support of Network Slice |
| EASNS | SA1 | 910032 | Enhanced Access to and Support of Network Slice |

2.3	Other related Work Items and dependencies

{List here other Work Items which relate to the proposed one, such as a Work Item in an earlier Release if further enhancing the feature from the previous Release)}

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 860039 | Study on enhancement of RAN slicing for NR | Enhancements for RAN slicing for NR were studied by RAN3 in Rel-17. The solutions with system impacts should be studied in SA2. |
| 911007 | Enhancement of RAN slicing for NR | The solutions in Enhancements for RAN slicing for NR can be taken into account by this study. |

Dependency on non-3GPP (draft) specification: None

{This section is to be typically used to identify the IETF dependencies. Delete the header "Dependency on non-3GPP (draft) specification:" if no such dependency}

3	Justification

RAN3 in Rel-17 has studied the scenarios and candidate solutions in TR 38.832 on the support of slice service continuity during UE mobility due to no support of the network slice or resource limitation of the network slice in the target RAN node. SA2 sent LS out in S2-2102068 to confirm that the scenarios described in the TR are valid(i.e. they do happen, due to the fact that network slices are not required to be available in all TAs of a PLMN). However the solutions with CN/UE impacts were discouraged, and before being pursued would require an SA2 study from end to end system point of view. Deployment improvement should be analysed accordingly..

There is a scenario that an S-NSSAI in the Requested NSSAI is rejected as it is not supported in a TA but it may be supported in nearby TAs. The AMF may need to include the nearby TAs in a Registration Area as it is optimal to do so for the S-NSSAIs in the Allowed NSSAIs. So, if this is done and the rejection cause code is "not supported in the RA", the UE cannot attempt to register with the rejected S-NSSAI until the UE moves out of the RA. If this is not desirable, then the AMF can only assign a RA that is limited to the current TA and TAs where the rejected S-NSSAI is not supported, which enable the UE to register with the rejected S-NSSAI when the UE moves into the nearby TA which supports the rejected S-NSSAI. This study should investigate the solutions to improve the ability to select network slices requested by the UE but  not available in the TA, and which are available in other TAs of the RA, to be requestable by the UE without forcing the AMF to reduce the size of the RA

SA1 has agreed a new requirement in TS 22.261 clause 6.1.2.1: For a roaming UE activating a service/application requiring a network slice not offered by the serving network but available in the area from other network(s), the HPLMN shall be able to provide the UE with prioritization information of the VPLMNs with which the UE may register for the network slice. SA2 should investigate mechanisms to support this new requirement.

When the UE performs registration, the UE generates a Requested NSSAI based on the Configured S-NSSAI, Allowed NSSAI and also may take the URSP rules into account. The UE may register S-NSSAIs which are not going to be used at once, and possibly not while registered in this RA. This may be perceived by operator’s customers as an issue due to NSAC procedure introduced by eNS_Ph2 as the UE may appear to be registered with a network slice but it is not actually using it nor the UE has any application that requests it. Similar considerations apply to PDU session establishment. This study should investigate the mechanism on how to ensure that the UE can form the Requested NSSAI for registration and Requested S-NSSAI for PDU session based on actual running applications, in addition to the Configured NSSAI and Allowed NSSAI, and how the UE can initiate deregistration from the slice or release the PDU session after the last application using the slice stops using the slice or PDU session.

The network at times needs to make a choice among which slices to provide or continue providing for a UE (e.g. the target cell candidates for HO may support different slices). Also, the CN determines the RFSP index for the Allowed NSSAI or RFSP index for target S-NSSAI to redirect the UE to proper target cell. Priority information from the UE regarding some or all of the S-NSSAIs in the Requested NSSAI may be useful for network to determine the set of allowed network slices to select for the UE and possibly to determine the target of a Handover, the target NSSAI and also the target RFSP index. This study should investigate the solutions for providing prioritisation information (if any) regarding S-NSSAIs

Services provided over certain network slices may be limited in Service area (not matching deployed TA boundaries) and/or in time-span of their deployment (e.g. services provided to cover some event). It would be desirable to define mechanisms to support deployments more optimally to minimize operations and disruption to enable such scenarios, with a specific focus on network slice termination.

When the UE in connected state requests new S-NSSAI in registration request and the existing AMF doesn’t support this new requested S-NSSAI, the new requested S-NSSAI will be rejected and the ongoing PDU sessions may be lost as the Ue needs to be placed in idle mode and requested to re-register. It would be desired that a new AMF can be select to serve new requested S-NSSAI. This study should investigate how to support AMF re-allocation for the UE in connected state to avoid the connections of the UE to be lost for ongoing PDU sessions

In Release 17 NSACF is defined to perform Network Slice Admission Control (NSAC). There could be multiple NSACFs within the PLMN. The interaction between these NSACFs was not defined in R17. Without this interaction the operator needs to split the maximum value into each NSACF via OAM. In the roaming case there is no support of dynamic NSAC in the home PLMN for number of UE registered in the network slice. This study should investigate whether and how to support dynamic interaction between NSACFs.

In Rel-17 RAN2 has been discussed a Slice Group concept to broadcast over-the-air, where a slice group consists of one or multiple slices, which can avoid publishing slice identities (S-NSSAI) in System Information (security concern and SI size concern). However how to signal such slice groups to the UE has not been defined. This study should discuss how the network can manage and provide slice group information to the UE.

4	Objective

The objective of this study is to investigate the feasibility of further enhancement on network slicing.

For the following objectives, it is expected to study the necessity and potential mechanisms identified as gaps to our specifications:

1.	Study whether and how to address, e.g. by providing service continuity, the following scenario: the network slicing deployments could be improved to avoid the following scenarios: an existing slice cannot serve the PDU session in current cell (due to OAM reasons) or target cell (due to mobility), or a slice was not allowed due to NSAC, or if the existing slice cannot meet the performance requirements of the applications.

3.	Study whether and how to enable a UE to initiate a registration for a rejected S-NSSAI that was rejected in a first TA of the RA but may be available in another TA of the RA.

4.	Study whether and how to enhance the information available to the UE in roaming scenarios regarding the the availability of network slices in VPLMNs available in the roaming country, in order to allow the UE to select and obtain services from the VPLMN supporting the network slices which UE may wish to use

5.	Study whether and how to enable the network to control the UE behaviour in registering and deregistering with network slices and establishing/releasing PDU sessions taking into account the running applications using the slice.

7.	Study whether and how to support the provisioning of the priority information regarding (some of) the S-NSSAI, e.g. for service continuity decisions or Allowed NSSAI decisions

8.	Study deployment considerations for optimising the temporary deployment of services in an area, and how existing mechanisms including network slicing can help support such scenarios (e.g. the service supported by a network slice may have a limited lifetime) , especially focusing on issues related to network slice termination. If existing mechanisms are concluded to be not sufficient to achieve the scenarios, study whether and how additional mechanisms can resolve the analysed gap.

9	Study deployment considerations when a service provided by existing network slices has a Service Area that does not overlap with the already deployed Tracking Areas, and how existing mechanisms including network slicing can help support such scenarios. If existing mechanisms are concluded to be not sufficient to achieve the scenarios, study whether and how additional mechanisms can resolve the analysed gap.

12. Study whether and how to support AMF re-allocation due to new S-NSSAI requested by the UE in connected state.

13.	Study whether and how to enhance support of networks with more than one NSACF per network slice

14.	Study how the network can manage and provide slice group information to the UE (FFS: depends on discussion on LS from RAN2).

Editor’s Note: Some items in this list are raising concerns and further discussion is needed to determine whether these items can be included in the list and how to update if it is included.

Existing mechanisms shall be reused to the extent possible to resolve the gaps for the scenarios in the objectives above. No new mechanism will be introduced until it is demonstrated that existing mechanisms cannot fulfil the scenarios described in the objectives.

Depending on the deployment guidelines collected during the study, the study will conclude whether and how to document these deployment guidelines in our specifications.

Interaction with RAN working groups is needed on any RAN impact.

TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT#1(Service continuity) | 2.5 | 1.5 | Yes | WT#1 is self-contained |
| WT#3(TAI slice support of rejected slice in RA) | 0.5 | 0.25 | No | WT#3 is self-contained |
| WT#4(VPLMN selection based on slice) | 2 | 1 | No | WT#4 is self-contained |
| WT#5(Requested slice controlled by network) | 1.5 | 0.75 | No | WT#5 is self-contained |
| WT#7(Slice priority) | 1.5 | 0.75 | No | WT#7 is self-contained |
| WT#8(Temporary deployment) | 1.5 | 1 | Maybe | WT#8 is self-contained |
| WT#9(Slice Service Area) | 2 | 1 | Maybe | WT#9 is self-contained |
| WT#12(Connected mode relocation) | 2 | 1 | Maybe | WT#12 is self-contained |
| WT#13(Inter NSACF) | 2 | 1 | No | WT#13 is self-contained Can be dropped if achieved in rel-17 |
| WT#14(Slice group ID for RAN) | 1.5 | 0.5 | No | WT#14 is self-contained Can be dropped if achieved in rel-17 |

Total TU estimates for the study phase:  17

Total TU estimates for the normative phase: 8.75

Total TU estimates:  25.75

5	Expected Output and Time scale

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |
| Internal TR | 23.xxx | Study on Enhancement of Network Slicing Phase 3 | SA#96 | SA#97 |  |

{Note 1:	Only TSs may contain normative provisions. Study Items shall create or impact only TRs.
"Internal TR" is intended for 3GPP internal use only whereas "External TR" may be transposed by OPs.}

{Note 2:	The first listed Rapporteur is the specification primary Rapporteur. Secondary Rapporteur(s) are possible for particular aspect(s) of the TS/TR. In this case, their responsibility has to be provided as "Remarks".}

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| {e.g. "22.281"} | {Possible values: - either free text (e.g. “CS aspects to be removed") - or “Specification to be withdrawn”} | {e.g. "TSG#89"} | {Free text} |

6	Work item Rapporteur(s)

ZHU Jinguo, ZTE, zhu.jinguo@zte.com.cn

Myungjune Youn, LGE, m.youn@lge.com: TBD

{Mandatory: <FamilyName>, <GivenName>, <Company>, <email address>}

{Optional: <FamilyName>, <GivenName>, <Company>, <email address>: Secondary task(s)}

{The first listed Rapporteur is the work item primary Rapporteur. The role of a Rapporteur is further described in www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec. By default, the primary Rapporteur shall ensure the production of the post-completion summary. 
Secondary Rapporteur(s) are possible for specific secondary task(s), such as: "Write the post-completion summary"; "In charge of a specific aspect of the work item (specify which)"; "Rapporteur for a secondary responsible WG (specify which)"}

7	Work item leadership

SA2

8	Aspects that involve other WGs

The following aspects may arise related to this WID:

-	Security aspects

-	Charging, OAM aspects

-	RAN aspects

9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Alibaba

Apple

AT&T

CATT

China Telecom

China Unicom

Convida Wireless LLC

Ericsson

Intel

InterDigital

KDDI

KT Corp.

Lenovo

LG Electronics

LG Uplus

Matrixx

MITRE

Motorola Mobility

NEC

Nokia

Nokia Shanghai Bell

NTT Docomo

OPPO

Oracle

Orange

Qualcomm

Samsung

Sanechips

Sharp

SK Telecom

T-Mobile USA

Spreadtrum

Tencent

Verizon UK Ltd

Xiaomi

ZTE