SA WG2 Meeting #S2-147E	S2-2107287

# Oct 18 – 22, 2021, Electronic

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3272 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support of slice priority and slice group information for cell (re)selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2, TEI17 |  |  |  |  |  | Date: |  |  | 2021-10-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | RAN2 provides an LS in this meeting that RAN2 has decided to support cell reselection with introducing slice priority and slice group concept. And RAN2 asks whether SA2 can support the 5GC providing information about slice priority and slice group to UE through NAS signaling, to help the cell reselection when RAN broadcast the slice group information. This CR propose to enhance 5G system capability to support 5GC providing slice priority and slice group information to UE through NAS signalling,and also coordinating the slice information among RAN,5GC and UE, in order to help RAN broadcasting and cell reselection. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description about - 5GC providing slice priority and slice group information to UE - Slice information coordination between RAN,5GC and UE for cell reselection |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Slice (S-NSSAI) based cell reselection can not be supported in Rel-17 |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## <<< Start of changes,all the text are new >>>

### 5.15.x	Support of slice priority and slice group information for cell (re)selection

In order to support cell (re)selection by UE with considering slice information in RRC_IDLE/RRC_INACTIVE, NG-RAN can broadcast slice information and related frequency(ies) priority to steer the UE to camp on a cell/frequency where the UE can obtain access to the possible high priority slice.

NOTE: Which slice(s) are considered high priority can be decided locally at the UE.

When the UE in RRC_IDLE/RRC_INACTIVE state, the UE may decide which slice(s) it would like to access to with high priority, considering the locally configured slice priority, and camp on the related frequency based on NG-RAN broadcast information.

The slice priority feature does not impact the processing of S-NSSAI(s) subject to NSSAA.

For the slice information NG-RAN broadcasting, it shoud be a slice group information which can avoid publishing S-NSSAI(s) in radio system information considering security concern and SIB size limitation. One slice belongs to only one slice group and each slice group is uniquely identified by a slice group identifier. AMF determines the relationship between S-NSSAI(s) and slice group ID(s) based on local configuration or operator policy.

NG-RAN provides supported S-NSSAI(s) per TA maybe with indication of supporting broadcasting slice group ID in NG Setup Request Message to AMF. AMF may determine the S-NSSAI(s) and the corresponding slice group ID(s) based on AMF slice capability and local configuration, and then send the S-NSSAI(s) and the corresponding slice group ID(s) to NG-RAN in NG Setup Response Message to support NG-RAN broadcasting slice group ID(s).

During UE registration procedure, the UE may indicate the support of slice group ID(s) to 5GC. And AMF can send the allowed S-NSSAI(s) and the corresponding slice group ID(s) to UE, based on NG-RAN node level S-NSSAI(s) and the corresponding slice group ID(s) and the UE’s Allowed NSSAI.

The AMF may update the UE Allowed NSSAI and corresponding slice group ID(s) when the NSSAA procedure complete, local configuration change or subscription update.

For the UE Initial Registration without slice group ID(s) information, the UE should select the cell without considering the NG-RAN broadcasting information.

## <<<End of changes >>>