3GPP TSG-WG SA2 Meeting #147E e-meeting	S2-2107321r04

# Elbonia, October 18 – 22, 2021													(revision of S2-2107321)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0448 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to resolve conflicts in ML model transferring |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In this release of the specification, we make consensus on the principle that AnLF only retrieves the ML model information from the set of configured MTLFs in the AnLF, however, Nnwdaf_AnalyticsSubscription_Transfer and Nnwdaf_AnalyticsInfo_ContextTransfer service are in conflict with that principle. Specifically, Nnwdaf_AnalyticsSubscription_Transfer allows source NWDAF to transfer ML model information (i.e., file address of ML model) to target NWDAF without any confirmation of the set of configured MTLFs in target NWDAF, and then, target NWDAF directly accesses the ML model while exploiting the transferred ML model information, even if the ML model comes from unconfigured MTLF in the target NWDAF. This breaks the principle. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to delete ML model information in input parameter of Nnwdaf_AnalyticsSubscription as well as clarifies the conditions that target NWDAF subscribes ML model(s) for the provided MTLF IDs from source NWDAF, and removes target NWDAF directly retrives ML model with ML model information over the procedures in clause 6.1B2.2 and 6.1B.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | AnLF can use ML model(s) from the unconfigured MTLFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1B.2.2, 6.1B.3, 7.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of the first change * * * *

6.1B.2.2	Analytics Subscription Transfer initiated by source NWDAF

The procedure in Figure 6.1B.2.2-1 is used by an NWDAF instance to request the transfer of analytics subscription(s) to another NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

If the source NWDAF discovers that the analytics consumer may change concurrently to this procedure, the source NWDAF should not perform the procedure. In such a case, the source NWDAF may send a message to indicate to the analytics consumer that it will not serve this subscription anymore.

NOTE 1:	To discover if the analytics consumer may change, the source NWDAF can, for example, check if the serving area of target NWDAF is within the serving area of the analytics consumer, if the serving area information is available.

NOTE 2:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available, therefore, not requiring an analytics subscription transfer as described in this clause. The procedure in Figure 6.1B.2.2-1 is applicable for analytics subscription transfer across NF Sets or if the NWDAF is not deployed in a Set.

Figure 6.1B.2.2-1: Analytics subscription transfer initiated by source NWDAF

0.	Source analytics consumer subscribes to Analytics from source NWDAF. The Analytics Consumer may send its NF ID or serving area, enabling NWDAF to determine whether this procedure is applicable. Optionally the source NWDAF subscribes to UE mobility events.

1.	[Optional] Source NWDAF determines, e.g. triggered by a UE mobility event notification, to prepare an analytics subscription transfer to target NWDAF(s), as specified in the procedure illustrated in clause6.1B.2.3.

2.	Source NWDAF determines, e.g. based on the UE location information received and the NF consumer's serving area either directly received in step 0 or indirectly received via NRF, to perform an analytics subscription transfer to target NWDAF(s). Therefore, the source NWDAF determines the analytics subscription(s) to be transferred to a target NWDAF.

3.	Source NWDAF performs an NWDAF discovery and selects the target NWDAF. NWDAF discovery may be skipped if the target NWDAF had already been discovered as part of a prepared analytics subscription transfer In the case of aggregated analytics from multiple NWDAFs, the source NWDAF may use the set of NWDAF identifiers related to aggregated analytics (see clause 6.1.3) to preferably select a target NWDAF that is already serving the consumer.

4.	Source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request service operation, a transfer of the analytics subscription(s) determined in step 2 to the target NWDAF. The request contains a callback URI of the analytics consumer. The request also contains active data source ID(s), ML model provider NWDAF ID(s), which are related to the analytics subscriptions requested to be transferred, if not already provided as part of the prepared analytics subscription transfer in the preparation procedure (see step 1). The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytic ID(s).

If the analytics subscription being transferred requires analytics aggregation in the Source NWDAF (i.e. the Source NWDAF as an Aggregator NWDAF produces analytics information for the analytics subscription being transferred by aggregating analytics outputs from other NWDAFs and possibly itself as specified in clause 6.1A), the request message may also include the analytics subscription aggregation information, i.e. information about the analytics subscriptions that the source NWDAF has with these NWDAFs that collectively serve the transferred analytics subscription for specific analytics ID(s), which includes IDs of these NWDAFs for specific analytics ID(s) and optionally input parameters of analytics exposure as defined in clause 6.1.3.

5.	Target NWDAF accepts the analytics subscription transfer and takes over the analytics generation based on the information received from the source NWDAF.

If ML model provider NWDAF ID(s) is provided in the Nnwdaf_AnalyticsSubscription_Transfer request and the provided ML model provider NWDAF ID(s) is locally configured in target NWDAF, target NWDAF may request or subscribe to the ML model(s) from the indicated ML model provider NWDAF(s) as specified in clause 6.2A, and use the ML model(s) for the transferred analytics subscription. Otherwise, target NWDAF may use the ML model trained by itself if the target NWDAF contains MTLF or may discover ML model provider(s) within the set of configured NWDAF ID(s) containing MTLF for the analytics subscription, and may request or subscribe to ML model(s) as specified in clause 6.2A, if needed.

If the analytics subscription being transferred requires analytics aggregation and the Nnwdaf_AnalyticsSubscription_Transfer request contains the analytics subscription aggregation information as described in step 3a, the target NWDAF, if supporting analytics aggregation, may use the analytics subscription aggregation information as indicated in the request to acquire analytics outputs and perform analytics aggregation for the transferred analytics subscription.

NOTE 3:	If not yet done during a prepared analytics subscription transfer, the target NWDAF allocates a new subscription Id to the received analytics subscriptions, following the behaviour of AmfEventSubscription as specified in TS 29.518 [25] clause 6.2.6.2.2 and UEContextTransfer as specified in TS 29.518 [25] clause 5.2.2.2.1.1.

NOTE 4:	The target NWDAF might already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 1 and, thus, might already have started to prepare for the analytics generation, e.g. by having already subscribed to relevant event notifications.

6.	Target NWDAF informs the analytics consumer about the successful analytics subscription transfer using a Nnwdaf_AnalyticsSubscription_Notify message and providing the new subscription Id, that had been assigned by the target NWDAF, in the Subscription Change Notification Correlation Id parameter of this message as specified in clause 7.2.4.

NOTE 5:	The existing Analytics context in the source NWDAF is not deleted directly but will be purged first when it was collected by the target NWDAF.

NOTE 6:	If this subscription is used as input for analytics aggregation by the analytics consumer, the analytics consumer may inform the other NWDAFs instance participating in this analytics aggregation that the Set of NWDAF identifiers of NWDAF instances used by the NWDAF service consumer for this analytics aggregating (see clause 6.1.3) has changed using the Nnwdaf_AnalyticsSubscription_Subscribe service operation.

7.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, the target NWDAF requests the "analytics context". The analytics context information transfer procedure is specified in clause 6.1B.3.

8.	[Optional] Target NWDAF subscribes to relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the Analytics.

9.	Target NWDAF confirms the analytics subscription transfer to the source NWDAF.

10.	[Optional] Source NWDAF unsubscribes with the data source(s) that are no longer needed for the remaining analytics subscriptions. In addition, Source NWDAF unsubscribes with the ML model provider NWDAF(s), if exist, which are no longer needed for the remaining analytics subscriptions.

NOTE 7:	At this point, the analytics subscription transfer is deemed completed, i.e. the source NWDAF can delete all information related to the successfully transferred analytics subscription.

11-12.	Target NWDAF at some point derives new output analytics based on new input data and notifies the analytics consumer about the new analytics using a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 6.1.1.

NOTE 8:	Notification correlation information allows the analytics consumer to correlate the notifications to the initial subscription request made with the source NWDAF in step 0.

# * * * * Start of the second change * * * *

6.1B.3	Analytics Context Information Transfer

The procedure depicted in Figure 6.1B.3-1 is used by an NWDAF instance to request analytics context information from another NWDAF instance, using the Nnwdaf_AnalyticsInfo_ContextTransfer service operation as defined in clause 7.3.3. This procedure, for example, can be invoked in the procedures described in clause 6.1B.2 to request the transfer of relevant analytics context information.

Figure 6.1B.3-1: Analytics Context Information Transfer

The procedure of analytics context information transfer comprises the following steps:

1.	The consumer NWDAF requests analytics context information by invoking Nnwdaf_AnalyticsInfo_ContextTransfer request service operation. The parameters that can be provided in the request are listed in clause 6.1B.4.

2.	The provider NWDAF responds with analytics context information to the consumer NWDAF. The analytics context information that can be provided in the response is listed in clause 6.1B.4.

If the provider NWDAF stores analytics context information (e.g. historical analytics, historical data) in ADRF, the provider NWDAF may include in the response the ADRF ID together with an indication of the type of data stored in the ADRF.

Upon receiving the analytics context information, the consumer NWDAF may:

-	provide the pending output analytics or historical analytics information to the analytics consumer per the subscription/request;

-	use the historical data and analytics metadata in the analytics context information to generate analytics;

-	subscribe to data collected for analytics with the data sources indicated in the analytics context information;

-	subscribe to trained ML model(s) from the indicated model provider NWDAF(s), if the indicated NWDAF(s) containing MTLF is locally configured in the consumer NWDAF, and use the ML model(s) for analytics;

-	use the ML model trained by the consumer NWDAF itself when the ML model provider NWDAF ID(s) is not provided in the Nnwdaf_AnalyticsSubscription_Transfer request or the provided ML model provider NWDAF ID(s) in the Nnwdaf_AnalyticsSubscription_Transfer request is not locally configured in consumer NWDAF; and/or

-	subscribe to output analytics from the indicated NWDAFs that collectively serve the transferred analytics subscription and perform analytics aggregation on the output analytics using the analytics metadata information, based on the analytics subscription aggregation information.

# * * * * Start of the third change * * * *

7.2.5	Nnwdaf_AnalyticsSubscription_Transfer service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Transfer.

Description: Requests to NWDAF to transfer analytics subscription(s) from the consumer NWDAF.

Inputs, Required:

-	Transfer type: indicates the type of the transfer request. The following values are supported:

-	Analytics subscription transfer preparation: requests the NWDAF to prepare for taking over the analytics subscription(s) and/or prepare for collecting Analytics Context(s).

-	Analytics subscription transfer: requests the NWDAF to take over the analytics subscription(s).

-	Analytics subscription transfer cancel: cancels a prepared analytics subscription request.

Inputs, Optional:

-	If this service operation is for "analytics subscription transfer preparation", the following parameter shall be provided:

-	(Set of) analytics subscription information with the following parameters:

-	All input parameters for the analytics exposure as specified in clause 6.1.3.

-	[OPTIONAL] Active data source ID(s): Instance Id or Set Id of the active data source(s) the consumer NWDAF is currently using for the analytics of this analytics subscription.

-	[OPTIONAL] Model provider NWDAF ID(s): Instance ID(s) of the ML model provider NWDAF(s) from which the consumer NWDAF currently subscribes to the ML model information used for the analytics.

NOTE: Target NWDAF containing AnLF is only allowed to use the locally configured (set of) NWDAF(s) containing MTLF (see clause 6.2A.0).

-	Subscription callback URI: indicates the callback URI of the analytics consumer of this subscription, i.e. the URI the analytics notifications are to be sent to.

-	Id of analytics consumer that is subscribed to receive analytics.

-	(Set of) analytics context identifier(s): identifies analytics context information available at the consumer NWDAF as defined in clause 6.1B.4.

-	If this service operation is to request analytics subscriptions transfer and if analytics subscription transfer preparation is not performed, the same parameters as those for transfer type "Analytics subscription transfer preparation" shall be provided.

-	If this service operation is to request analytics subscriptions transfer and if analytics subscription transfer preparation is performed, the same parameters as those for transfer type "Analytics subscription transfer preparation" can be provided with updated parameter values.

-	If this service operation is to request analytics subscriptions transfer cancel, the following parameter shall be provided:

-	Subscription Correlation Id.

Outputs Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *