SA WG2 Meeting #S2-147E  	S2-2107338r02

# 18 – 22 Oct, 2021, Elbonia	(revision of S2-xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3164 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on TSCTSF discovery and selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - In the Table 5.2.12.1-1: NF services provided by UDR, TSCTSF is also the consumer of Create/Update/Delete service operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the TSCTSF as consumer of Create/Update/Delete service operation |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | , 5.2.12.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 3rd change * * * *

5.2.12.1	General

The following Data Set Identifiers shall be considered in this release: Subscription Data, Policy Data, Application data and Data for Exposure. The corresponding Data Subset Identifiers and Data (Sub)Key(s) are defined in Table 5.2.12.2.1-1.

The set of Data Set Identifiers shall be extensible to cater for new identifiers as well as for operator specific identifiers and related data to be consumed.

The following table illustrates the UDR Services.

Table 5.2.12.1-1: NF services provided by UDR

| NF service | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Data Management (DM) | Query | Request/Response | UDM, PCF, NEF |
|  | Create | Request/Response | NEF, TSCTSF |
|  | Delete | Request/Response | NEF, TSCTSF |
|  | Update | Request/Response | UDM, PCF, NEF, TSCTSF |
|  | Subscribe | Subscribe/Notify | UDM, PCF, NEF |
|  | Unsubscribe |  | UDM, PCF, NEF |
|  | Notify |  | UDM, PCF, NEF |
| GroupIDmap | Query | Request/Response | NRF, SCP |

The following table shows the Exposure data that may be stored in the UDR along with a time stamp using Data Management (DM) Service:

NOTE:	When the data in Table 5.2.12.1-2 need to be monitored in real time, they should be monitored directly at the originating NF (e.g. registration state changes may be monitored via the Namf_EventExposure service) and not use the stored information from UDR if it is not the latest. It is expected that such dynamically changing information (e.g. UE reachability status) is used for statistical purpose and analytics.

Table 5.2.12.1-2: Exposure data stored in the UDR

| Category | Information | Description | Data key | Data Sub key |
| --- | --- | --- | --- | --- |
| Access and mobility information | UE location | Gives the Location or the last known location of a UE (e.g. Tai, Cell Id… both 3GPP and non-3GPP access location) | SUPI or GPSI |  |
|  | UE time zone | Current time zone for the UE | SUPI or GPSI |  |
|  | UE Access type | 3GPP access or non-3GPP access | SUPI or GPSI |  |
|  | UE RAT type | Determined as defined in clause 5.3.2.3 of TS 23.501 [2]. The values are defined in TS 29.571 [70] | SUPI or GPSI |  |
|  | UE registration state | Registered or Deregistered | SUPI or GPSI |  |
|  | UE connectivity state | IDLE or CONNECTED | SUPI or GPSI |  |
|  | UE reachability status | It indicates if the UE is reachable for sending either SMS or downlink data to the UE, which is detected when the UE transitions to CM-CONNECTED state or when the UE will become reachable for paging, e.g., Periodic Registration Update timer | SUPI or GPSI |  |
|  | UE SMS over NAS service status | SMS over NAS supported or not in the UE | SUPI or GPSI |  |
|  | UE Roaming status | It indicates UE's current roaming status (the serving PLMN and/or whether the UE is in its HPLMN) | SUPI or GPSI |  |
|  | UE Current PLMN | Current PLMN for the UE | SUPI or GPSI |  |
| Session management | UE IP address | UE IP address | SUPI or GPSI | PDU session ID or DNN |
| information | PDU session status | Active / released | SUPI or GPSI | PDU session ID or DNN or UE IP address |
|  | DNAI | DNAI | SUPI or GPSI | PDU session ID or DNN or UE IP address |
|  | N6 traffic routing information | N6 traffic routing information | SUPI or GPSI | PDU session ID or DNN or UE IP address |

# * * * * End of 3rd change * * * *