3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107351r09

# Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0451 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the Slice load level analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As indicated in the LS (S2-2107030/C3-214444) from CT3, the load level of the network slice instance as defined in Realease 15 and Release 16 is removed in Release 17 which is lead the backward compatibility issues. So the load level analytics should be resumed. On the other hand, in TS 23.288, for some Analytics ID such as Slice load level, NF load and so on, a new parameter i.e. “list of analytics subset” is used to indicate which analytics subset in the output analytics table should be provided by the NWDAF. However, the definition for the “list of analytics subset” parameter is missing in the Contents of Analytics Expoure in clause 6.1.3. At the same time, the CT3 LS (i.e. S2-2107030/C3-214444) is also asking how the NWDAF know which analytics subsets in the Slice Load Level analytics should be provided in the analytics result. Furthermore, currently the threshold value and matching direction are only applied to the Nnwdaf_AnalyticsSubscription_Subscribe service, however, these parameters can be also provided in the Nnwdaf_AnalyticsInfo_Request service to help NWDAF determine to response the consumer NF when the threshold value and matching direction are met. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Resume the load level of network slice instance; 2. Add the definition of “list of analytics subset” parameter into clause 6.1.3. 3. Clarify that reporting thresholds and matching direction can be provided with Nnwdaf_AnalyticsInfo_Request service operation when Analytics ID is set to "Load level information". |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. Backward compatibility issues exist. 2. The definition for the “list of analytics subset” parameter is unclear. 3. It is not sure how the NWDAF determine to response the consumer NF when the consumer NF also has requirement for the reporting threshold and matching direction. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1, 6.3.3A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

6.3.1	General

The NWDAF provides slice load level information to a consumer NF on a Network Slice level or a Network Slice instance level or both. The NWDAF is not required to be aware of the current subscribers using the slice. The NWDAF notifies slice specific network status analytics information to the consumer NF that is subscribed to it. A consumer NF may collect directly slice specific network status analytics information from NWDAF. This information is not subscriber specific.

The NWDAF services as defined in the clause 7.2 and clause 7.3 are used to expose slice load level analytics from the NWDAF to the consumer NF (e.g. PCF, NSSF or AMF).

The consumer of these analytics shall indicate in the request or subscription:

-	Analytics Id set to "Load level information";

-	The following Analytics Filter Information:

-	S-NSSAI and NSI ID;

--	optionally, the list of analytics subsets that are requested among those specified in clause 6.3.3A.

NOTE:	The use of NSI ID in the network is optional and depends on the deployment choices of the operator. If used, the NSI ID is associated with S-NSSAI. NSI ID is only applicable when the consumer of analytics is NSSF or AMF.

-	an optional Area of Interest;

-	an optional list of NF types;

-	optionally, the list of analytics subsets that are requested among those specified in clause 6.3.4.

-	optionally, Load Level Threshold value; and

-	optionally, "maximum number of objects" indicating the maximum number of Network Slice instances expected in output, when the Analytics Filter Information does not indicate an NSI ID.

# * * * * Next change * * * *

6.3.3A	Output analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the following analytics:

-	Network Slice instance load statistics information as defined in Table 6.3.3A-1.

-	Network Slice load statistics information as defined in Table 6.3.3A-2.

-	Network Slice instance load predictions information as defined in Table 6.3.3A-3.

-	Network Slice load predictions information as defined in Table 6.3.3A-4.

Table 6.3.3A-1: Network Slice instance load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| Network Slice instances (1…max) | List of Network Slice instance(s) within the S-NSSAI. |
| > NSI ID | Identification of the Network Slice instance. |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations of the Network Slice instance (average, variance). |
| > Number of PDU Sessions establishment (NOTE 1) | Number of PDU Session establishments of the Network Slice instance (average, variance). |
| > Resource usage (NOTE 1) | The usage of assigned virtual resources currently in use for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (NOTE 1) | Number of resource usage threshold crossings on the Network Slice instance provided if threshold is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (1…max) (NOTE 1) | Resource usage threshold crossing vector including time elapsed between each threshold crossing on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the statistics value or the expected value of the Load Level Threshold if threshold is provided by the consumer as Analytics Filter. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-2: Network Slice load statistics

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| > Number of UE Registrations (NOTE 1) | Number of UE registrations at the Network Slice (average, variance). |
| > Number of PDU sessions establishments (NOTE 1) | Number of PDU Session establishments at the Network Slice (average, variance). |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the statistics value or the expected value of the Load Level Threshold if threshold is provided by the consumer as Analytics Filter. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-3: Network Slice instance load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| Network Slice instances (1…max) | List of Network Slice instance(s) within the S-NSSAI. |
| > NSI ID | Identification of the Network Slice instance. |
| > Number of UE Registrations (NOTE 1) | Number of predicted UE registrations at the Network Slice instance. |
| > Number of PDU Sessions establishment (NOTE 1) | Number of predicted PDU Session establishments of the Network Slice instance. |
| > Resource usage (NOTE 1) | The predicted usage of assigned virtual resources for the NF instances (mean usage of virtual CPU, memory, disk) as defined in TS 28.552 [8] clause 5.7, belonging to a particular Network Slice instance. |
| > Resource usage threshold crossings (NOTE 1) | Number of predicted resource usage threshold crossings at the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Resource usage threshold crossings time period (1…max) (NOTE 1) | Predicted Resource usage threshold crossing vector including predicted time elapsed between each threshold crossing on the Network Slice instance provided that a threshold value is provided by the consumer as Analytics Filter. |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the statistics value or the expected value of the Load Level Threshold if threshold is provided by the consumer as Analytics Filter. |
| > Probability assertion | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.3.3A-4: Network Slice load predictions

| Information | Description |
| --- | --- |
| S-NSSAI | Identification of the Network Slice. |
| > Number of UE Registrations (NOTE 1) | Predicted Number of UE registrations at the Network Slice. |
| > Number of PDU sessions establishments (NOTE 1) | Predicted Number of PDU Session establishments at the Network Slice. |
| > Load Level (NOTE 1) | The load level of the Network Slice Instance indicated by the S-NSSAI and the associated NSI ID (if applicable) in the Analytics Filter, if threshold is not provided by the consumer as Analytics Filter. |
| > Crossed Load Level Threshold (NOTE 1) | The Load Level Threshold that are met or exceeded by the statistics value or the expected value of the Load Level Threshold if threshold is provided by the consumer as Analytics Filter. |
| > Probability assertion | Confidence of this prediction. |
| NOTE 1: Analytics subset that can be used in "list of analytics subsets that are requested". |  |

NOTE:	If no NSI ID is provided as analytics filter, slice load level related output analytics are provided according to Tables 6.3.3A-2 and 6.3.3A-4. Otherwise slice instance load level related output analytics are provided according to Tables 6.3.3A-1 and 6.3.3A-3.

# * * * * End of changes * * * *