3GPP TSG SA WG2 Meeting #146E 		S2-2107394r03

16 – 27 August, 2021, Elbonia

Title:	[draft] Reply LS on discovery and relay (re)selection

Response to:	LS on discovery and relay (re)selection (S2-2106991/R2-2104649)

Release:	Rel-17

Work Item:	5G_ProSe

Source:	SA2

To:	RAN2

Cc:	-

Contact person:	Qiang Deng (CATT)

dengqiang1@catt.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	S2-21xxxxx

1	Overall description

SA2 thanks RAN2 for the LS on discovery and relay (re)selection (S2-2106991/R2-2104649).

SA2 has discussed and agreed to include NCGI in the discovery message for 5G ProSe Layer-2 UE-to-Network Relay per the approved CR S2-21xxxxx as attached.

SA2 also would like to ask RAN2 to provide feedback on the following issues:

1) SA2 assumed 5G MOCN architecture is supported for 5G ProSe Layer-2 UE-to-Network Relay as described in clause 4.2.7.2 of TS 23.304, and would like to ask RAN2 to confirm this assumption. SA2 has also noted that those PLMN IDs are required for the 5G ProSe Layer-2 Remote UE if this assumption is confirmed before the layer-2 link has been established.

2) SA2 recognized TAI is needed for 5G ProSe Layer-2 Remote UE to determine the type of initial access message (Mobility Registration Update or Service Request), and would like to ask whether TAI is forwarded by 5G ProSe Layer-2 UE-to-Network to the 5G ProSe Layer-2 Remote UE in the PC5-AS message.

3) SA2 has been discussing this Editor’s Note in TS23.304 clause 6.4.3.5: “Editor’s Note Whether the Layer-2 link modification procedure is also applicable to ProSe Communication via 5G ProSe Layer-2 UE-to-Network Relay requires cooperation with RAN2.” SA2 understands that during the Layer-2 link establishment the Layer-2 UE-to-Network Relay and Remote UE do not interact with the QoS Info (the information about PC5 QoS Flows), meaning there is no PC5 QoS Flow in the PC5 unicast link between Layer-2 UE-to-Network Relay and Remote UE and the QoS handing is therefore setup by RAN. SA2 would like to ask as the Layer-2 link modification procedure is used to add/modify/remove QoS flow(s) in the PC5 unicast link, whether this procedure is applicable or not to the Layer-2 UE-to-Network Relay.

2	Actions

To RAN2

ACTION: 	SA2 kindly asks RAN2 to take above into account in the WI phase, and provides feedback on the raised issues.

3	Dates of next TSG SA WG 2 meetings

TSG-SA2 Meeting #148E		15 – 19 November 2021	Elbonia

TSG-SA2 Meeting #149E		TBD		Elbonia