SA WG2 Meeting #147E	S2-2107405r02

Oct 18 - 22, 2021, Electronic

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0015 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications on functional entities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - During shared delivery establishment, MB-SMF interacts with NG-RAN without interacting with the SMF. Accordingly, the functionality of MB-SMF and SMF need to be updated. - The SMF subscribes to multicast session information from the MB-SMF in order to modify the associated PDU session. The functionality of the SMF and MB-SMF also need to be updated to reflect this. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the functionality of MB-SMF and SMF for supporting shared delivery establishment and for the associated PDU Session management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect functional descriptions for the MB-SMF and SMF. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2.2, 5.3.2.3 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

#### 5.3.2.2	MB-SMF

The MB-SMF performs the following functions to support MBS:

-	General for multicast and broadcast sessions:

-	Supporting MBS session management (including QoS control).

-	Configuring the MB-UPF for multicast and broadcast data transport, based on the policy rules for multicast and broadcast services from PCF or local policy.

-	Allocating and de-allocating TMGIs.

-	Specific for broadcast sessions:

-	Interacting with RAN (via AMF) to control data transport using 5GC Shared MBS traffic delivery method.

-	Specific for multicast sessions:

-	Interacting with SMF to provide the SMF with MBS session context information.

-	Interacting with RAN (via AMF) to establish data transmission resources between MB-UPF and RAN nodes for 5GC Shared MBS traffic delivery method.

-	Controlling the MB-UPF for multicast data transport using 5GC Individual MBS traffic delivery method.

#### 5.3.2.3	SMF

In addition to the functions defined in TS 23.501 [5], the SMF performs the following functions to support MBS:

-	Discovering MB-SMF for a multicast session.

-	Authorizing multicast session join operation if needed.

.

-	Interacting with MB-SMF to obtain multicast session context information used as input to modify the PDU Session associated with MBS session.

-	Interacting with RAN and UPF for  multicast data transport using 5GC Individual MBS traffic delivery method.

NOTE:	SMF and MB-SMF may be co-located or deployed separately.

******************** End of Change ********************