3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107417r06

# Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 3176 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for UE onboarding and UE access with CH credentials |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Based on the agreement in 146E, the onboarding capability of AMF and SMF should be considered in AMF and SMF discovery and selection by using NRF, so AMF and SMF should register their onboarding capability to NRF, and the onboarding capability should be used included in the related NF discovery service. Meanwhile, clarify that: - The HNI (i.e. the realm of Network Specific Identifier based SUPI) is used for discovery and selection of AUSF/UDM/NSSAAF in the case of access to an SNPN using credentials owned by a Credentials Holder with AAA Server, this should be captured in NF profile of AUSF and UDM. - The HNI (i.e. the realm of Network Specific Identifier based SUPI) is used for discovery and selection of AUSF in the case of UE onboarding a DCS with AAA server, this should be captured in NF profile of AUSF. - The HNI (i.e. the realm if Network Specific Identifier based SUPI are used or the MCC and MNC if IMSI based SUPI are used) is used for discovery and selection of AUSF and UDM in the case of UE obboarding a DCS with UDM and AUSF, this should be captured in NF profile of AUSF and UDM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add SNPN onboarding capability of AMF and SMF in NRF register and discovery request service. - Clarify the HNI for discovery and selection of AUSF, UDM, NSSAAF in NRF register, subscribe and discovery request service in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of UE onboarding using a DCS with AAA server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Miss some key functionalities in NRF services for oboarding and UE access with credentials from CH. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.7.2.2, 5.2.7.2.5, 5.2.7.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.2.7.2.2	Nnrf_NFManagement_NFRegister service operation

Service Operation name: Nnrf_NFManagement_NFRegister.

Description: Registers the consumer NF in the NRF by providing the NF profile of the consumer NF to NRF, and NRF marks the consumer NF available.

Inputs, Required: NF type, NF instance ID, FQDN or IP address of NF, Names of supported NF services (if applicable), and PLMN ID e.g. if NF needs to be discovered by other PLMNs/SNPNs.

NOTE 1:	for the UPF, the addressing information within the NF profile corresponds to the N4 interface.

NOTE 2:	For the purpose of the Nnrf_NFManagement service, the SCP is treated by the NRF in the same way as NFs. Specifically, the SCP is designated with a specific NF type and NF instance ID. However, the SCP does not support services and related NF profile parameters do not apply (e.g. NF Set ID, NF service set ID, Endpoint Address(es) of instance(s) of supported service(s)), see clause 6.2.6.3 of TS 23.501 [2].

Inputs, Optional:

-	If the consumer NF stores Data Set(s) (e.g. UDR): Range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers, Data Set Identifier(s).

-	If the consumer is BSF: Range(s) of SUPIs, range(s) of GPSIs, Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, IP domain list as described in clause 6.1.6.2.21 of TS 29.510 [37], Range(s) of SUPIs, range(s) of GPSIs.

NOTE 3:	Range of SUPI(s) is limited in this release to a SUPI type of IMSI as defined in TS 23.003 [33].

-	If the consumer is UDM, UDR, PCF, BSF or AUSF, they can include UDM Group ID, UDR Group ID, PCF Group ID, BSF Group ID, AUSF Group ID respectively.

-	For UDM and AUSF, Routing Indicator, or Routing Indicator and Home Network Public Key identifier; Home Network Identifier: PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN. Optionally, some NFs may additionally include a Home Network Identifier (including the identification of the CH with AAA server or DCS with AAA server) in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of SNPN Onboarding using a DCS with AAA server.

-	For NSSAAF, Home Network Identifier in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in case of SNPN Onboarding using credentials from a DCS with AAA server.

-	If the consumer is AMF, it includes list of GUAMI(s). In addition, AMF may include list of GUAMI(s) for which it can serve as backup for failure/maintenance.

-	If the consumer is CHF, it may include Range(s) of SUPIs, Range(s) of GPSIs, or Range(s) of PLMNs as defined in TS 32.290 [42].

-	If the consumer is CHF, primary CHF instance and the secondary CHF instance pair. If the CHF does not provide NF set ID or NF Service Set ID, it shall provide a primary CHF instance and the secondary CHF instance pair, and otherwise it may do so.

-	If the consumer is P-CSCF, the P-CSCF IP address(es) to be provided to the UE by SMF.

-	If the consumer is HSS, IMPI range, IMPU range, HSS Group ID (as defined in TS 23.228 [55]) can be used as optional input parameters.

-	For the UPF Management: UPF Provisioning Information as defined in clause 4.17.6.

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	DNN(s) if the consumer is PCF or BSF. DNN(s) per S-NSSAI if the consumer is SMF, UPF or TSCTSF.

-	If the consumer is a trusted AF it may include one or multiple combination(s) of S-NSSAI and DNN corresponding to the AF. In addition, it may include supported Application Id(s), Event ID(s) supported by the AF and Internal-Group Identifier.

-	Information about the location of the NF consumer (operator specific information, e.g. geographical location, data centre).

-	TAI(s).

-	NF Set ID.

-	NF Service Set ID.

-	If the consumer is PCF or SMF, it includes the MA PDU Session capability to indicate if the NF instance supports MA PDU session or not.

-	If the consumer is PCF, it includes the DNN replacement capability to indicate if the NF instance supports DNN replacement or not.

-	If the consumer is NWDAF, it includes Analytics ID(s) (possibly per service), NWDAF Serving Area information and Supported Analytics Delay per Analytics ID(s) (if available). In addition, it includes Analytics aggregation capability and/ or Analytics metadata provisioning capability if such capability is provided by the NWDAF. If the consumer NWDAF contains MTLF, it may also include the Analytics Filter information for the trained ML model(s) per Analytics ID(s), if available. It may also include NF Set ID and NF Type of the NF data sources, if data management service is available. Details about NWDAF specific information are described in clause 6.3.13, TS 23.501 [2].

-	If the consumer is NEF, it may include Event ID(s) supported by AFs, the S-NSSAI and DNN corresponding to the untrusted AF served by the NEF, Application Identifier(s) supported by AFs, range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF. If the consumer is local NEF, it may include parameters of list of supported TAI or list of supported DNAI additionally.

-	If the consumer is NSACF, it includes S-NSSAI(s) of the network served by NSACF for which it manages network slice admission control, NSACF Serving Area information and NSACF service capabilities. Details about NSACF Service Area information and NSACF service capabilities are described in clause 6.3.22 of TS 23.501 [2].

-	Notification endpoint for default subscription for each type of notification that the NF is interested in receiving.

-	Endpoint Address(es) of instance(s) of supported service(s).

-	NF capacity information.

-	NF priority information.

-	If consumer is NF, SCP domain the NF belongs to.

-	If the consumer is SCP, it may include:

-	SCP domain(s) the SCP belongs to.

-	Remote PLMNs reachable through SCP.

-	Endpoint addresses or Address Domain(s) (e.g. IP Address or FQDN ranges) accessible via the SCP.

-	NF sets of NFs served by the SCP.

-	If the consumer NF is MB-SMF, it may include MB-SMF service area, and the MBS Session ID(s), Area Session ID(s), MBS service area(s) if available, as specified in TS 23.247 [78].

-	If the consumer is DCCF, the request may include DCCF Serving Area information, NF type of the NF data source, NF Set ID of the NF data sources. Details about DCCF discovery and selection are described in clause 6.3.19 of TS 23.501 [2].

-	If the consumer is EASDF, it may include S-NSSAI, DNN, N6 IP address of the PSA UPF, location as per NF profile and DNAI (if exists).

-	For SNPN, if the consumer is SMF or AMF, Capability to support SNPN Onboarding.

Outputs, Required: Result indication.

Outputs, Optional: None.

See clause 5.21.2.1 in TS 23.501 [2], the AMF registers itself to NRF.

# * * * * Second change * * * *

5.2.7.2.5	Nnrf_NFManagement_NFStatusSubscribe service operation

Service Operation name: Nnrf_NFManagement_NFStatusSubscribe.

Description: Consumer can subscribe to be notified of the following:

-	Newly registered NF along with its NF services.

-	Updated NF profile.

-	Deregistered NF.

Inputs, Required: callback URI.

Inputs, Optional:

-	PLMN ID of the target NF/NF service, in the case of the subscription to the status of NF/NF service instance(s) in home PLMN from the serving PLMN, or PLMN ID and NID in the case of SNPN (see clause 5.30.2.9.3 of TS 23.501 [2]).

-	Home Network Identifier: PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN. Optionally, some NFs may additionally include a Home Network Identifier (including the identification of the CH with AAA server or DCS with AAA server)  in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of SNPN Onboarding using a DCS with AAA server.

-	Validity Time, in case to indicate the time instant after which the subscription becomes invalid.

-	For updated NF profile subscription, conditions that trigger a notification from NRF. Includes monitored attributes in the NF profile (changes trigger a notification) or unmonitored attributes in the NF profile (changes do not trigger a notification)

-	The following parameters are mutually exclusive:

-	NF Type (if NF status of a specific NF type is to be monitored).

-	NF Instance ID or NF Instance ID list (if NF status of a specific NF instance or a list of NF instance is to be monitored).

-	NF Service name (if NF status for NF which exposes a given NF service is to be monitored).

-	NF Set (if NF status of a set of NF Instances belonging to a certain NF Set is to be monitored).

-	NF Service Set (if the status of a set of NF Service Instances belonging to a certain NF Service Set is to be monitored).

-	NF Group (if the NF status of NF Instances identified by a NF (UDM, AUSF, PCF, BSF, CHF, HSS or UDR) Group Identity is to be monitored).

-	SCP Domain (if the status of NF or SCP instances belonging to a certain SCP domain is to be monitored).

-	For the UPF Management defined in clause 4.17.6: UPF Provisioning Information as defined in that clause.

-	For AMF, Consumer may include list of GUAMI(s), or AMF Set or AMF Region, or TAIs.

-	For SMF, Consumer may include list of TAIs.

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	For NWDAF, Consumer may include Analytics ID(s) (possibly per service), TAI(s), NF Set ID and NF Type of NF data sources. If consumer is NWDAF, Consumer may also include the Analytics Filter information for the trained ML model(s) per Analytics ID(s), if available. Details about NWDAF discovery and selection are described in clause 6.3.13 of TS 23.501 [2].

-	For NEF, Consumer may include Event ID(s) provided by AF.

-	For DCCF, Consumer may include TAI(s), NF type of the NF data sources, NF Set ID of the NF data sources. Details about NWDAF discovery and selection are described in clause 6.3.19 of TS 23.501 [2].

Outputs, Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Validity Time.

Outputs, Optional: None.

NOTE:	Alternatively, other means such as OA&M can also be used to subscribe for NF status.

# * * * * Third change * * * *

5.2.7.3.2	Nnrf_NFDiscovery_Request service operation

Service operation name: Nnrf_NFDiscovery_Request

Description: provides the IP address or FQDN of the expected NF instance(s) and, if present in NF profile, the Endpoint Address(es) of NF service instance(s) to the NF service consumer or SCP.

Inputs, Required: one or more target NF service Name(s), NF type of the target NF, NF type of the NF service consumer.

If the NF service consumer intends to discover an NF service producer providing all the standardized services, it provides a wildcard NF service name.

Inputs, Optional:

-	S-NSSAI and the associated NSI ID (if available), DNN, target NF/NF service PLMN ID (or realm in the case of network specific identifier type SUCI/SUPI, see clause 4.17.5a), NRF to be used to select NFs/services within HPLMN or Credentials Holder, Serving PLMN ID (or PLMN ID and NID in the case of SNPN, see clause 4.17.5a), the NF service consumer ID, preferred target NF location, TAI.

NOTE 1:	For network slicing the NF service consumer ID is a required input.

-	FQDN for the S5/S8 interface of the SMF+PGW-C, to discover the N11/N16 interface of the SMF+PGW-C in the case of EPS to 5GS mobility.

-	If the target NF stores Data Set(s) (e.g., UDR, BSF): SUPI, GPSI, IMPI, IMPU, Data Set Identifier(s). (UE) IPv4 address, IP domain or (UE) IPv6 Prefix.

NOTE 2:	GPSI is relevant for BSF.

NOTE 3:	If the request includes a subscriber identifier the NRF may need to use the association between the supplied subscriber identifier and the appropriate NF Group ID as described in clause 6.3.1 of TS 23.501 [2] to determine the applicable set of NF instances for the response.

NOTE 4:	The (UE) IPv4 address or (UE) IPv6 Prefix is provided for BSF discovery: in that case the NRF looks up for a match within one of the Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes provided by BSF(s) as part of the invocation of Nnrf_NFManagement_NFRegister operation. The NRF is not meant to store individual (UE) IPv4 addresses or (UE) IPv6 prefixes.

-	If the target NF is UDM or AUSF, the request may include the UE's Routing Indicator, or the UE's Routing Indicator and Home Network Public Key identifier.

-	If the target NF is UDM or AUSF, the request may include the UE's HNI: PLMN ID in the case of PLMN, PLMN ID + NID in the case of SNPN. Optionally, some NFs may additionally include a Home Network Identifier in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in the case of SNPN Onboarding using a DCS with AAA server.

-	If the target NF is NSSAAF, the request may include Home Network Identifier in the form of a realm e.g. in the case of access to an SNPN using credentials owned by CH with AAA Server or in case of SNPN Onboarding using credentials from a DCS with AAA server.

-	If the target NF is AMF, the request may include:

-	AMF region, AMF Set, GUAMI and Target TAI(s).

-	If the target NF is UDR or UDM or AUSF or PCF or BSF, the request may include UDR Group ID or UDM Group ID or AUSF Group ID or PCF Group ID or BSF Group ID respectively.

NOTE 5:	It is assumed that the corresponding NF service consumer is either configured with the corresponding Group ID or it received it via earlier Discovery output.

-	If the target NF is UDM, the request may include SUPI, GPSI, Internal Group ID and External Group ID.

If the target NF is UPF, the request may include SMF Area Identity, UE IPv4 Address/IPv6 Prefix, supported ATSSS steering functionality

NOTE 6:	The (UE) IPv4 address or (UE) IPv6 Prefix is provided for UPF discovery: in that case the NRF looks up for a match within one of the Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes provided by UPF as part of the invocation of Nnrf_NFManagement_NFRegister operation. The NRF is not meant to store individual (UE) IPv4 addresses or (UE) IPv6 prefixes.

NOTE 7:	Discovering UPF at PDU Session Establishment time and creating the N4 association assumes full connectivity between SMF and UPFs.

-	If the target NF is CHF, the request may include SUPI or GPSI as specified in TS 32.290 [42].

-	If the target NF is PCF or SMF, the request may include the MA PDU Session capability to indicate that a NF instance supporting MA PDU session capability is requested.

-	If the target NF is PCF, the request may include the DNN replacement capability to indicate that a NF instance supporting DNN replacement capability is preferred.

-	If the target NF is NWDAF, the request may include Analytics ID(s) (possibly per service), TAI(s), Analytics aggregation capability, Analytics metadata provisioning capability, a Real-Time Communication Indication per Analytics ID, NF Set ID and NF Type of the NF data sources. Details about NWDAF discovery and selection are described in clause 6.3.13, TS 23.501 [2].

NOTE 8:	Analytics metadata provisioning capability is only applicable when NF service consumer is NWDAF.

-	If the target NF is HSS, the request may include IMPI, and/or IMPU and/or HSS Group ID.

-	If the NF service consumer needs to discover NF service producer instance(s) within an NF instance, the request includes the target NF Instance ID and NF Service Set ID of the producer.

-	If the NF service consumer needs to discover NF service producer instance(s) in an equivalent NF Service Set within an NF Set, the request includes the identification of the equivalent NF service Set and NF Set ID of producer.

NOTE 9:	TS 29.510 [37] specifies the mechanism to identify equivalent NF Service Sets.

-	If the NF service consumer needs to discover NF service producer instance(s) in the NF Set, the request includes the target NF Set ID of the producer.

-	If the target NF is SMF, the request may include:

-	the UE location (TAI); or

-	TAI list.

-	If the target NF is P-CSCF, the request may include UE location information, UE IP address/IP prefix, Access Type.

-	If the target NF is NEF, the request may include Event ID(s) provided by AF, and optional AF identification as described in clause 6.2.2.3 of TS 23.288 [50]. When the consumer is an AF, the request may include an External Identifier, External Group Identifier, or a domain name. If the target NF is local NEF, the request may include the parameters of list of supported TAI or list of supported DNAI additionally.

NOTE 10:	A consumer NF (e.g. AMF or SMF) may use AF FQDN (e.g. FQDN of the USS) in domain name for discovery and selection of the UAS NF/NEF for UAV authentication and authorization as specified in TS 23.256 [80].

-	If the target NF is SMF, the request may include the Control Plane CIoT 5GS Optimisation Indication or User Plane CIoT 5GS Optimisation Indication.

-	If the target NF is NSACF, the request may include S-NSSAI(s) of the network served by NSACF, NSACF Serving Area information, and NSACF service capability. Details about NSACF discovery and selection are described in clause 6.3.22 of TS 23.501 [2].

-	If the target NF is SCP, the request may include information about:

-	SCP domain(s).

-	Remote PLMN reachable through SCP.

-	Endpoint addresses or Address Domain(s) (e.g. IP Address or FQDN ranges) accessible via the SCP.

-	NF sets of NFs served by the SCP.

-	If the target NF is MB-SMF, the request may include the MBS Session ID and Area Session ID. Details about MB-SMF discovery and selection are described in clause 7.1.2 of TS 23.247 [78].

-	If the target NF is 5G DDNMF, the request may include SUPI, IP Address or FQDN of 5G DDNMF.

-	If the target NF is DCCF, the request may include TAI(s), NF type of the NF data sources, NF Set ID of the NF data sources. Details about DCCF discovery and selection are described in clause 6.3.19 of TS 23.501 [2].

-	If the target NF is EASDF, the request may include S-NSSAI, DNN, N6 IP address of the PSA UPF, location as per NF profile and DNAI(if exist). Details about EASDF discovery and selection are described in clause 6.3.23 of TS 23.501 [2].

-	If the target NF is AMF or SMF, the request may include the support of SNPN Onboarding to indicate whether the target NF instance supports SNPN Onboarding or not.

Outputs, Required: A set of NF instances, a validity period for the discovery result, containing per NF Instance: NF type, NF instance ID, FQDN or IP address(es) of the NF instance and if applicable, a list of services instances, where each service instance has a service name, a NF service instance ID, and optionally Endpoint Address(es)

Endpoint Address(es) may be a list of IP addresses or an FQDN for the NF service instance.

NOTE 11:	SCPs does not have any service instances.

Outputs, Optional: Per NF instance, other information in the NF profile listed in clause 6.2.6 in TS 23.501 [2] related to the NF instance, such as:

-	NF load information.

-	NF capacity information.

-	NF priority information.

-	If the target NF stores Data Set(s) (e.g. UDR): Range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers, Data Set Identifier(s). If the target NF is BSF or P-CSCF: Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, Range(s) of SUPIs, range(s) of GPSIs.

NOTE 12:	Range of SUPI(s) is limited in this release to a SUPI type of IMSI as defined in TS 23.003 [33].

-	If the target NF is UDM, UDR, PCF, BSF or AUSF, they can include UDM Group ID, UDR Group ID, PCF Group ID, BSF Group ID, AUSF Group ID respectively.

-	If the target NF is HSS, it can include HSS Group ID.

-	For UDM and AUSF, Routing Indicator, or Routing Indicator and Home Network Public Key identifier.

-	If the target NF is AMF, it includes list of GUAMI(s). In addition, it may include list of GUAMI(s) for which it can serve as backup for failure/maintenance.

-	If the target NF is CHF, it includes primary CHF instance and the secondary CHF instance pair(s), if configured in CHF instance profile.

-	For the UPF Management: UPF Provisioning Information as defined in clause 4.17.6.

-	S-NSSAI(s) and the associated NSI ID(s) (if available).

-	Information about the location of the target NF (operator specific information, e.g. geographical location, data centre).

-	TAI(s).

-	PLMN ID.

-	If the target is PCF or SMF, it includes the MA PDU Session capability to indicate if the NF instance supports MA PDU session or not.

-	If the target is PCF, it includes the DNN replacement capability to indicate if the NF instance supports DNN replacement or not.

-	If the target NF is NWDAF, it includes the Analytics ID(s) (possibly per service), NF Set ID and NF Type of the NF data sources, if available, NWDAF Serving Area information. In addition, it includes Analytics aggregation capability and/ or Analytics metadata provisioning capability, if such capability is provided by the NWDAF. It may include the Supported Analytics Delay per Analytics ID. If consumer is NWDAF, it may also include the Analytics Filter information for the trained ML model(s) per Analytics ID(s), if available. Details about NWDAF specific information are described in clause 6.3.13, TS 23.501 [2].

NOTE 13:	The Supported Analytics Delay is provided for an Analytics ID only when the NRF had received Real-Time Communication Indication for this Analytics ID in the NWDAF discovery request.

-	If the target is a trusted AF, it includes one or multiple combination(s) of the S-NSSAI and DNN corresponding to the AF. In addition, it may include supported Application Id(s), Event ID(s) supported by the AF and Internal-Group Identifier.

-	NF Set ID.

-	NF Service Set ID.

-	If the target NF is SMF, it may include the SMF(s) Service Area.

NOTE 14:	If no SMF Service Area is provided, the AMF assumes that a SMF can serve the whole PLMN.

-	If the target NF is P-CSCF, it includes P-CSCF FQDN(s) or IP address(es) and optional Access Type(s) associated with each P-CSCF.

-	If the target NF is NEF, it may include Event ID(s) provided by AF and/or it includes one or multiple combination(s) of the S-NSSAI and DNN corresponding to the untrusted AF served by the NEF.

-	SCP domain the NF belongs to.

NOTE 15:	Only one SCP domain is registered in NF profile for an NF.

-	If the target is SCP:

-	SCP domain(s).

-	Remote PLMNs reachable through SCP.

-	Endpoint addresses or Address Domain(s) (e.g. IP Address or FQDN ranges) accessible via the SCP.

-	NF sets of NFs served by the SCP.

-	If the target NF is 5G DDNMF, it may include IP Address or FQDN of 5G DDNMF.

-	If the target NF is MB-SMF, it may include the MBS Session ID(s), Area Session ID(s), MBS service area(s) as described in TS 23.247 [78].

-	If the target NF is DCCF, it includes DCCF serving area information, NF type of the NF data sources, NF Set ID of the NF data sources. Details about DCCF specific information are described in clause 6.3.19 of TS 23.501 [2].

See clause 4.17.4 and 4.17.5 for details on the usage of this service operation.

# * * * * End of changes * * * *