3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107536r04

# Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0462 | rev |  | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the Redundant Transmission Experience related analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As defined in the current specification, the description for the input data i.e. “UP with redundant transmission” in the Table 6.13.2-2 is not clear. It seems that the “UP with redundant transmission” indicates a time period during the redundant transmission is setup. During that time period, the Packet drop and/or packet delay measurement per QFI or GTP level could be collected to support data analytics on Redundant Transmission Experience. Furthermore, the description for the Redundant Transmission Experience in Table 6.13.3-1 and Table 6.13.3-2 is also not clear. Based on the input data as indicated in clause 6.13.2, the Redundant Transmission Experience may include the UL/DL packet drop rate or the UL/DL packet delay. Also, in clause 6.13.4, step 4: The NWDAF provide requested UE Presence Pattern analytics to the NF Where "UE Presence Pattern analytics" should be "Redundant Transmission Experience" to align with the requested Analytics ID in step 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the Redundant Transmission Experience related analytics. And some editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current descriptions of Redundant Transmission Experience related analytics is not clear enough. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.13.1, 6.13.2, 6.13.3, 6.13.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 6.13.1	General

This clause describes the Redundant Transmission Experience related analytics. This analytics may be used by the SMF to determine whether redundant transmission shall be performed, or (if it had been activated) shall be stopped.

The service consumer may be a NF (e.g. SMF).

The consumer of these analytics may indicate in the request:

-	Analytics ID = "Redundant Transmission Experience".

-	The Target of Analytics Reporting can be a single UE, any UE, or a group of UEs.

-	Analytics Filter Information optionally containing:

-	Area of Interest;

-	S-NSSAI;

-	DNN;

-	optional list of analytics subsets that are requested (see clause 6.13.3).

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Preferred level of accuracy of the analytics;

-	Preferred order of results for the list of Redundant Transmission Experience:

-	ordering criterion: "time slot start" or "Redundant Transmission Experience"; and

-	order: ascending or descending;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

# * * * * Next change * * * *

### 6.13.2	Input Data

The NWDAF supporting data analytics on Redundant Transmission Experience shall be able to collect UE mobility information from OAM, 5GC and AFs, and service data from AF, as described in clause 6.7.2.2. In addition, it shall be able to collect the information for PDU session which is established with redundant transmission from NF and OAM. UE mobility information is specified in Table 6.7.2.2-1 and service data from AF related to UE mobility in Table 6.7.2.2-2. In addition, it shall be able to collect performance measurement on user data congestion as specified in Table 6.8.2-1.

Additionally, the NWDAF collects the following input (see Table 6.13.2-1 and Table 6.13.2-2) according to existing measurements defined in clause 5.33.3 QoS Monitoring to Assist URLLC Service of TS 23.501 [2].

Table 6.13.2-1: Packet drop and/or packet delay measurement per QFI or GTP level

| Information | Source | Description |
| --- | --- | --- |
| UL/DL packet drop rate GTP-U | OAM (see NOTE 3) | UL/DL packet drop rate measurement on GTP path on N3. |
| UL/DL packet delay GTP E2E | UPF (see NOTE 2) | End-to-End delay measurements from UE to UPF. |
| UL/DL packet delay GTP-U | OAM (see NOTE 3) | UL/DL packet delay measurement round trip on GTP path on N3. |
| NOTE 1: The information in this table is provided both as the base to compare with the redundant transmission performance as well as when redundant transmission is enabled. NOTE 2: How NWDAF collects the information from UPF is not defined in this specification. NOTE 3: Refer to clause 5.1 of TS 28.552 [8] for the performance measurement in NG-RAN and clause 5.4.1 of TS 28.552 [8] for the performance measurement in UPF. In addition, Annex A of TS 28.552 [8] describes various performance measurements, in particular, clause A.61 indicates that the measurements on the one way DL and UL delay between PSA UPF and NG-RAN can be used to evaluate and optimize the DL and UL user plane delay performance between 5GC and NG-RAN. NOTE 4: The NWDAF data collection request/subscription will not activate the measurements for QoS Monitoring. When QoS Monitoring is activated the measurements are triggered according to existing procedures. When QoS Monitoring is not activated SMF may per configuration activate measurements for QoS Monitoring at least at the same time as it requests Analytics. |  |  |

Table 6.13.2-2: The information related to PDU Session established with redundant transmission

| Information | Source | Description |
| --- | --- | --- |
| DNN | SMF | Data Network Name associated for URLLC service. |
| UP with redundant transmission | SMF | Time period during the redundant transmission is setup. |

# * * * * Next change * * * *

### 6.13.3	Output Analytics

The NWDAF supporting data analytics on Redundant Transmission Experience shall be able to provide Redundant Transmission Experience analytics as defined in Table 6.13.3-1 and Table 6.13.3-2.

Table 6.13.3-1: Redundant Transmission Experience statistics

| Information | Description |
| --- | --- |
| UE group ID or UE ID, any UE | Identifies a UE, any UE, or a group of UEs. |
| DNN | Data Network Name associated for URLLC service. |
| Spatial validity | Area where the Redundant Transmission Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity should be the requested Area of Interest. |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot (average and variance). |
| > Redundant Transmission Experience | Observed Redundant Transmission Experience value during the Analytics target period derived from observed UL/DL packet drop rate GTP-U, UL/DL packet delay GTP_E2E and UL/D packet delay GTP-U. (Note 1) |
| >> UL/DL packet drop rate GTP-U (NOTE 2) | Observed UL/DL packet drop rate on GTP-U path on N3 (average, variance). |
| >> UL/DL packet delay GTP_E2E (NOTE 2) | Observed End-to-End delay from UE to UPF (average, variance). |
| >> UL/DL packet delay GTP-U (NOTE 2) | Observed UL/DL packet delay round trip on GTP-U path on N3 (average, variance). |
| > Ratio | Percentage on which UE, any UE, or UE group efficiently use the PDU session with redundant transmission. |
| Note 1: The Redundant Transmission Experience is a value between 0 and 5. It is a weighted combination of the observed UL/DL packet drop rate GTP-U, UL/DL packet delay GTP_E2E and UL/D packet delay GTP-U. The weight values are tuneable parameters. NOTE 2: This information element is an analytics subset that can be used in "list of analytics subsets that are requested". |  |

Table 6.13.3-2: Redundant Transmission Experience predictions

| Information | Description |
| --- | --- |
| UE group ID or UE ID, any UE | Identifies a UE or, any UE, a group of UEs. |
| DNN | Data Network Name associated for URLLC service. |
| Spatial validity | Area where the estimated Redundant Transmission Experience applies. If Area of Interest information was provided in the request or subscription, spatial validity should be the requested Area of Interest. |
| Time slot entry (1..max) | List of predicted time slots. |
| >Time slot start | Time slot start time within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > Redundant Transmission Experience | Predicted Redundant Transmission Experience value during the Analytics target period derived from predicted UL/DL packet drop rate GTP-U, UL/DL packet delay GTP_E2E and UL/D packet delay GTP-U. |
| >> UL/DL packet drop rate GTP-U (NOTE 2) | Predicted UL/DL packet drop rate on GTP-U path on N3 (average, variance). |
| >> UL/DL packet delay GTP_E2E (NOTE 2) | Predicted End-to-End delay from UE to UPF (average, variance). |
| >> UL/DL packet delay GTP-U (NOTE 2) | Predicted UL/DL packet delay round trip on GTP-U path on N3 (average, variance). |
| > Ratio | Percentage on which the UE, any UE, or UE group may efficiently use the PDU session with redundant transmission. |
| > Confidence | Confidence of this prediction. |
| Note 1: The Redundant Transmission Experience is a value between 0 and 5. It is a weighted combination of the predicted UL/DL packet drop rate GTP-U, UL/DL packet delay GTP_E2E and UL/D packet delay GTP-U. The weight values are tuneable parameters. NOTE 2: This information element is an analytics subset that can be used in "list of analytics subsets that are requested". |  |

# * * * * Next change * * * *

#### 6.13.4.1	Analytics Procedure

Figure 6.13.4.1-1 shows the analytics procedure. The NWDAF can provide analytics, in the form of statistics or predictions or both.

Figure 6.13.4.1-1: Redundant Transmission Experience analytics provided to an NF

1.	The analytics consumer sends a request to the NWDAF for analytics on a specific UE, any UE, or a group of UEs, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service. The NF can request statistics or predictions or both. The type of analytics is set to Redundant Transmission Experience. The NF provides the UE id or Internal Group ID in the Target of Analytics Reporting. Analytics Filter Information optionally contains DNN, S-NSSAI, Area of Interest, etc.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to events with all the serving AMFs for notification of location changes, and may subscribe to events with SMFs serving PDU Session on URLLC service for notification of redundant transmission related information.

The NWDAF may subscribe the service data from AF(s) by invoking Naf_EventExposure_Subscribe service or Nnef_EventExposure_Subscribe (if via NEF).

The NWDAF collects UE mobility information, packet measurement information and/or redundant transmission related information from OAM, following the procedure captured in clause 6.2.3.2.

NOTE:	The NWDAF determines the AMF serving the UE, any UE, or the group of UEs as described in clause 6.2.2.1.

This step may be skipped when e.g. the NWDAF already has the requested analytics available.

3.	The NWDAF derives requested analytics.

4.	The NWDAF provide requested Redundant Transmission Experience analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1.

5-7.	If at step 1, the NF has subscribed to receive notifications for Redundant Transmission Experience analytics, after receiving event notification from the AMFs, AFs and OAM subscribed by NWDAF in step 2, the NWDAF may generate new analytics and provide them to the NF.

If a service consumer is SMF, the Redundant Transmission Experience analytics can be used to make decision if that redundant transmission shall be performed or (if activated) shall be stopped regarding the PDU session for URLLC service.

# * * * * End of changes * * * *