3GPP TSG-WG SA2 Meeting #147E e-Meeting	S2-2107537r02

# Elbonia,  Oct 18 - Oct 22, 2021													   (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0020 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mechanisms to deliver MNO DNS configuration from 5GC to AF |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-09-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 23.548, an editor’s note says that the MNO DNS configuration in UE OS might be overrided and may not be used for FQDN resolving. So in this paper, another mechanism is designed. The 5GC delivers the MNO DNS configuration to AF or reuses the CAPIF architeture that exposure the MNO DNS configuration via the API. After obtaining the MNO DNS configuration, the AF can establish the session towards UE APP client, and delivers MNO DNS configuration to application client. Then, the application client can directly use the DNS configuration to trigger the DNS query. This design can avoid the situation of UE OS override the MNO DNS configuration and make sure the application which wants to use the MNO DNS service, to deliver DNS query to the DNS server MNO indicated. Also, this design has no UE impact. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 5GC provides the MNO DNS configuration to AF. AF establishes the PDU session towards UE APP client, and AF delivers the MNO DNS configuration to applicaton client. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | If the MNO DNS configuration in UE OS is overridden, the DNS query from application may not be sent to MNO DNS configuration. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Change 1 * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	3GPP TS 23.558: "Architecture for enabling Edge Applications (EA)".

[6]	IETF RFC 7871: "Client Subnet in DNS Queries".

[7]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[8]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[9]	3GPP TS 29.500: "Technical Realization of Service Based Architecture; Stage 3".

[10]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[X]	3GPP TS 23.222: " Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs ".

* * * End of changes* * *

* * * Start of Change 2 * * *

6.2.1	General

In Edge Computing deployment, an application service may be served by multiple Edge Application Servers typically deployed in different sites. These multiple Edge Application Servers that host service may use a single IP address (anycast address) or different IP addresses. To start an Edge Application Service, the UE needs to know the IP address(es) of the Application Server(s) serving the Service. The UE may do a discovery to get the IP address(es) of a suitable Edge Application Server (e.g. the closest one), so that the traffic can be locally routed to the Edge Application Server and service latency, traffic routing path and user service experience can be optimized.

EAS Discovery is the procedure by which a UE discovers the IP address(es) of a suitable Edge Application Server(s) using Domain Name System (DNS). EAS Re-discovery is the EAS Discovery procedure that takes place when the previously discovered Edge Application Server cannot be used or may have become non-optimal (e.g. at edge relocation).

DNS server may be deployed in different locations in the network as Central DNS (C-DNS) resolver/server or as Local DNS (L-DNS) resolver/server.

NOTE 1:	The C-DNS server and/or L-DNS resolvers/servers can use an anycast address.

NOTE 2:	The C-DNS server or L-DNS resolvers/servers can contact any other DNS servers for recursive queries, which is out of scope of this specification.

NOTE 3:	This specification describes the discovery procedure based on 5GS NFs as to ensure the UE is served by the application service closest to the UE's point of attachment. However, this does not exclude other upper layer solution that can be adopted by operator or service provider, like the EAS Discovery procedure defined in TS 23.558 [5], or other alternatives shown in Annex A and Annex B. How those other solutions work, or whether they are able to guarantee the closest application service for the UE, is out of the scope of this specification.

In order to provide a translation of the FQDN of an EAS into the address of an EAS as topologically close as possible to the UE, the Domain Name System may use following information:

-	The source IP address of the incoming DNS Query; and/or,

-	an EDNS Client Subnet (ECS) option (as defined in RFC 7871 [6]).

NOTE 4:	UE IP address can be subject to privacy restrictions, which means that it is not to be sent to Authoritative DNS / DNS Resolvers outside the network operator within EDNS Client Subnet option or as Source IP address of the DNS Query. UE source IP address can be protected by using NAT mechanism.

EAS (re-)discovery procedures described in this specification should use the top level domains (TLDs) in the public namespace by default.

If a private namespace is used, an Edge Computing Service Provider (ECSP) can provision DNS information in the EAS Deployment information via AF request with its AF-service-identifier, or DNN and NSSAI. Since private namespaces do not have a common root server or naming, the DNS information for each ECSP should be stored individually to prevent any overwriting of resolution entries.

NOTE 5:	The DNS information provided by ECSP in the EAS Deployment Information can be used to select the DNS settings for a PDU Session mainly if the PDU Session is specific for the ECSP services.

If the UE applications want to discover/access EAS by using the mechanisms defined in this TS, the UE shall support receiving DNS settings in PCO during PDU Session Establishment and PDU Session Modification, and the DNS queries generated by the UE shall be sent to the EASDF as DNS resolver indicated by the SMF.

The UE applications may get the DNS settings which indicated by 5GC from trusted AF which is controlled by MNO via user plane connection. The details can be referred to section 6.2.4.

The case of EAS (Re-)discovery over Distributed Anchor connectivity model is described in clause 6.2.2. For Multiple PDU Sessions connectivity model, the description in clause 6.2.2 also applies to the PDU Session(s) with Local PSA. The case of EAS (Re-)discovery over Session Breakout connectivity model is described in clause 6.2.3.

* * * End of changes* * *

* * * Start of Change 4(All new text) * * *

6.2.4	DNS setting provision to Application Client by trusted/MNO AF

The 5GC may interact with an AF (may via NEF) to provide the DNS settings (the same as the DNS setting in PCO delivered to UE) to UE Application(s) client. The AF is in the MNO domain.

The UE Application establishes a connection to the AF in the MNO domain over user plane via a PDU session. The AF communicates with the UE Application and provides DNS settings to UE Application. UE Application client which wants to use the MNO DNS service, can use this DNS settings to trigger the DNS query towards the DNS server indicated by MNO.

For an AF in trusted domain (which supports to provide DNS settings to UE Application), the SLA between the operator and the Application Service Provider (i.e. ASP) determines per Application ID in use by the ASP:

-	The AF for the UE Application to connect to (e.g. based on an FQDN).

-	The authentication information that enable the AF to verify the authenticity of the UE's Application.

Figure 6.2.4.1: Procedure for DNS settings provision to AF by 5GC and for DNS settings provision to Application by AF

(Option 1: 5GC provides DNS setting to AF, step 1)

1.	(Option 1) 5GC provides DNS setting to AF which is the same as the DNS setting indicated by SMF via PCO to UE. The SMF notifies the NEF of the DNS setting by invoking Nsmf_EventExposure_Notify service operation, and the NEF triggers the appropriate Nnef_TrafficInfluence_Notify message to deliver DNS setting towards AF.

(Option 2: AF invokes CAPIF API to get DNS setting, from step 2 to 5)

2.	The CAPIF supports publishing service APIs by the API provider, e.g. 5GC, SMF or NEF. The API invoker, e.g.:AF, can get the DNS setting from this service API. This step reuses the procedure in section 8.3.3 of TS 23.222[x].

3.	This step corresponds to the architectural requirements on discover service APIs. The AF can discover this API which can provide DNS setting according to the procedure in section 8.7.3 of TS 23.222[x].

4.	The procedure corresponds to the architectural requirements for authentication between the API invoker and the CAPIF core function. Upon receiving the authentication request, the CAPIF core function verifies the identity with the API invoker profile and authenticates the API invoker. This procedure refers to the section 8.10 of TS 23.222[x].

NOTE 1:	The security aspects and authentication process is specified in subclause 6.2 and subclause 6.3.1 of 3GPP TS 33.122 [12].

5.	The AF invokes the service API which can provide the DNS setting, and get the DNS setting from CAPIF core function, e.g. SMF, NEF or 5GC.

6.	The UE Application establishes a connection to the AF in the MNO domain or external to MNO domain over user plane via a PDU session. The AF communicates with the UE Application and provides DNS settings to UE Application.

7.	UE Application client which wants to use the MNO DNS service, can use this DNS settings to trigger the DNS query towards the DNS server indicated by MNO.

* * * End of changes* * *