3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2107547

# Elbonia, October 18 – 22, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0027 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on User Plane Management |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Clause 6.7 specifies the user plane processing for DL MBS data. There are two types of the tunnel between MB-UPF and NG-RAN/PSA: tunnel for multicast transport (i.e., common tunnel) and tunnel for unicast transport (i.e., separate unicast tunnel). However, although current clause 6.7 includes the descriptions related to issues above, it is not clear that in which case the multicast/unicast transport shall be used. Which may lead to unclear stage 3 implementation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document tries to clarify the cases for using multicast transport and unicast tranport. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear for stage 3 implementation on how to manage the tunnel between MB-UPF and PSA/NG-RAN. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

## 6.7	User plane management

The MB-UPF acts as the MBS Session Anchor of an MBS session, and if the MBSTF is involved in the MBS session, then the MBSTF acts as the media anchor of the MBS traffic. The MB-UPF receives only one copy of MBS data packets from AF or MBSTF.

The user plane between MBSTF and MB-UPF, or between MB-UPF and AF, may use either multicast transport or unicast tunnel for the MBS session (depending on application and capabilities of control interface). If the transport network does not support multicast transport, the user plane uses unicast tunnel for the MBS Session. The user plane between MBSTF and AF may use unicast tunnel, multicast transport or other means (e.g., HTTP download from external CDN). Unicast is used for the MBS Session, after receiving the downlink MBS data, the MB-UPF forwards the downlink MBS data without the outer IP header and tunnel header information.

The user plane data from the MB-UPF may be delivered to the following network elements in the following way:

-	For 5GC Shared MBS traffic delivery, if the transport network supports IP multicast and the NG-RAN node(s) supports MBS (i.e. if the NG-RAN node(s) supports reception of multicast data at N3mb and PTM/PTP towards UEs), NG-RAN node(s) may use multicast transport via a common GTP-U tunnel per MBS session; otherwise unicast transport via separate GTP-U tunnels per MBS session to the NG-RAN node(s) shall be used.

-	For 5GC Individual MBS traffic delivery, if the transport network supports IP multicast and the UPF(s) supports reception of multicast data over N19mb, UPF(s) may use multicast transport via a common GTP-U tunnel per MBS session; otherwise unicast transport via separate GTP-U tunnels per MBS session to the UPF(s) shall be used.

If the user plane uses unicast transport, the transport layer destination is the IP address of the NG-RAN or UPF, each NG-RAN or UPF allocates the tunnel separately and multiple GTP-U tunnels are used for the MBS Session. If the user plane uses multicast transport, a common GTP-U tunnel is used for both RAN and UPF nodes. The GTP-U tunnel is identified by a common tunnel ID and an IP multicast address as the transport layer destination, both assigned by 5GC.

The above is depicted in Figure 6.7-1. There could be more than one NG-RANs or UPFs that involves in the MBS traffic delivery.

Figure 6.7-1: Schematic showing user plane data transmission

The MB-SMF configures the MB-UPF to receive packets related to an MBS session.

For shared delivery, if unicast transport over N3mb applies, the MB-SMF configures MB-UPF to replicate the received MBS packets and forward them towards multiple RAN nodes via separate GTP tunnel. For shared delivery, if multicast transport over N3mb applies, the MB-SMF configures the MB-UPF to replicate the received MBS data and forwards the data via a single GTP tunnel.

For individual delivery, the MBS data received by the MB-UPF is replicated towards the UPF(s) where individual delivery is performed in the following way:

-	The MB-SMF configures the MB-UPF to receive packets related to an MBS session, to replicate those packets and forward them towards multiple UPFs via GTP tunnels if unicast transport over N19mb is applied, or via a single GTP tunnel if multicast transport over N19mb is applied.

-	The SMF(s) configures the UPF to receive packets related to a multicast session from an MB-UPF over N19mb, to replicate those packets and to forward them in multiple PDU sessions.

For the SMF and UPF, traffic replication and forwarding for an MBS session is realized by using for each MBS session an UPF internal interface ("MBS internal") and a two-step detection and forwarding process. In the first step, the packets received from a single data source are forwarded by the UPF to the UPF internal interface (i.e. Destination Interface set to "MBS internal" and MBS session ID indicated as Network instance in an FAR). In the second step, PDRs installed at the UPF internal interface (i.e. Source Interface set to "MBS internal" and MBS session ID indicated as Network instance) detect the packets and forward them to the respective outgoing interface.

NOTE:	This enables separate PFCP sessions for the related PDU sessions and for the reception of the MBS session.

For the MB-SMF and MB-UPF, traffic replication and forwarding for an MBS session is realized by using for each MBS session one PDR that detects the incoming MBS packets and points to one FAR that describes the forwarding of the data towards multiple destinations (UPFs or RAN nodes):

-	A PFCP session is created when the MBS Session is started, regardless of multicast or unicast transport over N3mb and N19mb.

-	For Multicast transport over N3mb and N19mb, the destination in the FAR contains the MB-UPF IP Multicast Distribution Info.

-	For unicast transport over N3mb and N19mb, the FAR in the PFCP session may contain multiple destinations represented by the NG-RAN N3mb Tunnel Info and UPF N19mb Tunnel Info (if applicable).

Editor's note:	Whether a two-step approach for 5G VN group is needed for SMF and UPF and thus whether new source/destination source types are needed for MBS traffic is FFS and needs CT WG4 feedback.

# * * * * End of changes * * * *