3GPP TSG-SA2 Meeting #147e	S2-2107584r02

Electronic, October 18-22, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3319 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for Paging Subgrouping for Paging Early Indication |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc., Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI17, NR_UE_pow_sav_enh |  |  |  |  |  | Date: |  |  | 2021-09-27 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Paging Early Indication is being introduced for NR as part of Rel-17 UE power saving, see discussion in S2-2107583. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Support for NR Paging Early Indication in RRC_IDLE and RRC_INACTIVE. - Support of Paging Early Indication support between UE and AMF. - Support of CN assigned Paging Early Indication with Paging subgrouping assistance information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for Paging Early Indication over NR and related inconsistency with RAN specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.4a, 5.4.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

**** FIRST CHANGE ****

### 5.4.4a	UE MM Core Network Capability handling

The UE MM Core Network Capability is split into the S1 UE network capability (mostly for E-UTRAN access related core network parameters) and the UE 5GMM Core Network Capability (mostly to include other UE capabilities related to 5GCN or interworking with EPS) as defined in TS 24.501 [47] and contains non radio-related capabilities, e.g. the NAS security algorithms, etc. The S1 UE network capability is transferred between all CN nodes at AMF to AMF, AMF to MME, MME to MME, and MME to AMF changes. The UE 5GMM Core Network Capability is transferred only at AMF to AMF changes.

In order to ensure that the UE MM Core Network Capability information stored in the AMF is up to date (e.g. to handle the situation when the USIM is moved into a different device while out of coverage, and the old device did not send the Detach message; and the cases of inter-RAT Registration Area Update), the UE shall send the UE MM Core Network Capability information to the AMF during the Initial Registration and Mobility Registration Update procedure within the NAS message.

The AMF shall store always the latest UE MM Core Network Capability received from the UE. Any UE MM Core Network Capability that an AMF receives from an old AMF/MME is replaced when the UE provides the UE MM Core Network Capability with Registration signalling.

If the UE's UE MM Core Network Capability information changes (in either CM-CONNECTED or in CM-IDLE state), the UE shall perform a Mobility Registration Update procedure when it next returns to NG-RAN coverage. See clause 4.2.2 of TS 23.502 [3].

The UE shall indicate in the UE 5GMM Core Network Capability if the UE supports:

-	Attach in EPC with Request type "Handover" in PDN CONNECTIVITY Request message (clause 5.3.2.1 of TS 23.401 [26]).

-	EPC NAS.

-	SMS over NAS.

-	LCS.

-	5G SRVCC from NG-RAN to UTRAN, as specified in TS 23.216 [88].

-	Radio Capabilities Signalling optimisation (RACS).

-	Network Slice-Specific Authentication and Authorization.

-	Parameters in Supported Network Behaviour for 5G CIoT as described in clause 5.31.2.

-	Receiving WUS Assistance Information (E-UTRA) and Paging Early Indication with Paging Subgrouping Support Information (NR).

-	CAG, see clause 5.30.3.3.

-	Subscription-based restrictions to simultaneous registration of network slices (see clause 5.15.12).

If a UE operating two or more USIMs, supports and intends to use one or more Multi-USIM features (see clause 5.38) in a PLMN for a USIM, it shall indicate in the UE 5GMM Core Network Capability for this USIM in this PLMN that it supports these one or more Multi-USIM features with the following indications:

-	Connection Release Supported.

-	Paging Cause Indication for Voice Service Supported.

-	Reject Paging Request Supported.

-	Paging Restriction Supported.

Otherwise, the UE with the capabilities of Multi-USIM features but does not intend to use them shall not indicate support of these one or more Multi-USIM features.

A UE not operating two or more USIMs shall indicate the Multi-USIM features are not supported.

NOTE:	It is not necessary for a UE operating two or more USIMs to use Multi-USIM features with all USIMs.

**** NEXT CHANGE ****

### 5.4.X	Paging Early Indication with Paging Subgrouping Assistance

### 5.4.X	General

The RAN and UE may use a Paging Early Indication with Paging Subgrouping (PEIPS) to reduce the UE's power consumption in RRC_IDLE and RRC_INACTIVE over NR (see TS 38.304 [50]).

### 5.4.X.2	Core Network Assistance for PEIPS

To support the Paging Early Indication with Paging Subgrouping (PEIPS), PEIPS Support information and the PEIPS Assistance Information is used by the NG-RAN to help determine whether PEIPS applies to the UE and the PEIPS sub-group used when paging the UE (see TS 38.300 [27]).

Editor’s note: The content of the PEIPS Assistance Information is to be determined by RAN2.

The UE may in the Registration Request message provide its PEIPS Support Information. If PEIPS Support Information is provided by the UE, then the UE in the Registration Request message may provide UE paging probability information. If the AMF support PEIPS assistance, the AMF indicates PEIPS Support Information to UE in Registration Accept message. The AMF may in addition, based on the PEIPS Support Information provided by the UE, use local configuration and/or previous statistical information for the UE, and if provided the UE paging probability information, to determine the PEIPS Assistance Information.

If the AMF has determined PEIPS Assistance Information for the UE, the AMF provides it to the UE in every Registration Accept message.

If the AMF provided the PEIPS Support Information to the UE, the AMF stores the PEIPS Support Information in the MM context and provides the PEIPS support information to NG-RAN in paging request. In addition, in order to support PEIPS for UEs in RRC_INACTIVE mode, the AMF provides the PEIPS Support Information to NG-RAN as part of the RRC Inactive Assistance Information

If the AMF has determined PEIPS Assistance Information, the AMF stores the PEIPS Assistance Information parameter in the MM context and provides it to the NG RAN when paging the UE. In addition, in order to support PEIPS for UEs in RRC_INACTIVE mode, the AMF provides the PEIND Assistance Information to NG-RAN as part of the RRC Inactive Assistance Information.

Both the UE and NG-RAN use the PEIPS Support Information and the PEIPS Assistance Information to determine whether PEIPS applies and the paging subgroup tp apply as defined in TS 38.300 [27].

UE and AMF shall not signal PEIPS Assistance Information in Registration Request, Registration Accept messages when the UE has an active emergency PDU session.

Editor’s note: It is FFS in conjunction with RAN1/RAN2 the handling of false wake ups from UEs in same paging subgroup.

**** END OF CHANGES ****