3GPP TSG-SA2 Meeting #147-e	S2-2107608

# Online, , 18th-22nd Oct 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 035 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification on handover procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo Mobile Communications Co. LTD, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | [7.2.3.2], [7.2.3.3] 1. It is not clear how END MARK is used for MBS session. Further more, SMF involves in PATH SWITCH but MB-SMF does not, UPF is not know when to send END MARK if shared tunnel for target is newly created. 2. Duplicate description for N19mb shared tunnel management, which has been described in session join procedure. 3. Forwarding is still discussed by RAN WG3, no conclusion [7.2.4.2.3] 4. RAN3 indicates a issue that if MBS related informatin sent to target that not support MBS, “full configuration” may be used and all services will be interrupted. 5. This clause only focus on handling of Area Session ID, but service area also needs to be handled. [7.2.4.3.3] 6. Source RAN and Target RAN both check the MBS service area isagreed in RAN WG. SMF performs area restriction if needed. 7. Only consider the source RAN is MBS capable. 8. Does not consider the case when handover from outside to inside. 9. Some editorial modification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | [7.2.3.2], [7.2.3.3] 1. Remove END MARK in figure. 2. Refer to session join procedure for N19mb shared tunnel management. 3. Remove forwarding in figure. [7.2.4.2.3] 4. Add a EN that whether source RAN send MBS related information in the same container for PDU Session related information needs RAN3 confirmation. 5. Add a bullet that 7.2.4.3.3 for handling of service area also is applied. [7.2.4.2.3] 6. Specify that source RAN also performs MBS service area check and SMF also performs if needed. 7. Add bullets for the cases when source RAN is non-MBS capable, including Xn and N2 HO. 8. Add bullet for handover from outside to inside. 9. Some editorial modification. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification is not clear |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.2, 7.2.3.3, 7.2.3.4, 7.2.4.2.3, 7.2.4.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.2.3.2	Xn based handover from MBS supporting NG-RAN node

This clause describes an Xn based handover with MBS traffic delivered to the UE at the source NG-RAN node supporting 5G MBS.

Figure 7.2.3.2-1: Xn based handover with MBS Session

The following additions apply compared to clause 4.9.1.2 of TS 23.502 [6]:

Before Handover:

The source NG RAN has been provided with MBS Session Resource information (including the MBS Session ID and multicast QoS flow information) and the UE Context information contains a mapping information within the PDU Session Resource associated with the MBS Session Resource, e.g. including mapped unicast QoS Flows associated with the multicast QoS flow(s) of the MBS Session Resource.

Handover Preparation Phase:

At Xn handover, the target NG-RAN is provided with information by the source NG-RAN which causes:

-	a 5G MBS non-supporting target NG-RAN node to prepare the unicast resources according to associated QoS flow(s) information.

‐	a 5G MBS supporting target NG-RAN node to allocate to the UE shared NG-RAN resources according to the MBS session information. If the 5GC Shared MBS traffic delivery for the indicated multicast MBS Session has not been established in target NG-RAN, target NG-RAN triggers setup of the resources for the 5GC shared MBS traffic delivery, see clause 7.2.1.4 for details. This is parallel with following steps 1-6.

1.	Target NG-RAN to AMF: the target NG-RAN sends N2 Path Switch Request to AMF.

The target NG-RAN node indicates whether it supports of MBS to SMF in N2 SM information. Per the received N2 SM information, the SMF knows whether the target NG-RAN node supports 5G MBS and determines the delivery method, i.e., whether the 5GC Shared MBS traffic delivery or 5GC Individual MBS traffic delivery is used for MBS data transferring.

The SMF differentiates two cases:

Case A)	 The target NG-RAN supports 5G MBS. Step 3 applies and step 4 is skipped.

3.	SMF to UPF: The SMF invokes N4 Session Modification procedure with the UPF (PSA) only for unicast PDU Session.

Case B)	 The target NG-RAN does not support 5G MBS. Step 3 is skipped, steps 4 applies.

4.	This steps is same as described in step 13 of clause 7.2.1.3.

Editor's note:	Details on data forwarding, if applicable, needs to wait for RAN WGs.

5.	SMF to AMF: The SMF responds to AMF through Nsmf_PDUSession_UpdateSMContext response.

6.	AMF to target NG-RAN: The AMF sends the path switch Ack to target NG-RAN.

# * * * * Second change * * * *

#### 7.2.3.3	N2 based handover from MBS supporting NG-RAN node

This clause describes the N2 based handover with MBS traffic delivered to the UE at the source NG-RAN node supporting 5G MBS.

Figure 7.2.3.3-1: N2 based handover with MBS Session

The following additions apply compared to clause 4.9.1.3 of TS 23.502 [6]):

1.	Source NG-RAN to S-AMF: Handover Required (RAN container (MBS Session information, associated PDU session information, associated QoS flow information and corresponding multicast QoS flow information)).

2.	S-AMF to T-AMF: The T-AMF is provided with associated PDU Session information and the MBS session related information.

4.	T-AMF to Target NG-RAN: The Target NG-RAN prepares the radio resource based on the received information:

-	If the Target NG-RAN does not support MBS, the MBS Session related information is not used. The Target NG-RAN uses the associated PDU Session information to allocate resource to deliver MBS data. The MBS data are transmitted via the associated QoS flows within the associated PDU Session.

-	If the Target NG-RAN supports MBS, the Target NG-RAN uses the multicast MBS Session related information to allocate RAN resources to deliver the MBS data. If 5GC Shared MBS traffic delivery for the indicated multicast MBS session has not been established towards the Target NG-RAN, the Target NG-RAN initiates the shared delivery establishment towards the MB-SMF via AMF as described in clause 7.2.1.4.

5.

Target NG-RAN to T-AMF: The target NG-RAN sends Handover Request Ack to T-AMF.

The target NG-RAN node indicates whether it supports of MBS to SMF in N2 SM information. Per the received N2 SM information, the SMF knows whether the target NG-RAN node supports 5G MBS anddetermines the delivery method, i.e. whether the 5GC shared MBS traffic delivery or 5GC Individual MBS traffic delivery is used for MBS data transferring.

12.	T-AMF to SMF: The AMF invokes Nsmf_PDUSession_UpdateSMContext request towards SMF, the message includes the N2 SM information received from the target NG-RAN.

13-14. Same as described in steps 3-4 of clause 7.2.3.2.

15. SMF to T-AMF: The SMF sends the Nsmf_PDUSession_UpdateSMContext response to the T-AMF.

Editor's note:	Details on data forwarding, if applicable, needs to wait for RAN WGs.

# * * * * Third change * * * *

#### 7.2.3.4	Xn/N2 based handover from non-MBS supporting NG-RAN node

When the UE has joined the multicast MBS session and the source NG-RAN node does not support 5G MBS, the 5GC Individual MBS traffic delivery method is used for the multicast MBS session data delivery. When the Xn/N2 based handover procedure is triggered, the UE is handed over to the target NG-RAN node per existing Xn /N2 based handover procedure defined in TS 23.502 [6].

The following applies for an Xn based handover from an NG-RAN node not supporting 5G MBS:

-	The source NG-RAN node requests the associated QoS Flow(s) in the associated PDU session to be handed over to the target NG-RAN node.

-	In the Path Switch Request message, the target NG-RAN node indicates whether it supports 5G MBS to the SMF in the N2 SM information.

-	After successful handover, if the target NG-RAN node supports MBS, the SMF triggers modification of the associated PDU Session  at the target NG-RAN node by including the multicast MBS session related information in N2 SM Information as described in step 7 of clause 7.2.1.3, which may trigger the target NG-RAN node to initiate establishment of shared delivery as described in clause 7.2.1.4..

-	The SMF changes the MBS session data delivery method from 5GC Individual MBS traffic delivery method to 5GC shared MBS traffic delivery method, and sends N4 Session modification message to UPF to configure UPF to not forward the received multicast MBS session data via the associated PDU Session. If there are no multicast MBS session data forwarding via the associated PDU session(s) is needed, the SMF may also release the shared tunnel between the UPF and MB-UPF.

The following applies for an N2 based handover from an NG-RAN node not supporting 5G MBS:

-	During handover preparation phase, the SMF includes the multicast MBS session related information in N2 SM Information as described in step 7 of clause 7.2.1.3 and sends it to the target NG-RAN. The target NG-RAN indicates whether it supports 5G MBS to SMF in N2 SM information. If the target NG-RAN node supports 5G MBS, the target NG-RAN node adds the UE into the MBS session context. If 5GC Shared MBS traffic delivery for the indicated multicast MBS session has not been established towards the target NG-RAN, the target NG-RAN uses the multicast MBS Session related information to allocate RAN resources to deliver the MBS data and initiates the shared delivery establishment towards the MB-SMF as described in clause 7.2.1.4.

-	Based on the received indication of supporting 5G MBS in N2 SM information from the target NG-RAN, the SMF determines the MBS data delivery method for the multicast MBS session. The SMF configures UPF to not forward the received multicast service data to the associated PDU Session via N4 Session modification message, i.e. SMF changes 5GC individual MBS traffic delivery to 5GC shared MBS traffic delivery. If there are no multicast MBS session data forwarding via the associated PDU session(s), the SMF may also release the shared tunnel between the UPF and MB-UPF.

# * * * * End of changes * * * *