SA WG2 Meeting #147e	S2-2107646R02

# October 18th – 22nd, 2021; Elbonia               	    		 	 				(revision of S2-2105398)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.548 | CR | 0025 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Procedure for local NEF selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The 23.501 CR (23.501 CR2856R2) agreed so far is not enough to specify the local NEF discovery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Overall procedural description of local NEF selection is provided in this CR: Figure 6.4.2.2 describes how an initially selected NEF may redirect the AF to a suitable local NEF. A companion CR to 23.502 updates services: |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 3204 |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R01: UDM based variant. 7646r01 Forms a tandem with 7647r01 |  |  |  |  |  |  |  |  |

FIRST CHANGE

6.4.2.2	Local NEF Discovery

As specified in TS 23.501 [2] clause 6.2.5.0, the NRF may be used by the AF to discover the L-NEF. To become discoverable, the L-NEF registers with an NRF deployed within the operator's domain where the AF resides.

The AF uses existing procedures as described in TS 23.502 [3], clause 4.17.4 to discover the L-NEF. If the AF only knows the NEF and it initiates a service operation towards this NEF, e.g. a Nnef_AFSessionWithQoS_Update_request procedure, the NEF may re-direct the request to an L-NEF as described in Figure 6.4.2.2. NEF may use NRF to find a suitable L-NEF for the re-direct and it may return the L-NEF IP address/FQDN to the AF in the response message.

Figure 6.4.2.2 describes how a NEF may redirect the AF to a suitable local NEF.

Figure 6.4.2.2-1: NEF redirecting an AF to a more suitable local NEF

0. The local NEF registers onto NRF (Nnrf_NFManagement_NFRegister) with its locality and the list of the S-NSSAI it supports. This registration may also containthe parameters referred to clause 5.2.7.2.2 in TS 23.502[3].

1.	PDU Session Establishment (as defined in TS 23.502 [3] clause 4.3.2.2.1 or clause 4.3.2.2.2). During the SM Policy Association Establishment procedure (e.g. at step 7 of Figure 4.3.2.2.1-1), when the PCF registers to the BSF (Nbsf_Management_Register), the PCF provides in Nbsf_Management_Register following information:  UE address(es), SUPI, GPSI, DNN, S-NSSAI,  .

2.	The AF sets up an AF session with required QoS procedure as defined in clause 4.15.6.6 of TS 23.502 [3] further providing, as specified in TS 23.501 [2] clause 6.3.14, an indication or a new event for local/edge, that the request targets a local NEF. The request is addressed to an initial NEF.

NOTE: The initial NEF may be a central NEF or a local NEF that may have become unsuitable due to UE mobility

A new event for local/edge, e.g.: QoS monitoring for edge/local or specific edge event. The newly designed edge event will cause the NEF to select a local NEF before establish the AF session to serve AF.

The indication that the request targets a local NEF may trigger the initial NEF to look for a (more) suitable NEF per the rest of this procedure. Also, this indication is used to identify that whether network provides the service (e.g.: QoS monitoring) to local or edge

Based on the UE address (i.e. IP address or MAC address) provided in the AF request the initial NEF needs to determine the locality of the Local PSA serving the related PDU Session; for this purpose the initial NEF first determines the target (DNN, S-NSSAI): if the target (DNN, S-NSSAI) is not provided in the AF request, the initial NEF determines the target (DNN, S-NSSAI) based on the identity of the requesting AF.

3.	The NEF can buffer or store the local NEF address, which related to the certain UE location, AF location, PSA location and certain PDU session. If another AF requests for a QoS monitoring for local or edge event towards NEF or has the indication, which the AF location is the same as the stored in NEF, the NEF can directly redirect the request towards local NEF in step 9 without any parameter gathering or local NEF discovery in NRF.

4.	The initial NEF issues a Nbsf_Management discovery procedure: the initial NEF provides as input the UE address and the target (DNN, S-NSSAI) and gets as a result the SUPI of the UE  .

5	The NEF issues a Nudm_UECM_Get service operation to get the SMF ID of the SMF serving the PDU Session providing the SUPI of the UE and the (DNN, S-NNSSAI

6.	The initial NEF initiates Nsmf_EventExposure_Subscribe to the SMF indicated by the UDM in step 4 : the initial NEF provides as input the SUPI and the target (DNN, S-NSSAI) and gets as output the locality and the instance ID of the Local PSA (L-PSA) and the DNAI;

7.	The initial NEF issues a Nnrf_NFDiscovery_Request request targeting a NEF and providing as discovery parameter:

the target S-NSSAI, the locality of the L-PSA UPF and DNAI

8.	The NRF answers (Nnrf_NFDiscovery_Request response) with suitable target local NEF(s).

9.	When the own initial NEF identity is not provided in the NRF response, the initial NEF determines it is not the most suitable NEF to handle the AF request and redirects the AF towards a (more suitable) local NEF.

NEXT CHANGE (2)

NEXT CHANGE (3)

NEXT CHANGE (4)

NEXT CHANGE (5)

END OF CHANGES