SA WG2 Meeting #S2-147E	S2-2107742

18 October – 22 October 2021, Electronic, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0042 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Modification on the mobility proceudre |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Tencent |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the N2 HO, the SMF has got the NG-RAN MBS capabilty in the HO request ACK. And if multicast transport is used, the BM-SMF shall allocate the multicast transport address. In the Xn HO for inactive MBS session, there is EN for RAN confirmation. It proposes to remove it. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In the N2 HO, the SMF based on the received NG-RAN MBS capability in the HO Request ACK. If multicast transport is used, the BM-SMF shall allocate the multicast transport address. Removing the EN in the clause 7.2.3.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear in the N2 HO, which step is used to SMF obtaining the NG-RAN capability. The EN in clause 7.2.3.6 is not resolved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.3, 7.2.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.2.3.3	N2 based handover from MBS supporting NG-RAN node

This clause describes the N2 based handover with MBS traffic delivered to the UE at the source NG-RAN node supporting 5G MBS.

Figure 7.2.3.3-1: N2 based handover with MBS Session

The following additions apply compared to clause 4.9.1.3 of TS 23.502 [6]):

2.	Source NG-RAN to S-AMF: Handover Required (RAN container (MBS Session information, associated PDU session information, associated QoS flow information and corresponding multicast QoS flow information)).

4.	SMF to T-AMF: The T-AMF is provided with associated PDU Session information and the MBS session related information.

5.	T-AMF to Target NG-RAN: The Target NG-RAN prepares the radio resource based on the received information:

-	If the target NG-RAN does not support 5G MBS, the MBS Session information is not used. The target NG-RAN uses the associated mapping PDU Session information to allocate resource to deliver MBS data. The MBS data are transmitted via the associated QoS flows within the associated PDU Session.

-	If the target NG-RAN supports 5G MBS, the target NG-RAN uses the MBS Session information to allocate resource to deliver the MBS data.

6-10.	If Target NG-RAN supports 5G MBS and the MBS delivery for the indicated MBS Session has not yet been established towards target NG-RAN, the target NG-RAN initiates the shared delivery establishment towards the MB-SMF. Details is defined in the clause 7.2.1.4.

Target NG-RAN to T-AMF: The target NG-RAN sends Handover Request Ack to T-AMF.

The target NG-RAN node indicates whether it supports of 5G MBS to SMF in N2 SM information. Per the received N2 SM information, the SMF knows whether the target NG-RAN node supports 5G MBS and determines the delivery method, i.e. whether the 5GC shared MBS traffic delivery is used for MBS data transferring.

17.	T-AMF to SMF: The AMF invokes Nsmf_PDUSession_UpdateSMContext request towards SMF, the message includes the received N2 SM message in step 16.

Based on the received target NG-RAN MBS capability in step 7, the SMF can differentiate two cases:

Case A)	The Target NG-RAN supports 5G MBS. Step 18 applies and steps 19~23 are skipped.

18.	SMF to UPF (PSA): The SMF invokes N4 Session Modification procedure with the UPF (PSA) only for unicast PDU Session. The SMF instructs the UPF to send the end marker packet towards the source NG-RAN and to send subsequent packets towards the target NG-RAN within the unicast PDU Session.

Case B)	The target NG-RAN does not support 5G MBS. If the UPF (PSA) is not yet configured to forward multicast data via unicast, steps 19 to 23 apply.

19	SMF to UPF: The SMF may invokes N4 Session Modification procedure with UPF (PSA), the SMF instructs the UPF (PSA) to forward the multicast data received from the MB-UPF via the associated QoS Flow(s) within the associated PDU Session (i.e., 5GC Individual MBS traffic delivery method will be used). The SMF provides the mapping information between the multicast QFI and the corresponding mapped associated QFI to the UPF (PSA), the UPF (PSA) forwards the multicast data via the PDU session based on the mapping information. If the delivery tunnel for the MBS session from MB-SMF to UPF is not established yet, the SMF instructs the UPF (PSA) to allocate a tunnel endpoint for the reception of multicast data from the MB-UPF.

If delivery of the multicast data from MB-UPF to UPF needs to be configured, steps 20 apply.

20. Establishment of resources for 5G Individual MBS delivery as specified in 7.2.1.3.

21. SMF to T-AMF: The SMF sends Nsmf_PDUSession_UpdateSMContext Response to target AMF.

Editor's note:	Details on data forwarding, if applicable, needs to wait for RAN WGs.

Next CHANGE

#### 7.2.3.6	Xn/N2 based handover for inactive MBS session

If the MBS session in "Inactive" state, comparing the handover procedure for the MBS session in "Active" state, the following additional procedures apply:

-	For Xn based handover, the information that MBS session is inactive is provided from the source RAN node towards the target RAN node.

-	For N2 based handover, the information that MBS session is inactive is provided from SMF towards the target RAN node.

-	For the MBS supporting NG-RAN node, the target NG-RAN establishes the shared tunnel with the MB-UPF as usual. However, as the MBS session is inactive state, the NG-RAN node will not allocate related radio resource.

-	After a handover to a not supporting MBS target RAN node, the SMF removes the associated QoS flow(s) information.

Editor's Notes: Details on data forwarding, RAN confirmation is required.

NOTE: Whether the associated QoS Flow(s) are removed from UE, NG-RAN, or only resource in NG-RAN is removed is up to implementation.

END OF CHANGES