SA WG2 Meeting #S2-147E	S2-2107743r01

18 October – 22 October 2021, Electronic, Elbonia	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0043 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the MBS service actication/deactivation/update and related service operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There are twn ENs in service deactivation and service update. The MBSSession_statussubscribe/unsubscribe/notify service operation are defined but not used. It may be dulpicated of MBSSession_ContextStatus. The parameters, i.e. MBS session ID, subscription correlation ID in MBSSession_ContextStatusSubscribe/ContextStatusSubscribe/StatusNotify service operation are not aligned. There are two alt: - Always use MBS session ID; - In the subscribe using the MBS session ID, in the unsubscribe and notify, using the subscription correlation ID Subscription correlation ID is mainly related to event exposure but not for StatusNotify, therefore it is proposed to use MBS Session ID which is sufficient. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing the EN in service deactivation and service update. Removing the MBSSession_statussubscribe/unsubscribe/notify service operation. Keep alignment in MBS session ID, subscription correlation ID in MBSSession_ContextStatusSubscribe/ContextStatusSubscribe/StatusNotify service operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The ENs are not resolved. The MBSSession_statussubscribe/unsubscribe/notify service operation are dulplicated. The parameter in MBSSession_ContextStatusSubscribe/ ContextStatusSubscribe/StatusNotify service operation are not align. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.5.2, 7.2.5.3, 9.1.3.3, 9.1.3.4, 9.1.3.5, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 7.2.5.2	MBS session activation procedure

The following can trigger the MBS session activation procedure:

-	AF requests MB-SMF to activate the MBS session;

-	MB-UPF receives the multicast data and notifies MB-SMF.

Figure 7.2.5.2-1: MBS session activation procedure

In this procedure, steps 2 to 10 and steps 11 to 14 can be executed in parallel.

1.	The procedure may be triggered by the following events:

-	When MB-UPF receives downlink data for a MBS session, based on the instruction from MB-SMF (as described in clause 7.2.5.3), MB-UPF sends N4mb Notification (N4 Session ID) to the MB-SMF for indicating the arrival of DL MBS data.

-	AF sends MBS Activation request (TMGI) to the MB-SMF directly or via NEF.

2.	MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify (MBS session ID) to SMF(s).

Based on the received MBS session ID, SMF set the related MBS session state as "Active" state and finds the list of UEs that joined the MBS session identified by the related TMGI. If SMF determines the user plane of the associated PDU session(s) of the UE(s) with respect to TMGI are activated already, steps 3-7 will be skipped for those UE.

3.	SMF sends Namf_MT_EnableGroupReachability Request to AMF, including (UE list, TMGI, associated information). When later UE is reachable, the associated information is used by the AMF to identify and notify the related SMF to activate the associated PDU Session.

After receiving the request, for each UE in the list, the AMF determines CM state of the UE: see steps 4 - 7.

4.	[Optional] If the UE involved in the MBS Session is in CM-CONNECTED state, the AMF responds the list of the UE involved in the MBS Session and in CM-CONNECTED state, using Namf_MT_EnableGroupReachability Response (UE list). Step 5-6 will not be executed for that UEs in the list.

5.	[Optional] If AMF determines that there are any UEs in CM-IDLE state and involved in the MBS Session, and AMF figures out the paging area considering all the UE(s), which need be paged. The AMF sends a paging request message to the NG-RAN node(s) belonging to this Paging Area with the TMGI as the identifier to be paged if the related NG-RAN node(s) support the MBS session. If the NG-RAN node(s) does not support MBS, the AMF sends Paging message to the NG-RAN node(s) per UE without using the MBS Session ID as described in step 4b in clause 4.2.3.3 of TS 23.502 [6].

NOTE 1:	The details of the paging are specified by the RAN WGs.

6.	The UE(s) in CM-IDLE state sends Service Request message to AMF, see clause 4.2.3 of TS 23.502 [6].

7.	After receiving the Service Request sent by the UE(s), based on the received associated information in step 3 the AMF identifies and notifies the related SMF of the UE(s), which are reachable now.

8.	For those UE(s), which the associated PDU Session is activated before step 2 and if the associated QoS flow information has been provided to RAN before, step 8-9 are skipped.

After receiving MBS_Session_Notification Response message, SMF determines the related UE is in CM- CONNECTED state and sends Namf_Communication_N1N2MessageTransfer (N2 SM message (MBS Session identifier, associated QoS profiles, mapping information between the unicast QoS flow and multicast QoS flow)) to AMF for the UE which is identified in step 3. The mapping information between the unicast QoS flow and multicast QoS flow is included to support the 5GC Individual MBS traffic delivery.

NOTE 2:	N2 SM message is included in which message depends on what associated information is included in step 3.

9.	AMF sends N2 request message (N2 SM message (MBS Session identifier, associated QoS profiles, mapping information between the unicast QoS flow and multicast QoS flow) to the RAN node.

10a.	If the shared tunnel has not been established before, the shared tunnel is established at this step, as defined in clause 7.2.1.4. The NG-RAN configures UE with RRC messages if needed.

10b.	NG-RAN responses to SMF as steps 9 to 12 defined in clause 7.2.1.3. If 5GC Individual MBS traffic delivery is used, the SMF configures the UPF for individual delivery and if necessary requests the MB-SMF to configure the MB-UPF to send multicast data to the UPF.

11.	If the MB-SMF finds there are shared tunnel established, step11-15 are performed. The MB-SMF sends Namf_MBSCommunication_N2MessageTransfer Request (Activation, TMGI) to the AMF for those NG-RAN nodes, which have shared tunnel with MB-UPF. This step can be performed in parallel with step 2.

NOTE 3:	The messages in step 11 and 12 is MBS-specific messages and it is possible that the AMF(s) in step 10 are not associate to any UEs involved in the MBS Session.

12.	AMF sends NGAP activation message (TMGI) to the RAN nodes.

13.	RAN nodes responses to AMF by NGAP activation response message. Steps 8 of clause 7.2.1.3 are performed to establish RAN resources to transmit multicast session data to the UE(s). The NG-RAN shall not release the radio connection of a UE that has joined into the multicast session only because no unicast traffic is received for the UE.

14.	AMF to MB-SMF: Namf_MBSCommunication_N2MessageTransfer Response ().

15.	MB-SMF may send N4mb Session Modification Request to MB-UPF. MB-UPF responses to MB-SMF with N4mb Session Modification Response acknowledging the MB-SMF request. See clause 4.4 for more details.

Next CHANGE

#### 7.2.5.3	MBS session deactivation procedure

Figure 7.2.5.3-1: MBS session deactivation procedure.

In this procedure, steps 3 to 4 and steps 5 to 9 are executed in parallel.

1.	The procedure may be triggered by the following events:

-	When MB-UPF detects there is no data receives for the MBS Session, MB-UPF sends MB-N4 Notification (N4 Session ID) to the MB-SMF for deactivating the MBS session.

-	AF sends MBS Deactivation request (TMGI) to the MB-SMF directly or via NEF.

2.	MB-SMF send N4mb Session Modification Request to MB-UPF (TMGI, indication for buffering and notifying MB-SMF). See clause 4.4 of TS 23.502 [6] for more details.

MB-UPF to MB-SMF: N4mb Session Modification Response acknowledging the MB-SMF request.

The MB-SMF also indicate to MB-UPF to remove the shared tunnel between UPF and MB-UPF, which is used for Individual MBS traffic delivery.

3.	The MB-SMF sends Nmbsmf_MBSSsession ContextStatusNotify request (MBS session ID) to the SMFs.

Based on the received MBS session ID, the SMF set the indicated MBS session state to "Inactive" state.

‐	If SMF find there exist UE(s) that joined the indicated MBS session and use 5GC Individual MBS traffic delivery, step 4 is performed for those UE(s).

‐	If SMF find there are no UE(s) that joined the indicated MBS session and use 5GC Individual MBS traffic delivery, no further operation for SMF is required.

4.	For those UE(s) which the 5GC individual delivery is used, step 3a and steps 4-8 in clause 4.3.3.2 of TS 23.502 [6] are performed to remove the associated QoS flow(s).

NOTE:	Whether the associated QoS Flow(s) are removed from UE, NG-RAN, or only resource in NG-RAN is removed is up to implementation.

5.	If the MB-SMF finds there are shared tunnel established, the MB-SMF sends Namf_MBSCommunication_N2MessageTransfer Request (Deactivation, TMGI) to the AMFs.

6.	The AMF sends NGAP deactivation request message (TMGI) to the RAN nodes.

7.	NG-RAN keeps the MBS session context and N3mb tunnel for the MBS session.

If no UE(s) is in the MBS session context (e.g., due to UE becomes CM-IDLE state), the NG-RAN triggers release of the shared delivery as described in clause 7.2.2.4.

8.	NG-RAN acknowledges the NGAP deactivation Response message.

9.	The AMF invokes Namf_MBSCommunication_N2MessageTransfer Response to acknowledge the service for MB-SMF.

When the MBS session is "Inactive" state and handover procedure is triggered, it is defined in clause 7.2.3.6.

Next CHANGE

### 7.2.7	Multicast session service area update procedure

9. Determine affected UEs

Figure 7.2.7-1: MBS service area update procedure for multicast

This procedure is triggered by the MB-SMF receiving the updated service area for MBS, see clauses 7.1.1.6 and 7.1.1.7.

The AF providing the updated service area may also inform UEs at application level about the new service area via a service announcement. If a UE is located in a radio cell which was previously outside the service area and is now inside the updated service area, the UE may join the multicast service.

Steps 3 to 7 are optional and can be omitted. However, they allow to terminate data transmission towards cells served by the RAN node previously in the multicast service area and now outside the updated multicast service area as quickly as possible.

3.	The MB-SMF may send Namf_MBSCommunication_N2MessageTransfer request (N2 SM message (TMGI, MBS service area, [Area Session ID]) to the AMF(s) handling NG-RAN nodes within the multicast session (and for multicast session with location dependent content within the same location area). The MB-SMF may alternatively inform only NG-RAN nodes handling the multicast session but where the MBS service area was restricted in such a way that some or all cells served by the RAN node previously in the multicast service area are now outside the updated MBS service area.

4.	The involved AMF forwards the N2 SM information received from MB-SMF to the RAN nodes via NGAP Session update Request (N2 SM message (TMGI, MBS service area, [Area Session ID])) message.

5.	NG-RAN node updates the MBS session context with the new MBS service area. If an NG-RAN node is informed via MB-SMF about an updated MBS service area of the multicast session and some cells it serves previously within the service area are now outside the updated service area it ends transmission of the related multicast data towards those cells. If the RAN node no longer serves any cells within the service area with users inside the multicast session, it requests (via AMF) from the MB-SMF that the distribution of the multicast data towards the NG-RAN node is terminated, see clause 7.2.2.4.

6.	The NG-RAN acknowledges NGAP Session update Request by sending an NGAP Session update Response message to the AMF.

7.	The AMF sends Nmbsmf_ MBSSession_ContextUpdate to the MB-SMF.

8.	MB-SMF sends Nmbsmf_MBSSession_ContextStatusNotify (subscription correlation info or MBS session ID, Event ID, MBS service area, [Area Session ID]) to each SMF with a related subscription. It is assumed that each SMF serving UEs within the multicast session performs a related subscription for the multicast session at the MB-SMF when the first such UE joins.

9.	SMF determines the affected UEs it serves based on the subscription correlation identifying the multicast session and Area Session ID (if exists) included in the message 8.

The subsequent steps 10 to 12 are executed separately for each affected UE. Signalling related to steps 11 and 12 may be combined in the same messages between SMF and AMF, and between AMF and NG-RAN, e.g. as NGAP PDU session modify request/response. In an alternative implementation, step 10 is executed for a list of UEs and only step 11 and 12 are executed separately for each affected UE.

10.	The SMF may query AMF or NG-RAN for the current location of the UE (e.g. cell ID and/or TAI) to determine whether the UE is within the updated service area.

11.	For a multicast local MBS service, the SMF informs each UE involved in the multicast session that it serves about the changed service area by providing information about the multicast session ID and updated service area in a N1 container. For a UE previously inside the MBS service area but now outside the updated MBS service area of the multicast session, the SMF may also signal towards the UE that it has been removed from the multicast session to trigger UE resources for the reception of multicast data to be released and/or a related notification towards the user.

12.	The SMF also updates the PDU session resources associated to the multicast session with the new MBS service area.

Next CHANGE

#### 9.1.3.3	Nmbsmf_ MBSSession_ContextStatusSubscribe service operation

Service operation name: Nmbsmf_ MBSSession_ContextStatusSubscribe

Description: Service Consumer NF can use this service operation to request information (e.g. QoS information) about a multicast session and to subscribe to notification of events about the multicast session context.

Inputs, Required: Multicast Session ID, notification target address, Events ID(s).

Inputs, Optional: immediate reporting indication.

Outputs, Required: -.

Outputs, Optional: If immediate reporting indication is set: Event information (e.g. QoS information for multicast session, multicast session status (activated, deactivated), multicast session service area for local multicast service), Start time of multicast session, if consumer is SMF: indication that the MBS session allows any UE to join.

#### 9.1.3.4	Nmbsmf_MBSSession_ContextStatusNotify service operation

Service operation name: Nmbsmf_ MBSSession_ContextStatusNotify

Description: This service operation, which is applicable to multicast MBS session, is used by the MB-SMF to notify its consumers about events of an MBS Session.

Inputs, Required: MBS Session ID, Event ID, Notification Correlation Information.

Inputs, Optional: Event information (QoS information of MBS Session, MB service area).

Outputs, Required: Operation execution result indication.

Outputs, Optional: Cause.

#### 9.1.3.5	Nmbsmf_ MBSSession _ ContextStatusUnsubscribe service operation

Service operation name: Nmbsmf_MBSSession_ContextStatusUnsubscribe

Description: This service operation, which is applicable to multicast MBS session, is used by the consumer to unsubscribe to notifications about MBS context events.

Inputs, Required: MBS session ID.

Inputs, Optional: None.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

END OF CHANGES