3GPP TSG-SA WG2 Meeting #147E (e-meeting)	S2-2107847

# October 18 – 22, 2021, Elbonia										    (revision of S2-2107159r05)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3251 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Authentication and Subscription information checking for Disaster Roaming service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics, KT Corp., LG Uplus, SK Telecom, Ericsson, vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MINT |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is still open issue for Disaster Roaming in the existing specification as follows; Editor's note: It is FFS if Disaster Roaming indication needs to be provided from AMF to other NFs (e.g. SMF, AUSF, UDM) During the authentication and subscription information checking, - Option 1: “Disaster Roaming indication” is transferred from AMF to AUSF/UDM - Option 2: The configuration via OAM based on operator policy and the request by the government agencies when the disaster roaming occurs (No additional signaling) |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to specify both options for operator’s flexible deployment and preference |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The authentication of UE and subscription information checking cannot be performed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.40.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.40.4	Registration for Disaster Roaming service

For a UE to receive Disaster Roaming service from a PLMN providing Disaster Roaming service, the UE sends a NAS Registration Request message with Registration Type value "Disaster Roaming":

-	When the AMF in the PLMN providing Disaster Roaming service receives a NAS Registration Request with Registration Type set to "Disaster Roaming",

-	the AMF controls if the UE is allowed to access Disaster Roaming service in the area with Disaster Condition as specified in TS 23.502 [3] clause 4.2.2.2.2;

-	the AMF may provide the Disaster Roaming service indication to AUSF and UDM as specified in TS 23.502 [3] clause 4.2.2.2.2, and TS 33.501 [29].

NOTE 1:	The AUSF and the UDM are configured with Disaster Condition via OAM based on operator policy and the request by the government agencies. Based on this local configuration, the AUSF can execute authentication of the UE, and the UDM can provides the subscription data for a Disaster Roaming service to the AMF.

NOTE 2:	The AMF is recommended to provide to AUSF/UDMs if the HPLMN is from a different country. For AUSF/UDMs of HPLMNs of the same country the detection can be based on home operator policy and local configuration.

Editor's note: The details of authentication are determined by SA WG3, including whether the Disaster Roaming service indication is needed. Disaster Roaming service indication may be removed, e.g. based on the feedback.

To support the Disaster Roaming service, the PLMN providing Disaster Roaming service is configured to support communication with the network entities in the HPLMN of the UE, i.e. configurations related to roaming interfaces for communication between serving PLMN and HPLMN shall be deployed in the affected entities. This communication between the PLMNs need only be enabled during the Disaster Condition.

The Disaster Roaming service is limited to the impacted geographic area with Disaster Condition. The NG-RAN nodes and AMF in the PLMN providing Disaster Roaming service are configured with the area information, i.e. a list of TAIs which can be formulated by the PLMN providing the Disaster Roaming service based on the geographic area with Disaster Condition in the other PLMN(s).

The AMF in the PLMN providing Disaster Roaming service provides the mobility restriction list to the NG-RAN as specified in clause 5.3.4.1.1 considering the area with Disaster Condition, and also indicating that EPC is not an allowed core network.

* * * * End of Changes * * * *