3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2108007

# Elbonia, October 18 – 22, 2021	(revision of S2-2107551r01)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.247 | CR | 0029 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates on cases that SMF rejects UE join request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5MBS |  |  |  |  |  | Date: |  |  | 2021-10-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1.It is specified in clause 7.2.1.3 that “If no MB-SMF is assigned for the MBS session ID, the SMF may select an MB-SMF and request it configure the multicast session or the SMF may reject the join request.” It is clarified that in that case SMF select the MB-SMF based on local configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | If no MB-SMF can be assigned to the MBS session ID (according to the MB-SMF profile registered in configuration procedure or preconfigured by the operator), the NRF may provide empty profile to SMF to indicate that no MB-SMF is assigned for the MBS session ID. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The scenario that “If no MB-SMF is assigned for the MBS session ID” for preconfigured MBS session is not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 7.1.2	MB-SMF discovery and selection for multicast/broadcast session

To facilitate the MB-SMF discovery/selection for one multicast/broadcast MBS session, the following mechanism is used:

-	The MB-SMF registers its capability related to multicast/broadcast session management (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s), TMGI range, service area) as part of its profile to the NRF by invoking Nnrf_NFManagement_NFRegister. In addition, when an multicast session is configured, and the MBS session ID is not yet included in the MB-SMF profile, the MB-SMF updates its profile towards the NRF with the MB Session ID (i.e. TMGI or source specific IP multicast address).

NOTE:	The operator can preconfigure MB-SMF for specific source IP multicast address range or TMGI range.

-	When the UE joins the multicast MBS session via PDU session modification procedures, the SMF serving the PDU session invokes the Nnrf_NFDiscovery_Request Request including the MBS session ID for multicast provided by the UE and optionally other information, i.e. the S-NSSAI and the associated NSI ID (if available), DNN, etc., to query the NRF for MB-SMF information. Based on the MBS session ID and other information for query, the NRF decides whether an MB-SMF serving the MBS session exists (i.e. the NRF decides whether the requested MB Session ID is in the profile of a MB-SMF, which is configured as defined in clause 7.1.1.2 or 7.1.1.3 or preconfigured by the operator). If so, the NRF provides in Nnrf_NFDiscovery_Request Response the information of the MB-SMF currently serving the MBS session. The SMF serving the PDU session selects the MB-SMF currently serving the multicast MBS session, based on the MB-SMF information provided by the NRF. For local MBS services, the SMF takes MB-SMF service area, UE location into account when selecting the MB-SMF. If no MB-SMF serving the multicast session exists, the NRF provides empty MB-SMF profile to the SMF.

-	When the multicast MBS session context is deleted from the MB-SMF, e.g. due to MBS session release, the MB-SMF updates its profile towards the NRF, i.e., removing the MBS session ID which is no longer served by the MB-SMF.

-	During MBS session information provisioning procedures defined in clause 7.1.1.2, unless the MB-SMF information is available by other means, e.g. locally configured in the NEF/MBSF/AF, the NEF/MBSF/AF queries the NRF with information of the multicast/broadcast session (e.g. S-NSSAI(s) and the associated NSI ID(s) (if available), DNN(s)), and selects the MB-SMF(s) based on the MB-SMF information provided by the NRF. For local MBS services, the NEF/MBSF/AF takes MB-SMF service area into account when selecting the MB-SMF(s).

# * * * * End of changes * * * *