3GPP TSG-WG SA2 Meeting #147E e-meeting 	S2-2108136

# Elbonia, October 18 – 22, 2021	(revision of S2-2107355)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0660 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Policy for Handling of PMIC exchange with UE(s) in idle mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2021-10-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following EN was added during the During the SA2#145E. Editor's note: Handling of PMIC exchange with UE(s) in idle mode is FFS. Normally, PMIC is delivered via NAS to a UE in idle mode with paging. However, all the UEs with a DNN/S-NSSAI may be affected by a Synchronization Activation/Modification/Deactivation request from an AF/TSCTSF. It may cause delays and congestions to the other UEs in the same paging zone, as all the paging resources of the paging zone can be comsumed by the Synchronization Activation/Modification/Deactivation request from the AF/TSCTSF. During tha SA2#146E, the following is added to 5.23 of 23.501. - If the originating NF does not require immediate delivery, it may indicate that the AMF is allowed to use ATC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - TSCTSF may indicate that the AMF is allowed to use ATC when it sends PMIC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec will have ambiguity handling of PMIC exchange with UE(s) in idle mode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.23 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** BEGIN 1st CHANGE *****

6.1.3.23	Support of integration with Time Sensitive Networking

Time Sensitive Networking (TSN) support is defined in TS 23.501 [2], where the 5GS represents logical TSN bridge(s) based on the defined granularity model. The TSN AF and PCF interact to perform QoS mapping as described in clause 5.28.4 of TS 23.501 [2].

The PCF provides the following parameters to the TSN AF or TSCTSF:

-	5GS user-plane Node information:

-	5GS Bridge ID;

-	UE-DS-TT Residence time;

-	port number of the Ethernet port of DS-TT;

-	MAC address of the Ethernet port of DS-TT (i.e. DS-TT port MAC address).

-	Port Management Information Container and the related port number.

-	User plane node Management Information Container.

The TSN AF may use this information to construct IEEE 802.1 managed objects, to interwork with IEEE 802.1 TSN networks, as described in TS 23.501 [2] and TS 23.502 [3].

The TSN AF or TSCTSF may use the PTP Port state of NW-TT and DS-TT in the Port/User plane node Management Information Container to determine the Port Pairs that will be used for (g)PTP delivery. Based on this the TSN AF or TSCTSF may request appropriate QoS treatment for the (g)PTP flows from PCF.

The TSN AF or TSCTSF request related to TSN configuration or gPTP flow delay requirement is sent on the AF session associated with the DS-TT port MAC address. The TSN AF or TSCTSF decides the TSN QoS information (i.e. priority, delay, maximum TSC Burst Size and Maximum Flow Bitrate) and TSC Assistance Container based on the received configuration information of 5GS Bridge from the CNC as defined in clause 5.28.2 of TS 23.501 [2], the bridge delay information at the TSN AF and the UE-DS-TT Residence time.

The TSN AF or TSCTSF provides the Flow Descriptions (including Ethernet Packet Filters), TSC Assistance Container, and the related QoS information to the PCF by setting up an AF session with required QoS as described in clause 6.1.3.22. In addition, the TSN AF or TSCTSF may provide the following parameters to the PCF:

-	Port Management Information Container and related Port number as applicable;

-	User plane node Management Information Container.

The PCF assigns the ARP to a value preconfigured for TSN services. The SMF binds the PCC Rule to a QoS Flow as defined in clause 6.1.3.2.4.

NOTE:	TSC burst size can represent the maximum burst size of the TSN streams that have been aggregated.

***** END OF CHANGES *****