3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-220xxxx

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue on enhancements of 5G VN group communication

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing a Key Issue for FS_GMEC based on the Work Task 2.1.

1. Introduction/Discussion

The Work Task 2.1 of FS_GMEC is as following:

-	Work Task 2.1: Support group communication for a 5G VN which supports multiple SMFs, including support of SMF redundancy for reliability of the 5G VN group communication

In Release 16, a dedicated SMF is responsible for all the PDU Sessions for communication of a certain 5G VN. When a 5GVN spans over a large area or different group member accesses to the 5GVN at different places, multiple SMFs may be involved to serve a 5GVN due to limited SMF service area.

When multiple SMFs are involved to serve a 5GVN, multiple UPFs controlled by these different SMFs may be involved to enable communication of the 5GVN. In this case, how to establish the N19 tunnels between these UPFs (different topology may be established) is not clear. After that, how to update the configuration on these UPFs due to UE mobility also needs further study.

Based on the above information, it is proposed to providing a Key Issue for FS_GMEC based on the Work Task 2.1.

2. Text Proposal

It is proposed to capture the following changes vs. 23.700-74 V0.0.0.

# * * * * First change * * * *

5	Key Issues

5.X	Key Issue #X: Enhancements of 5G VN group communication

5.X.1	Description

In Release 16, a dedicated SMF is responsible for all the PDU Sessions for group communication of a certain 5G VN. When a 5GVN spans over a large area or different UE group member accesses to the 5GVN at different places, multiple SMFs may be involved to serve a 5GVN due to limited SMF service area.

When multiple SMFs are involved to serve a 5GVN, multiple UPFs controlled by these SMFs may be involved to enable group communication of the 5GVN. A SMF needs to be aware of other SMFs, and 5GVN related UPF information managed by other SMFs, as well as the UE group members connected to the related UPF under other SMFs. The SMF also needs to update the configuration on the UPF when another UE group member starts/stops to access to the 5GVN.

On the other hand, when the 5GVN group is served by multiple UPFs, different topology of the UPF (e.g., full-meshed or other type) may be established, how to manage the N19 tunnels between these UPFs is not clear.

This key issue will study how to support 5G VN group communication when a 5GVN is served by multiple SMFs, which includes:

-	What architecture enhancements are needed and how to support communication between SMFs (e.g. whether a group session management function or enhancement to existing network function is needed);

-	How to manage the N19 tunnels between UPFs served by different SMFs;

-	How to configure or update the UPF(s) to enable N19-based forwarding between UPFs controlled by different SMFs, e.g., due to UE mobility;

# * * * * End of changes * * * *