3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-220xxxx

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	Huawei, HiSilicon

Title:	Key Issue on simultaneously sending data to different groups with different QoS

Document for:	Approval

Agenda Item:	9.2

Work Item / Release:	FS_GMEC / Rel-18

Abstract: Providing a Key Issue for FS_GMEC based on the Work Task 3.

1. Introduction/Discussion

The Work Task 2.1 of FS_GMEC is as following:

Work Task 3: Whether additional mechanism or enhancement is needed and how to support group communication allowing UE to simultaneously send data to different groups, where each group has a different QoS policy (requirement regarding 5SEI as indicated in clause 6.13.2 of TS 22.261)

This WT aims to study how to satisfy the following requirements from [TS 22.261, clause 6.13.2]:

-	The 5G system shall allow a UE to request a communication service to simultaneously send data to different groups of UEs at the same time.

-	The 5G system shall allow different QoS policy for each group the UE communicates with.

The original requirement is from clause 5.13 of TR 22.867 as following:

For a PMU deployed at a given bus in the grid, different types of data are generated to feed different applications of divergent criticality (such as monitoring, control, protection, state estimation, etc.). Also, the same data can be needed by different destinations requiring different QoS treatment. Because of that, these different streams of data is needed by different destinations with different QoS. Additionally, the algorithms a TSO or DSO decides to use further influences the difference in criticality (QoS requirements in other words) of various streams of data this PMU generates.

The basic scenario is summarized as that a UE with one dedicated application can belong to multiple groups, this application can generate the data and replicates multiple copies of the data, and then each copy is sent to a different destination that corresponds to a different group. The transmission of the copy within the different group needs different QoS because of the different purpose of data usage for the group.

Based on the above information, it is proposed to providing a Key Issue for FS_GMEC based on the Work Task3.

2. Text Proposal

It is proposed to capture the following changes vs. TR 23.700-74 V0.0.0.

# * * * * First change * * * *

5.X	Key Issue #X: Sending same data simultaneously to different groups with different QoS

5.X.1	Description

This key issue aims to study how to satisfy the following requirements from TS 22.261 [6], clause 6.13.2:

-	The 5G system shall allow a UE to request a communication service to simultaneously send data to different groups of UEs at the same time.

-	The 5G system shall allow different QoS policy for each group the UE communicates with.

This key issue will study whether additional mechanism or enhancement is needed and how to support group communication allowing UE to simultaneously send data to different groups, where each group has a different QoS policy.

# * * * * End of changes * * * *