3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200255r06

Elbonia, February 14 – 25, 2022

Title:	[DRAFT] Reply LS on Clarification on the change of Target NSSAI

Response to:	LS (S2-2200051/C3-216445) on Clarification on the change of Target NSSAI from CT3

Release:	Rel-17

Work Item:	eNS_Ph2

Source:	SA2

To:	CT3

Cc:

Contact Person:

Name:	Haiyang,Sun

Tel. Number:

E-mail Address:	sunhaiyang3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 would like to thank CT3 for their LS on Clarification on the change of Target NSSAI.

Specific to the CT3 questions in C3-216445/S2-2200051, SA2 would like to provide the following feedback:

Question: Can SA2 confirm that the target NSSAI may be generated more than once during the lifetime of the UE registration (i.e. the change of target NSSAI may happen)? For example, if the subscribed NSSAI or requested NSSAI changes, will the Target NSSAI be changed accordingly? Can SA2 indicate if there are other situations that require the trigger to be invoked.

SA2 Answer: SA2 confirmed that Target NSSAI may be changed when the UE initiates a new UE registration. SA2 didn’t find any other situation that can trigger the change of Target NSSAI. When the new Target NSSAI is generated during the UE registration and if the AMF does not select a new PCF the AMF uses the existing AM Policy association to obtain the RFSP for a Target NSSAI.

2. Actions:

To CT3 group.

ACTION: 	SA2 asks CT3 group to take the above feedback into account.

3. Date of Next TSG SA WG2 Meetings:

See https://portal.3gpp.org/Home.aspx?tbid=385&SubTB=385#/