3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200268r02

# Elbonia, February 14 – 25, 2022	(revision of S2-220xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 3480 | rev | 1 | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on 5G access stratum time distribution |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT |  |  |  |  |  | Date: |  |  | 2022-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In current text, it implies that the time synchronization error budget is provided only for the 5G access stratum time distribution method. In fact, both for 5G access stratum time distribution method and (g)PTP based synchronization method, the time error budget can be provided, and it is optional. This issue has been discussed in S2-2108633, but the related part has not been agreed last meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that the time synchronization error budget is used for both the 5G access stratum time distribution method and (g)PTP based synchronization method. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description for the time synchronization error budget providing is not correct. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 3333 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 5.27.1.9	Support for control of 5G access time distribution

The AF may request time synchronization and it may include time synchronization error budget in the request. If the AF included a time synchronization error budget in the AF request, the TSCTSF derives the error budget available for the NG-RAN to provide the 5G access stratum time to each targeted UE via the Uu interface (referred to as Uu time synchronization error budget hereafter). The TSCTSF uses the time synchronization error budget provided by AF (directly or via NEF) in order to derive the Uu time synchronization error budget.

To derive the Uu time synchronization error budget for each targeted UE, the TSCTSF takes the following into account:

-	selected time synchronization distribution method (5G access stratum time distribution or (g)PTP based time distribution);

-	whether 5GS operates as a boundary clock and acts as a GM or whether a clock connected to DS-TT/NW-TT acts as GM in case of (g)PTP based time distribution;

-	PTP port states in case of (g)PTP based time distribution;

-	CN and Device parts of the time synchronization error budget (which may be predefined, or calculated by the 5GS by means that are based on implementation).

If the AF has not included a time synchronization error budget, the TSCTSF uses a preconfigured time synchronization error budget to derive the Uu time synchronization error budget. TSCTSF provides a time distribution indication and the Uu time synchronization error budget to NG-RAN as described in clause 4.15.9.4 of TS 23.502 [3]. Based on this, NG-RAN provides the 5G access stratum time to the UE according to the Uu interface time synchronization error budget as provided by the TSCTSF (if supported by UE and NG-RAN).

NOTE:	This release of the specification assumes that deployments ensure that the targeted UEs and the NG-RAN nodes serving those UEs support Rel-17 propagation delay compensation as defined in TS 38.300 [27].

# * * * * End of changes * * * *