3GPP TSG-WG SA2 Meeting #149E e-meeting  	S2-2200926r01

Elbonia, February 14 – 25, 2022	(revision of S2-220xxxx)

Title:	[DRAFT] LS reply on RSC determination in the remote UE for 5G ProSe Layer-3 UE-to-network relay scenario

Response to:	LS (S2-2200050 / C1-217452) on RSC determination in the remote UE for 5G ProSe Layer-3 UE-to-network relay scenario from CT1

Release:	Release 17

Work Item:	5G_ProSe

Source:	[Huawei to be] SA2

To:	CT1

Cc:	-

Contact Person:

Name:	Guanglei Li

E-mail Address:	liguanglei3 AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 23.304 CR0080.

1. Overall Description:

SA2 thanks CT1 for their LS about RSC determination in L3 U2N relay scenarios, and how the Layer-3 Remote UE determines the RSC to use based on the current stage 2 specification.

Please find below SA2 answer to the question:

…

In this process, CT1 wonders how the remote UE can determine the RSC for relay discovery as it seems to be absent (as no other component is allowed to be transferred together with the “5G ProSe Layer 3 UE-to-Network Relay Offload indication” in the same RSD, and thus the remote UE has no idea which RSC should be used for this application).

Furthermore, given not suitable RSC is selected by the remote UE, it may result in that the relay UE establishes relay PDU session using the PDU session parameters associated with the RSC not suitable for the remote UE.

So CT1 believes that, based on the current stage 2 specification, the remote UE cannot determine the right RSC for relayed traffic, and hence CT1 would like to ask the following question to SA2.

Question: As in the above-mentioned scenario, so how can the remote UE determine the correct RSC for relay discovery?

SA2 Answer:

As pointed out by CT1, SA2 agrees that how the Layer-3 Remote UE can determine the correct RSC is absent in the specification. To resolve this issue, SA2 has agreed the attached CR for Layer-3 Remote UE RSC determination, in which an additional association between RSC and Traffic descriptor is added in ProSe Policy. After Remote UE decides to go with Layer-3 UE-to-Network Relay for a specific Traffic descriptor using the URSP, and then Remote UE uses the association between RSC and Traffic descriptor in ProSe policy to decide the used RSC.

2. Actions:

To CT1

ACTION:

SA2 kindly asks CT1 to take the answer into account.

3. Date of Next TSG SA WG2 Meetings:

See https://portal.3gpp.org/Home.aspx?tbid=385&SubTB=385#/