3GPP TSG-WG SA2 Meeting #149E e-meeting 	S2-2201041r22

Elbonia, February 14th – 25th, 2022	(revision of S2-220xxxx)

Source:	ZTE, Nokia, Nokia Shanghai Bell, Intel

Title:	FS_XRM: Media Unit definition

Document for:	Approval

Agenda Item:	9.19

Work Item / Release:	FS_XRM / Rel-18

Abstract: propose definition of media unit.

1. Introduction

In SP-211646 the following objectives are approved to better support advanced media services, e.g., High Data Rate Low Latency (HDRLL) services, AR/VR/XR services, and tactile/multi-modality communication services, including the following working tasks(WT):

WT#1: Enhancements for supporting multi-modality service:

-	Study whether and how to enable delivery of related tactile and multi-modal data (e.g., audio, video and haptic data related to a specific time) with an application to the user at a similar time, focusing on the need for policy control enhancements (e.g. QoS policy coordination).

WT#2: Enhancements of network exposure to support interaction between 5GS and application：

WT#2.1: Study whether and how interaction between AF and 5GS is needed for application synchronization and QoS policy coordination among multiple UEs or between multiple QoS flows per UE.

WT#2.2:  Study exposure of 5GS QoS information (e.g., QoS capabilities) and network conditions to the Application to enable quick codec/rate adaptation help to provide desired QoE (e.g. such as assist in alleviating 5GS congestion).

NOTE1: Parameters for exposure may coordinate with RAN and SA4.

WT#3: Study whether and how the following QoS and policy enhancements for XR service and media service transmission are performed:

WT#3.1:  Study the traffic characteristics of media service enabling improved network resources usage and QoE.

WT#3.2:  Enhance QoS framework to support  media units granularity  (e.g., video/audio frame/tile, Application Data Unit, control information), where media units consist of PDUs that have the same QoS requirements.

WT#3.3: Support differentiated QoS handling considering different importance of media units. e.g., eligible drop packets belong to less important media units to reduce the resource wasting.

NOTE2: Coordination with RAN WGs may be needed for the above bullets.

WT#3.4: Whether and how to support uplink-downlink transmission coordination to meet RTT (Round-Trip Time) latency requirements between UE and N6 termination point at the UPF.

WT#3.5: Potential policy enhancements to minimize the jitter, focusing on i.e. requirement provisioning from AF, extension of PCC rule.

WT#4: Study potential enhancements of power management considering traffic pattern of media services:

WT 4.1: Void.

WT 4.2: Power saving enhancement e.g. support trade-off of throughput/latency/reliability considering device battery life, whether and how to enhance CDRX, considering XR/media traffic pattern.

WT#5: Void.

NOTE3: Whether to define a network slice type supporting media services can be determined during normative phase.

In order to start the study, it would be good to agree the following definition

Media Unit: Group of packets that have the same QoS requirements and have to be delivered as single packet in 5GS.

Group of Media Unit: Group of media units that have different priority or importance and need to be coordinated when delivered in 5GS.

1. Text Proposal

It is proposed to capture the proposals as work assumption in TR 23.700-60.

# * * * * First change (all texts new)* * * *

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Application Data Unit: A unit of information transmitted at the application layer. An ADU is composed of one or more PDUs which are processed together at the application layer by the receiver.

Media Data Unit: A unit of information in the 5G system. An MDU corresponds to a set of one or more ADUs. An MDU is associated with importance information that is common to all PDUs of that MDU.

# * * * * End of changes * * * *