3GPP TSG-SA2 Meeting #149E (e-meeting)	S2-2200134r02

# Elbonia, Feb 14 - 25, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0061 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Resolve EN for Mobility Restriction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe |  |  |  |  |  | Date: |  |  | 2022-01-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | #1 Term “UE-to-Network Relay” is not correctly used, below is an example: “In a Forbidden Area, a 5G ProSe UE-to-Network Relay and a 5G ProSe Remote UE shall follow the principles of a UE in limited-service state as specified in clause 5.9” If a 5G ProSe enabled UE is in forbidden area, the UE cannot communicate with the network and cannot act Relay. #2 The following EN on Non-Allowed Area remains unresolved: Editor's note: It is FFS on mobility restriction when the 5G ProSe UE-to-Network Relay and 5G ProSe Remote UE locate in Non-allowed area. For Layer-3 Remote UE, Service Area Restriction is not applicable as there is not direct signalling connection between Layer-3 Remote UE and the network. For a ProSe enabled UE in Non-Allowed Area, whether such UE can perform relay operations, principles of a UE in limited-service state in clause 5.9 should be followed, that is, such UE should not perform relay operations. For Layer-2 Remote UE, principles of Non-Allowed Area as defined in clause of TS 23.501 should be followed for communication with network via Layer-2 Relay. In this case, relay discovery and communication are implied. For Core Network type restriction or RAT restriction, the 5G ProSe enabled UE is not allowed to register in 5G network, therefore principles of a UE in limited-service state shall be followed, that is, such UE should not perform relay operations. Closed Access Group is not specified for 5G ProSe. Emergency and priority services are not addressed, and propose to add a note to clarify. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace “UE-to-Network Relay” with “5G ProSe enabled UE” when needed. Resolve the EN as follows: Servcie Area Restriction is not applicable to 5G ProSe Layer-3 Remote UE. For a ProSe enabled UE in Non-Allowed Area, whether such UE can perform relay operations, principles of a UE in limited-service state in clause 5.9 should be followed. For Layer-2 Remote UE, principles of Non-Allowed Area as defined in clause of TS 23.501 should be followed for communication with network via Layer-2 Relay. In this case, relay discovery and communication are implied. For 5G ProSe enabled UE that can't communicate with the network due to reasons such as Core Network type restriction or RAT Restriction, principles in clause 5.9 shall be followed . Closed Access Group information is not specified for 5G ProSe UE-to-Network Relaying. Add a note about emergency and priority services The EN is removed with the above clarifications. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN remains unresolved and lack of clarifications on mobility restrictions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** FIRST CHANGE ***************

### 5.4.3	Mobility Restrictions for 5G ProSe UE-to-Network Relaying

The handling of Mobility Restrictions for 5G ProSe enabled UE- follows the principles as specified in clause 5.3.4.1 of TS 23.501 [4] with the following additions and clarifications:

-	Forbidden Area:

In a Forbidden Area,  a 5G ProSe enabled UE is not allowed to perform the Relay operation. If the 5G ProSe UE-to-Network Relay operates in a Forbidden Area of the 5G ProSe Remote UE, the 5G ProSe Remote UE is not allowed to access the network via this 5G ProSe UE-to-Network Relay.

-	Servcie Area Restriction:

Servcie Area Restriction is not applicable to 5G ProSe Layer-3 Remote UE.

-	Non-Allowed Area:

In a Non-Allowed Area of its serving PLMN, 5G ProSe enabled UE follows the principles of a UE in limited service state as specified in clause 5.9. It can not perform the Relay operation as 5G ProSe Layer-2 UE-to-Network Relay or 5G ProSe Layer-3 UE-to-Network Relay based on the conditions described in clause 5.9.

In a Non-Allowed Area, a 5G ProSe Layer-2 Remote UE follows the principles of a UE in limited service state as specified in clause 5.9 and it may only access the network via a 5G ProSe Layer-2 UE-to-Network Relay in an allowed area based on the conditions described in clause 5.9.

-	Core Network type restriction, RAT type restriction:

In these cases, a 5G ProSe enabled UE is not able to register in the network and stays in limited service state, and the principles in clause 5.9 shall apply.

NOTE 1: 	Closed Access Group information is not specified for 5G ProSe.

NOTE 2: 	Principles of operation for emergency services and high priority access UEs (incl. exceptions from mobility restrictions) are not specified in Rel-17.

****************** END OF CHANGES ***************